# ENGINEERING EVALUATION

Tesoro Refining & Marketing Company LLC

PLANT NO. 14628

APPLICATION NO. 27990

AVON WHARF MOTEMS PROJECT

BACKGROUND

The Tesoro Refining & Marketing Company LLC.  (Tesoro) is applying for an Authority to Construct and Permit to Operate for the Avon Wharf MOTEMS Project, including the following equipment:

S-1560	Avon Wharf Berth 1A, abated by

A-1560	Vapor Recovery System for Avon Wharf Berth 1A

S-1561	Emergency Diesel Generator, Caterpillar Model C9, 398 Bhp

S-1562	East Avon Wharf Emergency Diesel Firewater Pump, Caterpillar Model C18, 700 Bhp

S-1563	West Avon Wharf Emergency Diesel Firewater Pump, Caterpillar Model C18, 700 Bhp

S-1564	Recovered Oil Fixed Roof Tank A-938, 3800 gallons

S-1567	East Diesel Tank, 1000 gallons, exempt per Regulation 2-1-123.3.2

S-1568	West Diesel Tank, 1000 gallons, exempt per Regulation 2-1-123.3.2

This Project also includes the following permitted equipment:

S-100	Avon Wharf Loading Berth 1 (to be decommissioned)

S-108	Avon Wharf Berth 5 (to be demolished)

S-1469	Avon Wharf Firewater Pump Engine, 400 HP (to be demolished)

S-1508	Avon Berth 1 Recovered Oil Tank (to be decommissioned)

S-1509	Avon Berth 5 Recovered Oil Tank (to be demolished)

PROJECT DESCRIPTION

The existing Avon Wharf consists of 2 loading berths - Berth 1 (S-100) and Berth 5 (S-108).  Berth 5 has not been operated since June, 2009.  The Avon Wharf is required to undergo seismic upgrades to meet the higher building standards requirements of the US Coast Guard  Marine Oil Terminal Engineering and Maintenance Standards (MOTEMS).  These requirements also include upgrading the fire suppression system at the Wharf and health and safety improvements.  This project does not change the Wharf loading/unloading operations, the current loading capacity or marine cargo carrier traffic.  Existing refinery pumps and pipeline hydraulic limitations dictate loading capacity.  There will be no increase in pump flow rates or in the hydraulic limitations on the pipelines.

The piping manifold with the loading hoses is being relocated approximately 60 feet to the east, adjacent to the existing Wharf structure.  The District considers this to be a new loading berth, pursuant to Regulation 2-1-232, and therefore is treating cargo carrier emissions associated with this berth as new emissions subject to offsets.  Cargo carrier emissions are calculated based on a throughput limit of 30 million barrels per year.  This is the same limit as the existing grandfathered throughput limit for Berth 1, which is being decommissioned.  The existing vapor recovery system will be demolished and replaced with a new vapor recovery system to capture emissions from loading operations at the Wharf.  The project also includes one new emergency generator and two new fire water pumps.  A new recovered oil tank will replace the existing recovered oil tank on Berth 5.  The new tank will collect oil retained in the loading hoses and collect rainfall during the rainy season.

Berth 1 (S-100) will be decommissioned, and Berth 5 (S-108) will be demolished.  The Recovered Oil Tanks A-906 (S-1508) will be decommissioned, and A-907 (S-1509) will be demolished.  These permits will be surrendered and archived.  The Avon Wharf Diesel Firewater Pump (S-1469) will be demolished.  The S-1469 permit will be surrendered once demolition is complete.

Avon Wharf Berth 1A (S-1560)

The existing Avon Wharf (Berths 1 and 5) are grandfathered sources that were never granted an authority to construct by the District.  The capacity limits in Tesoro's Title V Permit are limits that serve as reporting thresholds and are not firm limits that can be used as noncompliance with the permit (as detailed in Title V Standard Condition J.2).

The BAAQMD considers the seismic upgrade of the Avon Wharf to be a new source because Tesoro is constructing a new Berth 1A, adjacent to the existing Berth 1.  Thus source S-1560 Avon Berth 1A is subject to Regulation 2, Rule 2, New Source Review.  Emissions specific to the S-1560 Avon Berth 1A source are new fugitive components, loading emissions and marine cargo carrier emissions (per the 2-2-215 definition of Facility).  Marine cargo carrier emissions are subject to BAAQMD 2-2-302 and 2-2-303 (Offsets). Marine cargo carrier emissions are not subject to BACT, TBACT or PSD per 2-2-206, 2-2-244 and 2-2-215.2, respectively.  Tesoro will be offsetting the marine cargo carrier emissions estimated from product transfer at the Avon Wharf at a rate of 30,000,000 barrels per year.  This capacity is equivalent to the current grandfathered capacity of Avon Wharf Berth 1, which will be decommissioned.  Upon issuance of this permit, the Avon Wharf Berth 1A will have a fully offset federally enforceable product transfer limit of 30,000,000 barrels per year.

There are 12 existing pipelines from the refinery to the Avon Wharf.  Nine of the 12 existing lines are in hydrocarbon service.  The end of each pipeline will be demolished, and approximately 350 feet of the 12 existing pipelines will be constructed from Isolation Station 1 to the new Berth 1A.

Marine Vapor Recovery (A-1560)

The existing marine vapor recovery system, A-14, will be replaced by a new Marine Vapor Recovery System, A-1560.  The vapor recovery system recovers hydrocarbon vapors during the loading of gasoline and other volatile compounds into marine cargo carriers.  The recovered vapors will continue to be sent to the existing vapor recovery system at the No. 1 Gas Plant (A-14), and are used in Tesoro’s 40-pound fuel gas system.  There will be no change to the size or to the compression capacity or compressing rates and there will be no changes to the tie in to No. 1 Gas Plant or the 40-pound fuel gas system.  The abatement device will be as efficient, or more efficient than, the abatement device being replaced.

The A-1560 Vapor Recovery System includes a displaced gas enrichment system that injects a propane/propylene mixture into the displacement gases when the cargo carrier tank being loaded contains vapors that will adversely impact the fuel gas system (e.g., high oxygen content).

The replacement vapor recovery system will be skid mounted.  The components are the same as in the current system and include the following equipment and safety devices:

- Oxygen Analyzer

- Detonation Arrestor

- Electronic Control Panel

- Vapor Piping, instruments and valves

- Enrichment Gas Piping and Instruments

- Pressure/Vacuum Relief Valve with Flame Screen

Emergency Standby Diesel IC Engine Generator Set (S-1561)

The Emergency Standby Diesel IC Engine Generator Set at the Avon Wharf will be a permanent stationary source.  The engine will only be used when power is lost to the Wharf facilities.  The proposed engine is a Caterpillar Model C9 with a 2015 engine rated at 398 brake horsepower (bhp). The engine is 4-stroke-cycle, in-line, 6-cylinder.  The aspiration is turbocharged and air-to-air aftercooled.  The engine is EPA certified and the Reference EPA Standard Engine Family is FCPXL08.8NZS.  The proposed new emergency standby IC engine will have a maximum fuel usage of approximately 19.4 gallons of diesel fuel per hour.  Testing and maintenance operation  will be limited to 50 hours per year per year, based on the PM10 standard in the Airborne Toxic Control Measure for Stationary Compression Ignition Engines (ATCM).  The new engine complies with the Airborne Toxic Control Measure for Stationary Compression Ignition Engines (ATCM), NSPS Subpart IIII, MACT Subpart ZZZZ, and will meet the Best Available Control Technology (BACT).

East and West Fire Water Pumps (S-1562 and S-1563)

The two new emergency fire water pumps with diesel IC engine drivers at the Avon Wharf will be permanent stationary sources. The engines are identical and will be used for emergency operation.  The proposed engines are Caterpillar Model C-18, model year 2012, rated at 700 bhp. The engines are 4-stroke-cycle, in-line, 6-cylinder.  The aspiration is turbocharged and air-to-air aftercooled.  The engines are EPA certified and the Reference EPA Standard Engine Family is CCPXL18.1NZS.  The proposed IC engines will each have a maximum fuel usage of approximately 35.9 gallons of diesel fuel per hour.  Testing and maintenance will be limited to 70 hours per year per IC engine, based on the direct-drive firewater pump provisions of the Airborne Toxic Control Measure for Stationary Compression Ignition Engines (ATCM).  The new engines will comply with the Airborne Toxic Control Measure for Stationary Compression Ignition Engines (ATCM), NSPS Subpart IIII, MACT Subpart ZZZZ, and will meet the Best Available Control Technology (BACT) for this IC engine.

Recovered Oil Tank (S-1564)

S-1564, Recovered Oil Tank (Tank A-938) is a double walled fixed roof tank.  The new tank will be 6 feet in width, 16 feet long and 5.5 feet tall, with a capacity of 3,800 gallons.  Tank S-1564 will contain water runoff and recovered oil from the Avon Wharf.  The proposed throughput will be 250 thousand gallons (kgal)/year.  While most of the throughput will be water, recovered oil at the Avon Wharf would be clean product, predominantly gasoline and diesel.  Therefore the vapor pressure can vary.  The tank will be at ambient temperature (approximately 70 degrees F).  Since the new tank will have submerged fill, Tesoro requests that the tank be permitted to contain materials with true vapor pressure less than 11 pounds per square inch (psia).

The installation of the tank will include new piping and fugitive components.  Welded piping will be used, to minimize the number of flanges and connectors for the project.

Diesel Tanks (S-1567 and S-1568)

Each Firewater Pump will be serviced by a Diesel fuel tank.  Each tank has a diameter of 4 feet, and a height of 9.4 feet, for a total capacity of 880 gallons.  Diesel fuel has an initial boiling point above 302ºF and emission calculations demonstrate that emissions of naphthalene and xylene are below the Regulation 2, Rule 5 trigger levels.  Therefore, pursuant to Regulation 2-1-123.3.2, these tanks are exempt from the permitting requirements of Regulations 2-1-301 and 2-1-302.

#### EMISSION CALCULATIONS

The requirements for evaluating a new source are detailed in Regulation 2-2-604, Emission Increase Calculation Procedures, New or Modified Sources.  The applicable sub-section is 2-2-604.1:

2-2-604	Emission Increase Calculation Procedures, New or Modified Sources: The APCO shall determine the annual emission increase, expressed as tons per year, from:

604.1	A new source based on the maximum emitting potential of the new source or the maximum permitted emission level of the new source, approved by the APCO, subject to federally enforceable limiting conditions.

604.2	A modified source by subtracting either the baseline annual emission rate, as calculated using the methodology in Section 2-2-605, from the new maximum permitted emission level of the modified source, approved by the APCO, subject to federally enforceable limiting conditions.

Emissions for this project are estimated for each of the following new sources:

- S-1560 Avon Wharf Berth 1A

- S-1561	Emergency Diesel Generator

- S-1562 and S-1563    East and West Avon Wharf Emergency Diesel Firewater Pump

- S-1564	Recovered Oil Fixed Roof Tank A-938

- Project Fugitive Emissions

Additional emission calculations are included to determine contemporaneous emissions reductions for the shutdown  of Berth 1, including the following:

- Berth 1 Marine Cargo Carrier Emissions

- Berth 1 Loading Operations Emissions

- Berth 1 Vapor Recovery System

There are no contemporaneous emissions reductions associated with the shutdown of Berth 5 because Berth 5 was not operated during the 3-year period that is evaluated to determine the emissions.

A.	S-1560 Avon Wharf Berth 1A

Emissions for S-1560 are Marine Cargo Carrier Emissions and Product Loading Emissions.

- Marine Cargo Carrier Emissions

Marine cargo carrier emissions are calculated based on the methodology and assumptions included in Appendix A of this evaluation (which are based on the ARB, Port of Long Beach and Port of Los Angeles inventory methodologies).

Emissions were calculated using a three-year baseline from August 2013 through July 2016.  During this baseline period, there were 180 cargo carrier calls at Berth 1.  114 of these Berth 1 calls were tankers (aka Ocean Going Vessels, or OGV) and 66 were barges.  115 of the Berth 1 calls loaded materials and 90 delivered materials (25 calls included both operations).   Emissions were estimated for each call. The baseline is used to calculate contemporaneous emissions reductions that will be achieved when the existing Berth 1 is decommissioned.  The baseline emission calculations are also used to derive overall emission factors used to calculate S-1560 permitted emissions and offset requirements.  This approach for calculating future permitted emissions assumes that the future operation of S-1560 Berth 1A will be similar to the operation of Berth 1 during the 3-year baseline (i.e., the same ratio of tankers to barges, the same ratio of material loaded/unloaded to the barge/tanker material storage capacity, the same ratio of material loading to material unloading, and the same mix of products that were loaded at Berth 1).

Tesoro has provided a cargo carrier emissions workbook that summarizes the quantity of material throughput and estimates emissions from tankers, barges and the required tug boats for the baseline period of August 2013 through July 2016.  These emissions are summarized in the following table.

| Year | Month | Total Barrels Throughput | Emissions Total, Lbs |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NOX | CO | POC | PM10 | SO2 |
| 2013 | August | 1,135,915 | 8,549.79 | 1,472.75 | 497.20 | 261.39 | 1,153.61 |
| 2013 | September | 776,336 | 6,100.78 | 1,055.77 | 345.84 | 178.76 | 775.06 |
| 2013 | October | 711,266 | 6,833.38 | 964.13 | 365.00 | 213.44 | 1,042.95 |
| 2013 | November | 984,714 | 10,385.91 | 1,733.58 | 577.25 | 309.47 | 1,386.73 |
| 2013 | December | 1,699,992 | 14,340.70 | 2,308.44 | 805.04 | 434.21 | 1,982.76 |
| 2014 | January | 1,151,480 | 10,221.23 | 1,999.18 | 616.38 | 230.09 | 258.18 |
| 2014 | February | 850,700 | 9,267.36 | 1,585.26 | 495.13 | 192.22 | 223.10 |
| 2014 | March | 1,383,586 | 12,635.21 | 2,268.44 | 755.93 | 284.17 | 334.37 |
| 2014 | April | 1,437,009 | 14,720.62 | 3,006.69 | 854.23 | 328.32 | 337.00 |
| 2014 | May | 789,934 | 7,894.48 | 2,692.51 | 556.47 | 211.05 | 137.28 |
| 2014 | June | 1,129,178 | 8,867.91 | 1,731.45 | 531.93 | 199.68 | 219.15 |
| 2014 | July | 1,023,626 | 12,136.87 | 1,680.95 | 640.83 | 244.37 | 317.89 |
| 2014 | August | 651,690 | 6,740.00 | 1,249.51 | 388.88 | 146.70 | 166.63 |
| 2014 | September | 790,994 | 7,980.54 | 1,208.26 | 456.97 | 171.06 | 211.42 |
| 2014 | October | 953,134 | 9,530.00 | 1,507.91 | 535.26 | 197.58 | 239.06 |
| 2014 | November | 653,179 | 7,686.63 | 2,505.04 | 538.42 | 204.15 | 150.40 |
| 2014 | December | 1,173,402 | 11,093.49 | 1,792.38 | 649.83 | 254.05 | 320.68 |
| 2015 | January | 945,676 | 5,928.09 | 2,064.79 | 402.51 | 162.89 | 89.77 |
| 2015 | February | 1,538,475 | 8,809.42 | 3,224.47 | 621.97 | 257.80 | 109.50 |
| 2015 | March | 1,543,846 | 9,304.27 | 2,563.07 | 578.96 | 230.56 | 160.76 |
| 2015 | April | 625,150 | 5,388.30 | 969.91 | 337.08 | 125.11 | 148.50 |
| 2015 | May | 823,702 | 5,703.16 | 869.55 | 347.65 | 130.89 | 171.92 |
| 2015 | June | 355,441 | 2,769.96 | 413.50 | 155.42 | 56.37 | 69.69 |
| 2015 | July | 572,717 | 7,394.39 | 1,477.44 | 448.23 | 171.97 | 180.96 |
| 2015 | August | 98,638 | 2,592.25 | 405.27 | 142.04 | 56.49 | 72.56 |
| 2015 | September | 111,246 | 1,334.12 | 193.94 | 73.38 | 28.39 | 37.10 |
| 2015 | October | 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2015 | November | 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2015 | December | 224,331 | 2,411.88 | 319.76 | 127.75 | 50.52 | 68.67 |
| 2016 | January | 308,157 | 5,055.83 | 824.93 | 288.57 | 109.19 | 126.84 |
| 2016 | February | 77,818 | 486.54 | 336.54 | 52.63 | 19.64 | 0.45 |
| 2016 | March | 150,583 | 1,186.71 | 178.40 | 67.02 | 25.02 | 31.58 |
| 2016 | April | 220,854 | 2,042.56 | 304.07 | 129.68 | 53.21 | 75.98 |
| 2016 | May | 1,014,649 | 13,699.74 | 2,152.99 | 722.66 | 285.50 | 352.99 |
| 2016 | June | 715,364 | 7,783.99 | 1,387.19 | 450.56 | 168.19 | 189.21 |
| 2016 | July | 486,695 | 5,851.16 | 1,719.08 | 385.05 | 145.25 | 110.27 |
| Baseline Total |  | 27,109,477 | 252,727 | 50,167 | 14,942 | 6,138 | 11,253 |
| Baseline Totals, tons |  |  | 126.364 | 25.084 | 7.471 | 3.069 | 5.627 |

The annual average throughput for the 3-year baseline is 27,109,477 / 3 = 9,036,492 barrels/year.  The ratio of the proposed 30,000,000 barrels/year to the 3-year baseline throughput of 9,036,492 barrels/year is 3.320.

Using the total material throughputs, overall emission factors can be derived in terms of lbs of pollutant per 1000 barrels of material throughput.  The following table summarizes the overall emission factors derivation.

| Year | Month | Total Barrels Throughput | Emissions Factors Lbs/1000Barrels |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NOX | CO | POC | PM10 | SO2 |
| 2013 | August | 1,135,915 | 7.527 | 1.297 | 0.438 | 0.230 | 1.016 |
| 2013 | September | 776,336 | 7.858 | 1.360 | 0.445 | 0.230 | 0.998 |
| 2013 | October | 711,266 | 9.607 | 1.356 | 0.513 | 0.300 | 1.466 |
| 2013 | November | 984,714 | 10.547 | 1.760 | 0.586 | 0.314 | 1.408 |
| 2013 | December | 1,699,992 | 8.436 | 1.358 | 0.474 | 0.255 | 1.166 |
| 2014 | January | 1,151,480 | 8.877 | 1.736 | 0.535 | 0.200 | 0.224 |
| 2014 | February | 850,700 | 10.894 | 1.863 | 0.582 | 0.226 | 0.262 |
| 2014 | March | 1,383,586 | 9.132 | 1.640 | 0.546 | 0.205 | 0.242 |
| 2014 | April | 1,437,009 | 10.244 | 2.092 | 0.594 | 0.228 | 0.235 |
| 2014 | May | 789,934 | 9.994 | 3.409 | 0.704 | 0.267 | 0.174 |
| 2014 | June | 1,129,178 | 7.853 | 1.533 | 0.471 | 0.177 | 0.194 |
| 2014 | July | 1,023,626 | 11.857 | 1.642 | 0.626 | 0.239 | 0.311 |
| 2014 | August | 651,690 | 10.342 | 1.917 | 0.597 | 0.225 | 0.256 |
| 2014 | September | 790,994 | 10.089 | 1.528 | 0.578 | 0.216 | 0.267 |
| 2014 | October | 953,134 | 9.999 | 1.582 | 0.562 | 0.207 | 0.251 |
| 2014 | November | 653,179 | 11.768 | 3.835 | 0.824 | 0.313 | 0.230 |
| 2014 | December | 1,173,402 | 9.454 | 1.528 | 0.554 | 0.217 | 0.273 |
| 2015 | January | 945,676 | 6.269 | 2.183 | 0.426 | 0.172 | 0.095 |
| 2015 | February | 1,538,475 | 5.726 | 2.096 | 0.404 | 0.168 | 0.071 |
| 2015 | March | 1,543,846 | 6.027 | 1.660 | 0.375 | 0.149 | 0.104 |
| 2015 | April | 625,150 | 8.619 | 1.551 | 0.539 | 0.200 | 0.238 |
| 2015 | May | 823,702 | 6.924 | 1.056 | 0.422 | 0.159 | 0.209 |
| 2015 | June | 355,441 | 7.793 | 1.163 | 0.437 | 0.159 | 0.196 |
| 2015 | July | 572,717 | 12.911 | 2.580 | 0.783 | 0.300 | 0.316 |
| 2015 | August | 98,638 | 26.280 | 4.109 | 1.440 | 0.573 | 0.736 |
| 2015 | September | 111,246 | 11.993 | 1.743 | 0.660 | 0.255 | 0.334 |
| 2015 | October | 0 |  |  |  |  |  |
| 2015 | November | 0 |  |  |  |  |  |
| 2015 | December | 224,331 | 10.751 | 1.425 | 0.569 | 0.225 | 0.306 |
| 2016 | January | 308,157 | 16.407 | 2.677 | 0.936 | 0.354 | 0.412 |
| 2016 | February | 77,818 | 6.252 | 4.325 | 0.676 | 0.252 | 0.006 |
| 2016 | March | 150,583 | 7.881 | 1.185 | 0.445 | 0.166 | 0.210 |
| 2016 | April | 220,854 | 9.248 | 1.377 | 0.587 | 0.241 | 0.344 |
| 2016 | May | 1,014,649 | 13.502 | 2.122 | 0.712 | 0.281 | 0.348 |
| 2016 | June | 715,364 | 10.881 | 1.939 | 0.630 | 0.235 | 0.264 |
| 2016 | July | 486,695 | 12.022 | 3.532 | 0.791 | 0.298 | 0.227 |
| Total Materials |  | 27,109,477 |  |  |  |  |  |
| Average Emission Factor |  |  | 10.117 | 2.005 | 0.602 | 0.242 | 0.394 |
| Maximum Emission Factor |  |  | 26.280 | 4.325 | 1.440 | 0.573 | 1.466 |
| Minimum Emission Factor |  |  | 5.726 | 1.056 | 0.375 | 0.149 | 0.006 |

Emission factors vary widely, primarily due to the size of any individual loading/unloading event compared to the storage capacity of the cargo carrier.  During the 3-year baseline period, barge cargo capacities range from about 24,000 to 185,000 barrels and tanker cargo capacities range from about 325,000 to 470,000 barrels.  Total material transferred over the Avon Wharf to/from a cargo carrier ranges from about 14,000 to 311,000 barrels.  Emission factors expressed in terms of pounds of pollutant per barrel of material transferred will be highest when a small transfer is removed from a large cargo carrier (e.g. a 2014 Berth 1 call unloaded  about 34,000 barrels of material from a 320,000 barrel capacity cargo carrier).  Since there is a wide range of emission factors, using an average emission factor to estimate permitted future emissions for S-1560 introduces uncertainty.  Not all cargo carrier capacities are known, but for the berth calls with known vessel capacities (143 calls out of 180 total), the average cargo carrier utilization [=(material transferred)/(carrier capacity)]  is about 68%.

Due to the wide range of emission factors, using an average emission factor to estimate permitted emissions for S-1560 is not appropriate, because the maximum PTE could be as high as the emissions derived by the maximum emission factor.  However, using the maximum emission factor would in all likelihood overestimate future emissions.  Therefore, the approach to estimate future permitted emissions is to derive the maximum consecutive 12-month emission factor.  The following table summarizes the overall consecutive 12-month emission factors derived from the 3-year baseline.

| Consecutive 12-Months ending |  | 12-Month Throughput Barrels | Emissions Factors Lbs/1000Barrels |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Month |  | NOX | CO | POC | PM10 | SO2 |
| 2014 | July | 13,073,736 | 9.3282 | 1.7209 | 0.5386 | 0.2361 | 0.6248 |
| 2014 | August | 12,589,511 | 9.5432 | 1.7694 | 0.5507 | 0.2361 | 0.5704 |
| 2014 | September | 12,604,169 | 9.6813 | 1.7794 | 0.5589 | 0.2352 | 0.5250 |
| 2014 | October | 12,846,037 | 9.7089 | 1.7883 | 0.5616 | 0.2296 | 0.4526 |
| 2014 | November | 12,514,502 | 9.7504 | 1.8973 | 0.5734 | 0.2272 | 0.3658 |
| 2014 | December | 11,987,912 | 9.9078 | 1.9376 | 0.5856 | 0.2222 | 0.2432 |
| 2015 | January | 11,782,108 | 9.7165 | 1.9770 | 0.5777 | 0.2204 | 0.2331 |
| 2015 | February | 12,469,883 | 9.1439 | 1.9994 | 0.5560 | 0.2135 | 0.2112 |
| 2015 | March | 12,630,143 | 8.7641 | 1.9974 | 0.5349 | 0.2065 | 0.1947 |
| 2015 | April | 11,818,284 | 8.5765 | 1.9622 | 0.5279 | 0.2035 | 0.1922 |
| 2015 | May | 11,852,052 | 8.3672 | 1.8028 | 0.5088 | 0.1962 | 0.1945 |
| 2015 | June | 11,078,315 | 8.4012 | 1.8098 | 0.5103 | 0.1969 | 0.1946 |
| 2015 | July | 10,627,406 | 8.3114 | 1.8674 | 0.5139 | 0.1985 | 0.1900 |
| 2015 | August | 10,074,354 | 8.3559 | 1.8861 | 0.5176 | 0.2004 | 0.1911 |
| 2015 | September | 9,394,606 | 8.2530 | 1.9146 | 0.5142 | 0.1997 | 0.1864 |
| 2015 | October | 8,441,472 | 8.0559 | 1.9522 | 0.5089 | 0.1989 | 0.1791 |
| 2015 | November | 7,788,293 | 7.7446 | 1.7943 | 0.4824 | 0.1893 | 0.1748 |
| 2015 | December | 6,839,222 | 7.5500 | 1.8279 | 0.4730 | 0.1858 | 0.1622 |
| 2016 | January | 6,201,703 | 8.1854 | 1.8159 | 0.5033 | 0.1963 | 0.1849 |
| 2016 | February | 4,741,046 | 8.9518 | 1.7663 | 0.5382 | 0.2065 | 0.2188 |
| 2016 | March | 3,347,783 | 10.2525 | 1.7890 | 0.6093 | 0.2311 | 0.2713 |
| 2016 | April | 2,943,487 | 10.5240 | 1.8085 | 0.6225 | 0.2384 | 0.2839 |
| 2016 | May | 3,134,434 | 12.4341 | 2.1078 | 0.7042 | 0.2732 | 0.3244 |
| 2016 | June | 3,494,357 | 12.5883 | 2.1694 | 0.7162 | 0.2771 | 0.3252 |
| 2016 | July | 3,408,335 | 12.4532 | 2.2950 | 0.7157 | 0.2762 | 0.3127 |
| Average Emission Factor |  |  | 9.3820 | 1.8974 | 0.5601 | 0.2198 | 0.2803 |
| Maximum 12-Month Emission Factor |  |  | 12.5883 | 2.2950 | 0.7162 | 0.2771 | 0.6248 |

Tesoro has no plans to make significant changes to the Berth 1A calls, compared to the Berth 1 calls. Thus the expectation is that

1.) the ratio of tankers to barges,

2.) the ratio of material loaded/unloaded to the barge/tanker material storage capacity,

3.) the ratio of material loading to material unloading, and

4.) the mix of products,

will not differ significantly from what occurred in the 3-year baseline period.  Therefore, Tesoro is confident that this 12 consecutive month maximum emission factor approach is valid to conservatively estimate future emissions.

Applying these maximum consecutive 12-month emission factors to the 30,000,000 barrels per year throughput results in the following future emissions.

| Berth 1A Total Annual Marine Cargo Carrier Emissions, 30,000,000 Barrels/year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | NOX | CO | POC | PM10 | SO2 |
| Emissions Factors Lbs/1000Barrels | 12.588 | 2.295 | 0.716 | 0.277 | 0.625 |
| Total Emissions, Lbs | 377,649 | 68,850 | 21,486 | 8,313 | 18,744 |
| Total Emissions, Tons | 188.825 | 34.425 | 10.743 | 4.157 | 9.372 |

- Product Loading Emissions

As part of the cargo carrier emission calculations for each cargo carrier call, Tesoro also provided the quantity of material loaded into each cargo carrier.  During the 3-year baseline period, of the 180 cargo carrier calls at Berth 1, 115 of the calls included loading operations.  Tesoro tracked the quantity of crude oil, gasoline (including blending components), diesel, jet naphtha, jet kerosene and residual oil loaded. However, only gasoline, diesel and residual oil were loaded during the baseline period.  In addition to these materials, Tesoro has reported the loading of ethanol, distillate oil, and FCCU fresh feed in the past, but these materials were not loaded during the 3-year baseline period.

During the baseline period, there was a total of 18,557,550 barrels of material loaded onto cargo carriers.  Of this material, 5.7% was gasoline, 5.2% was residual oil, and the remainder (89.1%) was diesel.

Emission calculations are based on emission factors from AP-42, Chapter 5.2.  Gasoline loading is subject to control requirements of Regulation 8, Rule 44, Section 304 which specifies a 95% emission reduction requirement.  In addition, BACT for Marine Loading  requires a 98.5% capture and control efficiency.

A-1560 Marine Vapor Recovery will be installed to comply with the abatement requirements for gasoline loading.  A-1560 collects the loading emissions, provides enrichment gas, if necessary, to ensure the gases are below the explosion limit, and discharges the enriched marine loading emissions into the A-14 vapor recovery system.  A-14 collects emissions from a variety of sources at the refinery.  The combined emissions are sent to the No. 1 Gas Plant where the gases are compressed, sent to fuel gas processing, and ultimately end up in the refinery 40# fuel gas system.  40# fuel gas is used in furnaces S-908, S-909 and S-912 in the No. 1 Feed Prep Unit and in the No. 3 Crude Unit.

Tesoro is already subject to a 99.5% destruction efficiency for S-908, S-909, and S-912 in Permit Conditions 13605 (for Alkyl ate Railcar Rack and Storage Tank S-1528 and S-323) and 21100 (for Naphtha/Heavy Reformate Tank S-1496).  Tesoro is also subject to a 98% destruction efficiency for S-908, S-909, and S-912 in Permit Conditions 20099 (for Oil Water Separator S-532), However, the 98% and 99.5% destruction efficiencies apply only to the furnaces because the measurement is specified across the combustion devices.  The BACT requirement of 98.5% capture and control efficiency is a better basis for estimating loading emissions at S-1560 Berth 1A.  This 98.5% overall efficiency provides a conservative emissions estimate for not only the furnace destruction efficiency (expected to exceed 99.5%), but also the capture system emissions at the berth, the pipeline to the No 1 Gas Plant, and any losses at the gas compression, the fuel gas treating, and the fuel gas distribution system to the furnaces.  Tesoro expects the capture system at Berth 1A to be 100% efficient because the A-1560 Vapor Recovery System operates at negative pressure during loading operations.

S-1560 Loading Emissions during the 3-year baseline period are summarized in the following table.

| Material | Tanker or Barge | AP-42 Uncontrolled Emission Factor, lb/1000gal | Total Quantity Loaded During 3-year Baseline, Barrels | Uncontrolled Emissions, lbs | Capture and Control Efficiency | Controlled Emissions, lbs |
| --- | --- | --- | --- | --- | --- | --- |
| Gasoline/ Components | Tanker | 1.8 | 741,149 | 56,031 | 98.5% | 840 |
|  | Barge | 3.4 | 314,945 | 44,974 | 98.5% | 675 |
| Diesel | Tanker | 0.005 | 15,427,374 | 3,240 | N/A | 3240 |
|  | Barge | 0.012 | 1,194,571 | 602 | N/A | 602 |
| Residual Oil | Tanker | 0.00004 | 618,232 | 1.04 | N/A | 1 |
|  | Barge | 0.00009 | 361,279 | 1.37 | N/A | 1 |
| Total 3-Year Baseline |  |  | 18,657,550 | 104,849 |  | 5,359 |
| Annual Average |  |  | 6,219,183 | 34,950 |  | 1786 |
| Annual Average, tons |  |  |  |  |  | 0.893 |

Total material throughput for Berth 1 during the 3-year baseline period was 27,109,477 barrels, including the 18,657,550 barrels of material loaded.  Using the overall average emission factor approach, future S-1560 Berth 1A loading emissions can be estimated using the derived average emission factor of 5359 lbs/27,109,477 barrels x1000/42 = 0.00471 lb/1000gal.

Tesoro is subject to 8-44-304 Emission Control Requirements.  8-44-304.1 provides 2 options for compliance.  The first option is to limit emissions to no more than 2 lbs/1000 barrels of organic liquid.  The section option is to reduce emissions by at least 95 percent by weight from uncontrolled conditions.  Although Tesoro is only required to meet one of these options, its operation meets both options.

The effective emission factor for gasoline loading is (840+675)/(741,149+314,945) x 1000 = 1.44 lb/1000 barrels gasoline loaded.  Thus, gasoline loading is expected to be in compliance with 2 lbs/1000 barrels emissions limit of the 8-44-304.

Tesoro will be permitted to load other regulated materials besides gasoline.  8-44-304 has an alternative emission control standard requiring emission reductions of at least 95% by weight.   All of Tesoro’s regulated material loading is expected to meet the alternative emission control option of 8-44-304 since the loading emissions capture and control efficiency is 98.5%, higher than the 95% emissions reduction requirement.

Per Regulation 8-44-301.2, Tesoro is also required to comply with 8-44-304 emission controls when loading non-regulated materials into a ship cargo vessel in which the previous cargo was a regulated material.  Tesoro is expected to comply with this emissions control requirement because the ships that will call on Berth 1A will keep records of the previous cargo, and Tesoro will have access to these records.  Therefore, Tesoro will know, ahead of time, when planning on loading a non-regulated material, that the gas displaced from the cargo tank will need to be routed to the A-1560 Marine Vapor Recovery System.  Note that per 8-44-220, a cargo tank is no longer considered to have held a prior cargo of a regulated organic liquid if it has been vented of organic gases and cleaned with an unregulated organic liquid or inorganic liquid.  The emission factors used in the table above are applicable for a typical overall situation based on observations that 41% of tested ship compartments were uncleaned, 11% ballasted, 24% cleaned, and 24% gas-freed (for barges, 76% were uncleaned).

Future Berth 1A Loading Emission estimates are summarized in the following table.

| Description | Units | Value |
| --- | --- | --- |
| Total Annual Material Throughput | Barrels | 30,000,000 |
|  | 1000 gallons | 1,260,000 |
| Loading Operations Emission Factor | Lb/1000gallons | 0.00471 |
| Total Annual Loading Emissions | Lbs | 5934 |
|  | Tons | 2.965 |
| Permitted Limit | Tons | 20.000 |

This future loading emissions estimate is based on the same ratio of materials loaded as in the 3-year baseline.  The following table summarizes the material throughputs associated with this future emissions estimate.

| Material | Tanker or Barge | Berth 1A Material Loading Throughputs, Barrels/year |
| --- | --- | --- |
| Gasoline/ Components | Tanker | 825,705 |
|  | Barge | 350,876 |
| Diesel | Tanker | 17,187,454 |
|  | Barge | 1,330,857 |
| Residual Oil | Tanker | 688,765 |
|  | Barge | 263,236 |

There will be variation in overall loading throughputs and emission rates based on the relative amounts of each material loaded in a year, and may not be exactly the same as during the 3-year baseline period.  In order to operate S-1560 with the flexibility required, Tesoro has requested that POC emissions from S-1560 Loading Operations be permitted at 20 tons/year.  This will be a federally enforceable fully offset limit.  Tesoro will keep the records needed to perform the calculations necessary to demonstrate compliance with this annual loading emission limit.

The 20 ton/year POC limit for loading operations will allow greater quantities of material loading compared to the materials loaded during the 3-year baseline.  This allowance deviates from the primary assumption in the determination of future ship emissions – there will not be any significant changes to the Berth 1A calls, compared to the Berth 1 calls.  In particular, the 20 ton/year limit potentially changes the ratio of material loaded to material unloaded.  In order to determine if this change will impact ship emissions, a review of the 3-year baseline ship calls was performed.  Ship calls that only involved loading operations were reviewed.  The results of this review found was that the average emission factor for loading-operations-only ship calls was lower than the average emission factor for all ship calls.  The conclusion is that allowing an increase in loading operations throughput potentially reduces ship emissions.  Therefore, there is no concern that allowing 20 tons/year POC emissions for loading operations will adversely affect ship emissions.

Peak daily emissions for the loading operations are determined by a review of each cargo carrier call during the 3-year baseline.  The results indicate there are days where POC emissions from gasoline loading exceed 10 lbs/highest day.

The total S-1560 emissions are summarized in the following table.

| Berth 1A Total Annual Emissions, Tons -- 30,000,000 Barrels/year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | NOX | CO | POC | PM10 | SO2 |
| Marine Cargo Carrier | 188.825 | 34.425 | 10.743 | 4.157 | 9.372 |
| Material Loading |  |  | 20.000 |  |  |
| Total Emissions, Tons | 188.825 | 34.425 | 30.743 | 4.157 | 9.372 |

B.	S-1561 Emergency Diesel Generator

Emergency standby diesel internal combustion (IC) engine criteria pollutant emissions from the combustion of diesel fuel include nitrogen oxides (NOx), sulfur dioxide (SO2), carbon monoxide (CO), precursor organic compounds (POC), and particulate matter less than 10 microns in diameter (PM10).  Emissions are estimated based the EPA “D2 Cycle Averages” Certification level for Engine Family FCPXL08.8NZS.

Sulfur dioxide is calculated based on 15 ppm sulfur content in diesel fuel.  This ultra-low diesel fuel is required to be used in California.  SO2 emissions are calculated by mass balance with 0.0015% sulfur in fuel by weight, using the engine peak fuel rate specification.  Emissions are estimated for a 398 brake horsepower (bhp) IC engine.  Annual emissions are limited to 50 hours of operation (testing and maintenance) to comply with the CARB ATCM PM10 standard.  S-1561 will have a fully offset federally enforceable limit of 50 hour/year allowed for testing and maintenance operation.

S-1561 emissions are summarized in the following table.

|  | Emission | Emission | Hourly | 24-hr Daily | Annual | Annual |
| --- | --- | --- | --- | --- | --- | --- |
|  | Factor | Factor | Emissions | Emissions | Emissions | Emissions |
| Pollutant | (g/hp-hr) | (lb/hp-hr) | (lbs/hour) | (lbs/day) | (lbs/yr) | (tons/year) |
| NOx | 2.55 | 0.00562 | 2.24 | 53.7 | 112 | 0.0559 |
| CO | 1.00 | 0.00221 | 0.88 | 21.1 | 43.9 | 0.0219 |
| POC | 0.18 | 0.00040 | 0.158 | 3.79 | 7.90 | 0.00395 |
| SO2 | 0.00177 | 0.0000039 | 0.0016 | 0.04 | 0.078 | 0.00004 |
| PM10 | 0.10 | 0.00022 | 0.088 | 2.106 | 4.388 | 0.00219 |

Notes: 	Maximum Annual Emissions are calculated using 50 hours/year allowed for testing and maintenance operation.

NOx, CO, POC and PM10 emissions factors are based on EPA "D2 Cycle Averages" certification.

SO2 emission factor is based on mass balance and 0.0015% sulfur in fuel by weight, and a maximum fuel rate of 19.4 gallons per hour.

C.	S-1562 and S-1563	East and West Avon Wharf Emergency Diesel Firewater Pump

East and West Fire Water Pump IC engines have criteria pollutant emissions from the combustion of diesel fuel include nitrogen oxides (NOx), sulfur dioxide (SO2), carbon monoxide (CO), precursor organic compounds (POC), and particulate matter less than 10 microns in diameter (PM10).  Emissions are estimated based the EPA “C1 Cycle Averages” Certification level for Engine Family CCPXL18.1NZS.

Sulfur dioxide is calculated based on 15 ppm sulfur content in diesel fuel.  This ultra-low diesel fuel is required to be used in California.  SO2 emissions are calculated by mass balance with 0.0015% sulfur in fuel by weight, using the engine peak fuel rate specification.  Emissions are estimated for a 700 brake horsepower (bhp) IC engine.  Annual emissions are limited to 70 hours of operation (testing and maintenance) each, to comply with the CARB ATCM PM10 standard.

The District normally does not permit an emergency standby firewater pump for more than 50 hours of testing and maintenance operation.  However, emergency firewater pumps are addressed separately in the ATCM.  93115.6(a)(4)(A)(1)(c) allows for the following:

Hours of operation limited to ‘hours necessary to comply with testing requirements of the National Fire Protection Association (NFPA) 25 – Standard for the Inspection Testing and Maintenance of Water-Based Fire Protection Systems,” 2002 edition, which is incorporated here by reference.’ No limit for emergency and emission testing for compliance with this regulation.

The NFPA 25, Chapter 8 Fire Pumps, Section 8.3 Testing, states that tests must be conducted weekly for a minimum of 30 minutes.  The Tesoro Fire Marshall has determined that allowing the IC firewater pump engines to run for one hour during each weekly testing operation  is better for the maintenance of the IC engines.  In addition to the weekly testing, annual hydraulic testing is required for several hours.  To allow for the required testing and to accommodate any additional testing associated with repairs, each engine will be permitted for 70 hours per year for testing and maintenance to ensure compliance with the NFPA 25 Standard.  S-1562 and S-1563 will each have a fully offset federally enforceable limit of 70 hour/year allowed for testing and maintenance operation.

S-1562 and S-1563 emissions for each engine are summarized in the following table.

|  | Emission | Emission | Hourly | 24-hr Daily | Annual | Annual |
| --- | --- | --- | --- | --- | --- | --- |
|  | Factor | Factor | Emissions | Emissions | Emissions | Emissions |
| Pollutant | (g/hp-hr) | (lb/hp-hr) | (lbs/hour) | (lbs/day) | (lbs/yr) | (tons/year) |
| NOx | 2.87 | 0.00633 | 4.43 | 106 | 310 | 0.155 |
| CO | 1.72 | 0.00379 | 2.65 | 63.7 | 186 | 0.093 |
| POC | 0.07 | 0.00015 | 0.108 | 2.59 | 7.6 | 0.0038 |
| SO2 | 0.00177 | 0.0000039 | 0.003 | 0.07 | 0.19 | 0.0001 |
| PM10 | 0.07 | 0.00015 | 0.108 | 2.59 | 7.6 | 0.0038 |

Notes: 	Maximum Annual Emissions are calculated using 70 hours/year allowed for testing and maintenance operation.

NOx, CO, POC and PM10 emissions factors are based on EPA "C1 Cycle Averages" certification.

SO2 emission factor is based on mass balance and 0.0015% sulfur in fuel by weight, and a maximum fuel rate of 35.9 gallons per hour.

D.	S-1564	Recovered Oil Fixed Roof Tank A-938

The annual emissions are calculated using EPA's TANKS 4.0.9d software.  The actual tank dimensions will be 6 feet in width, 16 feet long and 5.5 feet tall, with a capacity of 3,800 gallons.  TANKS 4.0.9d does not have a rectangular tank option, so a horizontal fixed roof tank was modeled with the appropriate height and a calculated “effective” diameter to match the total tank volume.  The TANKS software input is summarized in the following table.

| Tank Characteristics | Actual | Model Input |
| --- | --- | --- |
| Diameter, feet | 6 by 16 | 11.06 |
| Shell Height, feet | 5.5 | 5.5 |
| Maximum Fill Height, feet | 4.5 | 4.5 |
| Average Fill Height, feet | 4.5 | 4.5 |
| Working Volume, cubic feet | 45.27 | 45.27 |
| Working Volume, gallons | 3234.04 | 3234.04 |
| Shell Color | White | White |
| Cone Height | 0 (flat roof) | 0 |
| Cone Slope (ft/ft) | 0 | 0.06 (default) |

The material in the tank will contain some gasoline, diesel, and other hydrocarbon material that originates in the Berth 1A drain system (e.g., a few gallons drained from the loading/unloading hoses after material transfer to/from the cargo vessel is completed).  The majority of the liquid will be water from Berth 1A rain runoff.  Even though the tank will likely contain mostly water, organic material will be less dense and 'float' on the surface of the stored liquid.  The emissions estimate is conservative based on 250,000 gallons/year of gasoline with a maximum true vapor pressure of 11 psia.  S-1564 will have a fully offset federally enforceable throughput limit of 250,000 gallons per year.

The results of the TANKS emissions estimate are summarized in the following table.

|  | S-1564 Uncontrolled POC Emissions |  |
| --- | --- | --- |
| Source | Average Daily lbs/day | Annual, lbs/year |
| S-1564, Tank A-938, Max TVP = 11 psia | 8.67 | 3166 |

The average daily emissions are calculated by dividing the annual emissions by 365 days/year.  However, the EPA Tanks program is not intended to calculate short term emissions, so the average daily emission rate needs further analysis.

The highest day emissions are calculated as follows, using tank volume and assuming on the highest day, the tank will be near empty, will contain hydrocarbon gas created by the organic material floating on the tank contents, the tank will be completely filled within a 24-hour period, and the liquid filling the tank will displace the gas in the tank.

Maximum Daily Emissions =

3800 gal/day / 7.48 gal/cf *16 lb/lb-mole / 379 cf/lb-mole = 21.4 lb/day

Therefore, the S-1564 emissions are summarized in the following table.

|  | S-1564 Uncontrolled POC Emissions |  |  |
| --- | --- | --- | --- |
| Source | Maximum Daily lbs/day | Annual, lbs/year | Annual, tons/year |
| S-1564, Tank A-938, Max TVP = 11 psia | 21.4 | 3166 | 1.583 |

E. 	Project Fugitive Emissions

Fugitive emission calculations are based on the projected count of fugitive components, subdivided into process service, and applying the unit component emission factor that represents each components potential to emit.  These potential to emit emission factors are derived from Method 3 Correlation Equation of the 1999 CAPCOA guideline (Reference Table IV-3a) using the Regulation 8, Rule 18 Equipment Leak Standards.

While Regulation 8, Rule 44, Section 305 Equipment Leaks is the regulatory requirement for Berth 1A, Tesoro has agreed to meet the requirements of Regulation 8, Rule 18, Equipment Leaks, which are more stringent.  The following table summarizes the emission factors used to estimate fugitive emissions.

| Component | Service | Correlation Equation (kg/hr) | 8-18 Leak Standard (ppm) | Emission Factor kg/hr | Emission Factor lb/hr |
| --- | --- | --- | --- | --- | --- |
| Valves | All | 2.27E-06(ppm)0.747 | 100 | 7.08E-05 | 1.56E-04 |
| Pump seals | All | 5.07E-05(ppm)0.622 | 500 | 2.42E-03 | 5.33E-03 |
| Connectors | All | 1.53E-06(ppm)0.736 | 100 | 4.54E-05 | 1.00E-04 |
| Flanges | All | 4.53E-06(ppm)0.706 | 100 | 1.17E-04 | 2.58E-04 |
| Other (PRV) | All | 8.69E-06(ppm)0.642 | 500 | 4.70E-04 | 1.04E-03 |

Total estimated fugitive components for the project have been provided by Tesoro, and are summarized in the following table.

| Component Type | Service Type | Potential Emission Factor (lb/hour/source) | S-1560 Berth 1A | A-1560 Vapor Recovery | S-1564 Slop Tank | Total Compo-nents | Emissions lb/day |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Valves | Gas/Vapor | 1.56E-04 | 0 | 12 | 0 | 12 | 0.045 |
|  | Light Liquid | 1.56E-04 | 25 | 0 | 4 | 29 | 0.109 |
|  | Heavy Liquid | 1.56E-04 | 17 | 0 | 0 | 17 | 0.064 |
| Flanges/ Connectors | Gas/Vapor | 2.58E-04 | 0 | 64 | 0 | 64 | 0.396 |
|  | Light Liquid | 2.58E-04 | 40 | 0 | 14 | 54 | 0.334 |
|  | Heavy Liquid | 2.58E-04 | 5 | 0 | 0 | 5 | 0.031 |
| PSV's | Gas/Vapor | 1.04E-03 | 0 | 1 | 0 | 1 | 0.025 |
|  | Light Liquid | 1.04E-03 | 2 | 0 | 3 | 5 | 0.124 |
|  | Heavy Liquid | 1.04E-03 | 3 | 0 | 0 | 3 | 0.075 |
| Pumps | Light Liquid | 5.33E-03 | 6 | 0 | 2 | 8 | 1.024 |
|  | Heavy Liquid | 5.33E-03 | 0 | 0 | 0 | 0 | 0 |
| Total Fugitive Emissions |  |  |  |  | Lbs/Day |  | 2.227 |
|  |  |  |  |  | Lbs/Year |  | 813 |
|  |  |  |  |  | Tons/Year |  | 0.406 |

F. 	Berth 1 Marine Cargo Carrier Emissions

The Berth 1 POC Emissions during the 3-year baseline period were derived in Part A above, and are summarized in the following table.

| Year | Month | S-100 Berth 1 3-Year Baseline Emissions Total |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | NOX | CO | POC | PM10 | SO2 |
| Baseline Total, lbs |  | 252,727 | 50,167 | 14,942 | 6,138 | 11,253 |
| Annual Average, lbs |  | 84,242 | 16722 | 4,981 | 2,046 | 3,751 |
| Annual Average, tons |  | 42.121 | 8.361 | 2.490 | 1.023 | 1.876 |

G. 	Berth 1 Material Loading Operations Emissions

Berth 1 Emissions from Loading Operations were estimated in Part A.2 above, and are summarized in the following table:

|  | Total Quantity Loaded During 3-Year Baseline, Barrels | Uncontrolled Emissions, lbs | Controlled Emissions, lbs |
| --- | --- | --- | --- |
| Total 3-Year Baseline | 18,657,550 | 104,849 | 5,359 |
| Annual Average | 6,219,183 | 34,950 | 1,786 |
| Annual Average Loading Operations POC Emissions, tons |  | 17.475 | 0.893 |

H. 	Berth 1 Vapor Recovery System Emissions

The Berth 1 Vapor Recovery System was originally permitted in 1987 via Permit Application 31718.  In this permit application, the vapor recovery system was constructed and the emission reductions were banked (Bank #83).  In the calculations that determined the 260.8 tons of total emission reductions, 5.2 tons of fugitive emissions for the vapor recovery system were deducted.  Therefore, the fugitive emissions for the Berth 1 Vapor Recovery System are fully offset.

The offset fugitive emissions estimated in Application 31718 need to be RACT adjusted to determine the contemporaneous emission reductions.  This adjustment is calculated by applying the current PTE emission factors to the fugitive component counts that were the basis for the 1987 emissions.  The following table summarizes the Berth 1 Vapor Recovery System fugitive emissions.

| Component Type | Service Type | Potential Emission Factor (lb/hour/source) | Application 31718 Component Counts | Emissions lb/day |
| --- | --- | --- | --- | --- |
| Valves | Gas/Vapor | 1.56E-04 | 14 | 0.0524 |
|  | Light Liquid | 1.56E-04 | 2 | 0.0075 |
|  | Heavy Liquid | 1.56E-04 | 0 | - |
| Flanges/ Connectors | Gas/Vapor | 2.58E-04 | 32 | 0.1981 |
|  | Light Liquid | 2.58E-04 | 0 | - |
|  | Heavy Liquid | 2.58E-04 | 0 | - |
| PSV's | Gas/Vapor | 1.04E-03 | 0 | - |
|  | Light Liquid | 1.04E-03 | 3 | 0.0749 |
|  | Heavy Liquid | 1.04E-03 | 0 | - |
| Pumps | Light Liquid | 5.33E-03 | 1 | 0.1279 |
|  | Heavy Liquid | 5.33E-03 | 0 | 0 |
| Total Fugitive Emissions |  | Lbs/Day |  | 0.4608 |
|  |  | Lbs/Year |  | 168.21 |
|  |  | Tons/Year |  | 0.0841 |

#### TOTAL PROJECT CUMULATIVE INCREASE

The following table summarized the cumulative increase for this project.

| Avon MOTEMS Project Cumulative Increase, Tons |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | NOX | CO | POC | PM10 | SO2 |
| Emissions for New Sources |  |  |  |  |  |
| S-1560 Marine Cargo Carrier | 188.8245 | 34.425 | 10.743 | 4.157 | 9.372 |
| S-1560 Material Loading |  |  | 20.000 |  |  |
| S-1561 Diesel Generator | 0.056 | 0.022 | 0.004 | 0.002 | 0 |
| S-1562 Diesel Firewater Pump | 0.155 | 0.093 | 0.004 | 0.004 | 0 |
| S-1563 Diesel Firewater Pump | 0.155 | 0.093 | 0.004 | 0.004 | 0 |
| S-1564 Recovered Oil Tank |  |  | 1.583 |  |  |
| Fugitive Emissions |  |  | 0.406 |  |  |
| Total Emissions for New Sources | 189.191 | 34.633 | 32.744 | 4.167 | 9.372 |
| Contemporaneous Emissions Reductions |  |  |  |  |  |
| S-100 Berth 1 Cargo Carrier Emissions | 42.121 | 8.361 | 2.490 | 1.023 | 1.876 |
| S-100 Berth 1 Loading Operations |  |  | 0.893 |  |  |
| S-100 Berth 1 Vapor Recovery System |  |  | 0.084 |  |  |
| Total Contemporaneous Emissions Reductions | 42.121 | 8.361 | 3.467 | 1.023 | 1.876 |
| Total Cumulative Increase |  |  |  |  |  |
| Project Totals | 147.070 | 26.272 | 29.277 | 3.144 | 7.496 |

#### EMISSIONS OF TOXIC AIR CONTAMINENTS (TACs)

A summary of the TAC emissions applicable to each source is shown in the following table.

| TAC | S-1560 Marine Cargo Carrier | S-1560 Loading | S-1561 Genera-tor | S-1562 East Pump | S-1563 West Pump | S-1564 Slop Tank | Fugitives |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel Particulate Matter | X |  | X | X | X |  |  |
| Toluene |  | X |  |  |  | X | X |
| Ethylbenzene |  | X |  |  |  | X | X |
| Xylene (Total) |  | X |  |  |  | X | X |
| Benzene |  | X |  |  |  | X | X |
| Naphthalene |  | X |  |  |  | X | X |
| n-Hexane |  | X |  |  |  | X | X |
| Propylene |  |  |  |  |  |  | X |

A.	Cargo Carrier TAC Emissions

Diesel Particulate emissions for Berth 1A total 8,313 lbs annually.  However, only emissions from Cargo Carrier hoteling are evaluated for a project risk screen.  The total Berth 1A diesel particulate emissions during cargo carrier hoteling are summarized in the following table.

| Description | Units | Value |
| --- | --- | --- |
| Total Cargo Carrier Hoteling PM10 During 3-Year Baseline | Lbs | 2507 |
| Total 3-Year Baseline Tug Emissions During Cargo Carrier Hoteling | Lbs | 28 |
| Total 3-Year Baseline PM10 Hoteling Emissions | Lbs | 2535 |
| Total 3-Year Baseline PM10 Emissions | Lbs | 6138 |
| Percent PM10 Hoteling Emissions of Total PM10 Emissions | % | 41.30 |
| Total Future S-1560 Berth 1A PM10 Emissions (4.1565 tons/year) | Lbs/Year | 8313 |
| Total Future S-1560 Berth 1A Diesel Particulate Hoteling Emissions | Lbs/Year | 3433 |

B.	S-1560 TAC Emissions from Loading Operations

Total Loading Operations Emissions during the 3-year baseline period were estimated in Part A.2 of the Emissions Calculations section above.  Emissions from gasoline loading will be recovered to be used as refinery fuel gas remote from Berth 1A.  Emissions from loading operations of other material during the baseline period (almost all Diesel) are emitted at Berth 1A.  The loading operation emissions during the 3-year baseline period are summarized in the following table.

| Description | Units | Value |
| --- | --- | --- |
| Total 3-year Baseline Loading Operations Emissions | Lbs | 5359 |
| Total 3-Year Baseline Loading Emissions, Gasoline to Tanker | Lbs | 840 |
| Total 3-Year Baseline Loading Emissions, Gasoline to Barge | Lbs | 675 |
| Total 3-Year Baseline Gasoline Loading Emissions | Lbs | 1515 |
| Total 3-Year Baseline Gasoline Loading Emissions % of Total | % | 28.28 |
| Total 3-Year Baseline Loading Emissions, Diesel to Tanker | Lbs | 3240 |
| Total 3-Year Baseline Loading Emissions, Diesel to Barge | Lbs | 602 |
| Total 3-Year Baseline Diesel Loading Emissions | Lbs | 3842 |
| Total 3-Year Baseline Diesel Loading Emissions % of Total | % | 71.72 |

The total loading operations emissions for S-1560 Berth 1A, based on the proportions during the 3-year baseline (shown in the previous table) and the 20 ton/year POC loading operation s are summarized in the following table.

| Description | Units | Value |
| --- | --- | --- |
| Total S-1560 Loading Operations Emissions (based on 20 ton/yr limit) | Lbs/Year | 40,000 |
| Total Gasoline Loading Emissions % of Total | % | 28.28 |
| Total Diesel Loading Emissions % of Total | % | 71.72 |
| Total Gasoline Loading Emissions | Lbs/Year | 11,312 |
| Total Diesel Loading Emissions | Lbs/Year | 28,688 |

However, to conservatively estimate TAC emissions for loading operations, Tesoro has requested the TAC emissions be based on the following criteria:

| Description | POC Basis tpy | POC Basis lb/year |
| --- | --- | --- |
| Total S-1560 Loading Operations Emissions (based on 20 ton/yr limit) | 20 | 40,000 |
| Maximum Expected Gasoline Loading Emissions | 20 | 40,000 |
| Maximum Expected Diesel Loading Emissions | 5 | 10,000 |

TAC composition of gasoline and diesel are specified on the material MSDSs.  Composition of the loading operation vapor emissions are determined using Raoult's Law applied to the liquid material composition.  This was done when the emissions of S-1564 Recovered Oil Tank were evaluated, and the TAC compositions of gasoline and diesel were included in the TANKS 4.0d estimates (see Part E below).  The results of these tank emission estimates include the tank emissions TAC composition.  The gasoline and diesel compositions are summarized in the following table.

| TAC | Gasoline wt% | Diesel wt% | Gasoline Vapor wt% | Diesel Vapor wt% |
| --- | --- | --- | --- | --- |
| Toluene | 30% |  | 1.717% |  |
| Ethylbenzene | 5.0% |  | 0.095% |  |
| Xylene (Total) | 30% | 2% | 0.474% | 37.2% |
| Benzene | 1.3% |  | 0.261% |  |
| Naphthalene | 5% | 1% | 0.0018% | 0.471% |
| n-Hexane | 0.75% |  | 0.246% |  |

The annual TAC emissions from the S-1560 Loading Operations are summarized in the following table.

| TAC | Gasoline Vapor wt% | Gasoline Loading Emissions, lb/year | Diesel Vapor wt% | Diesel Loading Emissions, lb/year | Total Loading Emissions, lb/year |
| --- | --- | --- | --- | --- | --- |
| Toluene | 1.717% | 687 |  |  | 687 |
| Ethylbenzene | 0.095% | 38.0 |  |  | 38 |
| Xylene (Total) | 0.474% | 190 | 37.2% | 3,720 | 3,910 |
| Benzene | 0.261% | 104 |  | 0 | 104 |
| Naphthalene | 0.0018% | 0.720 | 0.471% | 47.1 | 48 |
| n-Hexane | 0.246% | 98.40 |  | 0 | 98 |
| Total POC lbs/year |  | 40,000 |  | 10,000 | 40,000 |

Hourly TAC emissions can be derived by an evaluation of each loading operation in the 3-year baseline period.  Using each loading operation calculated emissions and dividing it by the cargo carrier hoteling duration, a loading operation emission rate in lb/hr can be derived.  The peak emission rate for gasoline loading occurred during a loading operation on beginning on 5/6/2014 and is 5.46 lb/hr.  The peak emission rate for diesel loading occurred during a loading operation beginning on 5/17/2014 and is 2.42 lb/hr.

The maximum hourly TAC emissions from S-1560 Loading Operations are summarized in the following table.

| TAC | Gasoline Vapor wt% | Gasoline Loading Emissions, lb/hour | Diesel Vapor wt% | Diesel Loading Emissions, lb/hour | Total Loading Emissions, lb/hour |
| --- | --- | --- | --- | --- | --- |
| Toluene | 1.717% | 0.09380 |  |  | 0.0938 |
| Ethylbenzene | 0.095% | 0.00519 |  |  | 0.0052 |
| Xylene (Total) | 0.474% | 0.02589 | 37.2% | 0.900 | 0.9255 |
| Benzene | 0.261% | 0.01426 |  |  | 0.0143 |
| Naphthalene | 0.0018% | 0.00010 | 0.471% | 0.0114 | 0.0115 |
| n-Hexane | 0.246% | 0.01344 |  |  | 0.0134 |
| Total lbs/hour |  | 5.46 |  | 2.42 |  |

Gasoline and diesel loading operations can occur simultaneously (there were 7 cargo carrier calls during the 3-year baseline where both gasoline and diesel were loaded onto the cargo carrier).

The gasoline loading portion of the emissions from S-1560 Berth 1A will be recovered by A-1560 Marine Vapor Recovery System, and abated by the furnaces that burn 40# fuel gas (S-908, S-909 and S-912 in the process block of the refinery).  Therefore, only the diesel loading TACs are emitted at S-1560.

C.	Total S-1560 TAC Emissions

The total TAC emissions from S-1560 Berth 1A are summarized in the following table.

| TAC | Cargo Carrier Hoteling, lbs/year | Loading Operations, lb/year | Total S-1560 TAC Emissions lb/year | Loading Emissions, lb/hour |
| --- | --- | --- | --- | --- |
| Diesel PM | 3433 |  | 3433 |  |
| Toluene |  | 687 | 687 | 0.0938 |
| Ethylbenzene |  | 38 | 38 | 0.0052 |
| Xylene (Total) |  | 3,910 | 3,910 | 0.9255 |
| Benzene |  | 104 | 104 | 0.0143 |
| Naphthalene |  | 48 | 48 | 0.0115 |
| n-Hexane |  | 98 | 98 | 0.0134 |

D.	Diesel Engine TAC Emissions: S-1561, S-1562 and S-1563

Diesel particulate emissions for the new engines were calculated in Parts B and C of the Emissions Calculations section above.  The following table summarizes these emissions.

| TAC | S-1561 Diesel Generator, lbs/year | S-1562 Diesel Firewater Pump, lbs/year | S-1563 Diesel Firewater Pump, lbs/year | Total Diesel Engine lb/year |
| --- | --- | --- | --- | --- |
| Diesel PM | 4.39 | 7.60 | 7.60 | 19.59 |

E.	S-1564 Recovered Oil Tank TAC Emissions

S-1564 emissions were estimated in Part D of the Emissions Calculations section above.  These emissions were estimated using EPA's TANKS 4.09d program.  The input information for this program included TAC concentrations of gasoline and diesel.  The TANKS program calculates vapor emission compositions.  Both the liquid material concentrations and the vapor emission compositions are summarized in the following table.

| TAC | Gasoline wt% | Diesel wt% | Gasoline Vapor wt% | Diesel Vapor wt% |
| --- | --- | --- | --- | --- |
| Toluene | 30% |  | 1.717% |  |
| Ethylbenzene | 5.0% |  | 0.095% |  |
| Xylene (Total) | 30% | 2% | 0.474% | 37.2% |
| Benzene | 1.3% |  | 0.261% |  |
| Naphthalene | 5% | 1% | 0.0018% | 0.471% |
| n-Hexane | 0.75% |  | 0.246% |  |

The total POC emissions estimated by the TANKS program are summarized in the following table.

| Pollutant | Emissions from Diesel Storage | Emissions from Gasoline Storage |
| --- | --- | --- |
|  | Lb/yr | Lb/yr |
| Total POC | 4.25 | 3,166 |

Daily POC emissions were estimated to be 21.4 lb/day (estimated in Part D of the Emissions Calculations section above).

Hourly emissions would be based on the maximum fill rate specified for this tank.  This is specified at 8400 gallons/hr.  However, the tank capacity is  3800 gallons, therefore the tank can be filled in less than an hour.  The displaced gas from filling the tank volume was estimated when deriving the maximum daily emissions of 21.4 lb/day.  Based on the maximum fill rate for the tank, the maximum hourly emissions rate is also 21.4 lbs.

TAC emissions estimated by the TANKS program are summarized in the following table.

| Chemical | Diesel TANKS | Gasoline TANKS | Worst Case Material |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Lb/yr | Lb/yr | Lb/yr | lb/day | lb/hr |
| POC | 4.25 | 3,165.61 | 3,165.61 | 21.40 | 21.40 |
| Toluene |  | 56.95 | 56.95 | 3.85E-01 | 3.85E-01 |
| Ethylbenzene |  | 3.15 | 3.15 | 2.13E-02 | 2.13E-02 |
| Xylene (Total) | 1.58 | 15.73 | 15.73 | 1.06E-01 | 1.06E-01 |
| Benzene |  | 8.67 | 8.67 | 5.86E-02 | 5.86E-02 |
| Naphthalene | 0.02 | 0.06 | 0.06 | 4.06E-04 | 4.06E-04 |
| Hexane, n- |  | 8.16 | 8.16 | 5.52E-02 | 5.52E-02 |

F.	Fugitive TAC Emissions

Total fugitive POC emissions were calculated in Part E of the Emission Calculations section above.  Fugitive TAC emissions are estimating using the approach summarized in the following table.

| Device | S-1560 |  | A-1560 | S-1564 |
| --- | --- | --- | --- | --- |
| Service | Light Liquid | Heavy Liquid | Gas/Vapor | Light Liquid |
| Calculation Approach | Composition of Liquid Gasoline | Composition of Liquid Diesel | Enrichment Gas Composition | Composition of Liquid Gasoline |
| TAC Concentrations |  |  |  |  |
| Toluene | 30% |  |  | 30% |
| Ethylbenzene | 5.0% |  |  | 5.0% |
| Xylene | 30% | 2% |  | 30% |
| Benzene | 1.3% |  |  | 1.3% |
| Naphthalene | 5% | 1% |  | 5% |
| Hexane, n- | 0.75% |  |  | 0.75% |
| Propylene |  |  | 50% |  |

Applying the TAC concentrations above to the total fugitive POC emissions results in the fugitive TAC emissions summarized in the following table.

| Device | S-1560 |  | A-1560 | S-1564 | Total Emissions |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Service | Light Liquid | Heavy Liquid | Gas/Vapor | Light Liquid |  |  |  |
| Pollutant | lbs/day | lbs/day | lbs/day | lbs/day | lbs/day | Lbs/hr | Lb/year |
| Total POC | 1.159 | 0.169 | 0.466 | 0.432 | 2.226 | 0.093 | 813 |
| Toluene | 0.348 | - | - | 0.130 | 0.477 | 0.0199 | 174.15 |
| Ethylbenzene | 0.058 | - | - | 0.022 | 0.080 | 0.0033 | 29.03 |
| Xylene | 0.348 | 0.003 | - | 0.130 | 0.481 | 0.0200 | 175.39 |
| Benzene | 0.015 | - | - | 0.006 | 0.021 | 0.0009 | 7.55 |
| Naphthalene | 0.058 | 0.002 | - | 0.022 | 0.081 | 0.0034 | 29.64 |
| Hexane, n- | 0.009 | - | - | 0.003 | 0.012 | 0.0005 | 4.35 |
| Propylene | - | - | 0.233 | - | 0.233 | 0.0097 | 85.03 |

G.	Total Project TAC Emissions

The total project annual TAC emissions are summarized in the following table.

| Total Project TAC Emissions, lbs/year |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAC | S-1560 Marine Cargo Carrier | S-1560 Loading Operations |  | S-1561 Genera-tor | S-1562 East Pump | S-1563 West Pump | S-1564 Slop Tank | Fugitives | Total |
|  |  | Gasoline | Diesel |  |  |  |  |  |  |
| Diesel PM | 3433 |  |  | 4.39 | 7.60 | 7.60 |  |  | 3453 |
| Toluene |  | 687 |  |  |  |  | 56.95 | 174.15 | 918 |
| Ethylbenzene |  | 38.0 |  |  |  |  | 3.15 | 29.03 | 70 |
| Xylene (Total) |  | 190 | 3720 |  |  |  | 15.73 | 175.39 | 4101 |
| Benzene |  | 104 |  |  |  |  | 8.67 | 7.55 | 120 |
| Naphthalene |  | 0.720 | 47.1 |  |  |  | 0.06 | 29.64 | 78 |
| n-Hexane |  | 98.40 |  |  |  |  | 8.16 | 4.35 | 111 |
| Propylene |  |  |  |  |  |  |  | 85.03 | 85 |

The total project hourly TAC emissions are summarized in the following table.  Only TACs with acute toxic triggers are included.

| Total Project TAC Emissions, lbs/hour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAC | S-1560 Marine Cargo Carrier | S-1560 Loading Operations |  | S-1561 Genera-tor | S-1562 East Pump | S-1563 West Pump | S-1564 Slop Tank | Fugitives | Total |
|  |  | Gasoline | Diesel |  |  |  |  |  |  |
| Toluene |  | 0.0938 |  |  |  |  | 0.3850 | 0.0199 | 0.4987 |
| Xylene (Total) |  | 0.02589 | 0.900 |  |  |  | 0.1060 | 0.0200 | 1.0515 |
| Benzene |  | 0.01426 |  |  |  |  | 0.0586 | 0.0009 | 0.0737 |

HEALTH RISK ASSESSMENT

A comparison of the total project TAC emissions and the toxic triggers of Regulation 2, Rule 5 are shown in the following table.

| TAC | Emissions |  | Regulation 2-5 Trigger Levels |  |  |
| --- | --- | --- | --- | --- | --- |
|  | lb/hour | lb/year | Acute, lb/hr | Chronic, lb/year | Exceeds Trigger? |
| Diesel PM | NA | 3453 | NA | 0.34 | Yes |
| Toluene | 0.4987 | 918 | 82 | 12000 | No |
| Ethylbenzene | NA | 70 | NA | 43 | Yes |
| Xylene (Total) | 1.0515 | 4,101 | 49 | 27000 | No |
| Benzene | 0.0737 | 120 | 2.9 | 3.8 | Yes |
| Naphthalene | NA | 78 | NA | 3.2 | Yes |
| Hexane, n- | NA | 111 | NA | 270000 | No |
| Propylene | NA | 85 | NA | 120000 | No |

The Health Risk Assessment was conducted and the results transmitted January 7, 2017.  The project risk is considered acceptable with a Maximum Cancer Risk of 3.4 chances in a million, the chronic hazard index of 0.001 and the acute hazard index of 0.019.  These results demonstrate the project meets the requirements of Regulation 2-5-302.  In addition, most of the cancer risk is associated with the diesel particulate emissions from the cargo carriers.  Pursuant to Regulation 2-5-205, cargo carrier emissions are not subject to TBACT.

BEST AVAILABLE CONTROL TECHNOLOGY

In accordance with Regulation 2, Rule 2, Section 301, Best Available Control Technology (BACT) is triggered for any new or modified source with the potential to emit 10 pounds or more per highest day of POC, NPOC, NOx, CO, SO2 or PM10.  Cargo carrier emissions are not subject to Regulation 2, Rule 2, Section 301 BACT.

Based on the emission calculations above, the following sources are subject to a BACT determination.

S-1560 Berth 1A loading operations exceed 10 lb/day POC emissions (re: Emission Calculations Part A.2)

S-1561 Diesel Generator exceeds 10 lb/day NOx and CO emissions (re: Emission Calculations Part B)

S-1562 Diesel Firewater Pump exceeds 10 lb/day NOx and CO emissions (re: Emission Calculations Part C)

S-1563 Diesel Firewater Pump exceeds 10 lb/day NOx and CO emissions (re: Emission Calculations Part D)

S-1564 Recovered Oil Tank exceeds 10 lb/day POC emissions (re: Emission Calculations Part E)

A.	BACT for S1560 Avon Berth 1A Loading Operations

The daily POC emissions for gasoline loading exceed 10 pounds per day.  Therefore the requirements for BACT are triggered for POC.  BACT for marine loading of regulated materials for POC is vapor recovery and control system: compressor, condenser, and thermal incinerator w/ a destruction efficiency >98.5%.  A new marine vapor recovery system that is functionally identical to the one on S-100 Berth 1 is being installed as abatement system A-1560.  Vapors collected by the marine vapor recovery system are routed to A-14 No. 1 Gas Plant and the 40 pound fuel gas system.  The BACT/TBACT Guidelines Document 107.1 covers S-1560.

Loading emissions controls are expected to exceed the BACT 98.5% capture and control requirements.  Therefore, the S-1560 Berth 1A Loading Operations are considered to satisfy BACT.

B.	BACT for S-1561 Emergency Standby Diesel IC Engine, S-1562 East and S-1563 West Fire Water Pump IC Engines

The S-1561 daily NOx and CO emissions exceed 10 pounds per day.  Therefore the requirements for BACT are triggered for NOx and CO.  All other pollutants are emitted at rates less than 10 pounds per maximum day.  The BACT/TBACT Guidelines Document 96.1.3 covers S-1561.

The BACT requirement for NOx and CO is compliance with the emission standard allowed in the current CARB or EPA non-road tier standard for the appropriate horsepower range and the model year.  The emergency standby diesel IC engines are certified engines that will comply with the CARB ATCM Limits as summarized in the following table.

|  | S-1561 Emission | CARB/ATCM Standards |
| --- | --- | --- |
|  | Factor | 398 HP |
| Pollutant | (g/hp-hr) | (g/hp-hr) |
| NOx | 2.55 |  |
| CO | 1.00 | 2.6 |
| POC | 0.18 |  |
| NOx+POC | 2.73 | 3.0 |
| SO2 | 0.00177 |  |
| PM10 | 0.10 | .15 |

The S-1562 and S-1563 Firewater Pumps daily NOx and CO emissions exceed 10 pounds per day.  Therefore the requirements for BACT are triggered for NOx and CO.  All other pollutants are emitted at rates less than 10 pounds per maximum day.  The BACT/TBACT Guidelines Document 96.1.3 excludes firewater pumps because firewater pump engines were often not as clean as other emergency engines.  However, S-1562 and S-1563 are newer engines, and comply with the CARB/ATCM Limits, as summarized in the following table.

|  | S-1562, S-1563 Emission | CARB/ATCM Standards |
| --- | --- | --- |
|  | Factor | 700 HP |
| Pollutant | (g/hp-hr) | (g/hp-hr) |
| NOx | 2.87 |  |
| CO | 1.72 | 2.6 |
| POC | 0.07 |  |
| NOx+POC | 2.94 | 3.0 |
| SO2 | 0.00177 |  |
| PM10 | 0.07 | .15 |

Therefore, S-1561, S-1562 and S-1563 are considered to satisfy BACT.

C.	BACT for S-1564 Recovered Oil Tank A-938

The daily POC emissions could exceed 10 pounds per highest day.  A BACT Determination was conducted for POC.

The following table summarizes the Tesoro BACT determination for S-1564.

| POC BACT 1 Determination |  |  |  |
| --- | --- | --- | --- |
| Technology | Technically Feasible | Cost Effective | Rationale |
| Refrigerated Condensers | No | N/A | Typically only small quantities of hydrocarbons are transferred into the tank during line draining. Therefore, the majority of the time S-1564 is in service concentrations of hydrocarbons vapors that are below concentrations which a refrigerated condenser system could effectively capture hydrocarbon vapors. VOC concentrations must be at least 1% to 2% by volume for a refrigerated condenser system to work. A refrigerated condenser system would frequently ice up due to the high humidity and frequent occurrence of fog in the location where the wharf is located. |
| Recovery with A-1560 Berth 1A Marine Vapor Recovery System | No | N/A | Emissions from S-1564 are continuous and often below the lower explosion limit, making recovery incompatible. The A-1560 is designed to recover cargo carrier emissions from regulated (e.g., gasoline) loading operations. The displacement vapor from the marine tank often contains oxygen and is enriched with propane/propylene to stay above the upper explosion limit. A-1560 operates infrequently, batch-wise only when a regulated material is being loaded. Treating a high oxygen content continuous gas is not feasible. |
| Incineration | No | N/A | An incinerator would either function improperly due to the low flow rates or draw in sufficient ambient air to meet the incinerator minimum flow requirements and burn an excessive amount of auxiliary fuel. There are also safety concerns about locating an incinerator on the wharf because an incinerator could act as a source of ignition at times hydrocarbon vapors are present. US Coast Guard regulations at 33 CFR 154.2109 specify that a vapor destruction unit cannot be located within 30 meters of any tank vessel, berth or mooring at a facility. |
| Vapor Balancing | No | N/A | Vapor balancing is used when gasoline vapors from a tank are transferred to the tanker trailer delivering the gasoline as gasoline flows from the tanker into station’s storage tanks. There are no vessels associated with S-1564, therefore vapor balancing is not feasible. |
| Carbon Adsorption | Yes | No | The emission control cost is $63,650 per ton of POC. The BACT/TBACT workbook “Maximum Cost Guidelines for BACT” currently specifies that controls for POC cost less than $17,500 per ton removed to be cost effective. Therefore, use of activated carbon canisters to control emissions from Tank A-938 (S-1564) is not cost effective. |

Carbon absorption is technically feasible. Therefore a cost effective analysis is needed.  Tesoro estimated the following costs for installing Carbon Absorption system:

- $20,000 for the piping manifold and connecting hosed for an activated carbon canister system.

- $1,253 for each activated carbon canister.

- $8.75 to replace each canister based on 15 minutes and $35/hr.

- $2.25 disposal cost for each 180 lb canister based on $25/ton

A typical carbon adsorption system consists of two beds in series, a lead canister followed by a trim canister.  Once hydrocarbon breakthrough is detected from the lead canister, it is replaced with a new canister.  The trim canister is switched to the lead service and the new canister is installed as the trim bed.

The capital recovery factor, based on a 4% interest rate (10 Year Treasury Bond rate + 2, rounded up), and a 10 year equipment life, the capital recovery factor is 0.1233.

Simplified BAAQMD cost factors are 0.01 for Tax, 0.01 for Insurance, and 0.02 General and Administrative Expenses.  This totals to a 0.04 simplified factor.

The specification from the Calgon Ventsorb carbon canister estimates 41.4 lbs of hydrocarbon is absorbed for each 180 lbs of carbon.  This is based on an inlet concentration of 1000 ppm.

US EPA’s TANKS emission estimation software, Version 4.09d, was used to estimate volatile organic compound (VOC) emissions from the proposed storage tank. The estimated annual VOC emission rate for S-1564 is 3,166 pounds per year.  A 99% abatement factor is equivalent to a 3134 lbs POC reduction, or 1.567 tons.

Number of annual canisters = 3166 lbs / (41.7 lbs/canister) = 75.9 canisters per year.

The capital cost for the installation is $2506 for two canisters and $20,000 for the piping installation, or a total of $22,506.  Applying the 0.1233 capital recovery factor results in annual cost of $2,775.

The annualized cost of this installation and cost effectiveness are summarized in the following table.

| Description | Calculation | Amount |
| --- | --- | --- |
| Capital | Capital Cost X Recovery Factor | $2775 |
| Replacement Canisters | 76 canisters @ $1,253 each | $95228 |
| Replacement Installation | 76 canisters @ 8.75 each | $665 |
| Spent Canister Disposal Fee | 76 canisters @ $2.25 each | $171 |
| Tax, Insurance, and Administrative Expenses | Capital Cost X 0.04 | $900 |
| Total Annualized Cost |  | $99,739 |
| Annual POC Reduction, tons |  | 1.567 |
| Control Cost Effectiveness, $/ton |  | $63,650 |
| BACT Cost Effectiveness Limit, $/ton |  | $17,500 |
| Is Carbon Adsorption Cost Effective? |  | No |

The emission control cost is about $64,000 per ton of POC. The BACT/TBACT workbook “Maximum Cost Guidelines for BACT” currently specifies that controls for POC cost less than $17,500 per ton removed to be cost effective. Therefore, the BACT1 use of activated carbon canisters to control emissions from S-1564 is not cost effective.

Since BACT1 is not cost effective, BACT2 Achieved in Practice is required.  The BAAQMD BACT Handbook Document Number 167.2.1 covers Fixed Roof Organic Liquid Tanks with capacities under 20,000 gallons.  BACT2 is specified as a vapor recovery system with an overall efficiency of 95% or greater.  However, while S-1564 emissions were estimated conservatively assuming the tank is full of organic liquid, and required a permit for organic liquid storage, it actually will be storing mostly water from Berth 1A rain runoff.  There is no record of controls being achieved in practice for a 3,800 gallon tank storing mostly water.  Therefore, BACT2 control for S-1564 is not required.

OFFSETS

Offsets are required per Regulation 2-2-302 because Tesoro emits more than 35 tpy of POC and 35 tpy of NOx emissions.   Regulation 2-2-302 requires that offsets for POC and NOx be provided at a ratio of 1.15 to 1.0.  Regulation 2-2-303 requires that a Major Facility must pay offsets for PM10 and SO2 in excess of 1.0 ton per year for new or modified sources.  Regulation 2-2-303 requires that offsets for PM10 and SO2 be provided at a ratio of 1.0 to 1.0.

The total project cumulative increase,  and the offset requirements, are summarized in the following table.

| Total Cumulative Increase, tons |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | NOX | CO | POC | PM10 | SO2 |
| Total Emissions for New Sources | 189.191 | 34.633 | 32.744 | 4.167 | 9.372 |
| Total Contemporaneous Emissions Reductions | 42.121 | 8.361 | 3.467 | 1.023 | 1.876 |
| Project Offset Totals | 147.070 | 26.272 | 29.277 | 3.144 | 7.496 |
| Offset Ratio | 1.15:1.0 | NA | 1.15:1.0 | 1.0:1.0 | 1.0:1.0 |
| Required Offsets | 169.130 |  | 33.669 | 3.144 | 7.496 |
| Bank 915 | 8.063 |  |  |  | 3.525 |
| Bank 968 |  |  | 7.983 |  |  |
| Bank 1144 | 161.067 |  |  | 3.144 | 3.971 |
| Bank 1568 |  |  | 18.113 |  |  |
| Bank 1572 |  |  | 5.049 |  |  |
| Bank 1571 |  |  | 0.100 |  |  |
| Bank 1570 |  |  | 1.175 |  |  |
| Bank 1569 |  |  | 1.249 |  |  |

After the required offsets are surrendered, the banking certificates will have the following credits remaining.

| Remaining Credits, tons |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | NOX | CO | POC | PM10 | SO2 |
| Bank 915 | 0.000 | 2.938 | 0.000 | 0.000 | 0.000 |
| Bank 968 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Bank 1144 | 75.788 | 19.912 | 0.000 | 8.454 | 2893.006 |
| Bank 1568 | 0.000 | 0.000 | 0.434 | 0.000 | 0.000 |
| Bank 1572 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Bank 1571 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Bank 1570 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Bank 1569 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

New Source Performance Standards (NSPS)

S-1560 is not subject to any provisions of NSPS.

S-1561, S-1562 and S-1563 Diesel Engines are subject to NSPS 40 CFR 60 Subpart IIII, Standards of Performance for Stationary Compression Ignition Internal Combustion Engines, because they were manufactured after 4/1/2006.  The engines are EPA certified and are therefore expected to comply with the emission standards of NSPS Subpart IIII.

S-1564 Recovered Oil Tank is not subject to NSPS 40 CFR 60 Subpart Kb [Standards of Performance for Volatile Organic Liquid Storage Vessels (Including Petroleum Liquid Storage Vessels) for which Construction, Reconstruction, or Modification Commenced After July 23, 1984] because the tank is under 19,800 gallons in capacity.

National Emission Standards for Hazardous Air Pollutants (NESHAPS)

S-1560 is subject to 40 CFR 63 Subpart Y (NESHAP for Marine Vessel Loading of Organic Liquids) as required by 40 CFR 63 Subpart CC (NESHAP for Source Categories - Petroleum Refineries) 63.651 Marine Tank Vessel Loading Operation Provisions.  Tesoro will exceed the requirements of 40 CFR 63.562 Standards by reducing HAP emissions from loading operations by 98.5% or more.

S-1561, S-1562 and S-1563 Diesel Engines are subject to NESHAPS 40 CFR 63 Subpart ZZZZ NESHAP for Stationary Reciprocating Internal Combustion Engines (RICE).  Per 40 CFR 63.6590(c)(1), a new or reconstructed stationary RICE located at an major source must meet the requirements of 40 CFR 60 Subpart IIII, for compression ignition engines.  These engines are in compliance with the requirements of 40 CFR 60 Subpart IIII, as shown in the “NSPS” section of this evaluation.

S-1564 Recovered Oil Tank is subject to the requirements of 40 CFR 61 Subpart FF Benzene Waste Operations NESHAPS (BWON).  Tesoro complies with BWON 61.342(e)(2)(i) by treating waste streams to less than or equal to 6.0 Mg/year.  The existing S-1508 and S-1509 Berth 1 and Berth 5 Recovered Oil Tanks are also subject to BWON, permitted for a total of 1,689,000 barrels/year throughput (about 71,000,000 gal/yr).  Since these two existing tanks will be demolished or decommissioned, and the permitted throughput of S-1564 will be substantially lower at 250,000 gallons/year, the addition of S-1564 is not expected to affect Tesoro’s compliance with 40 CFR 61 Subpart FF.

S-1564 is subject to 40 CFR 63 Subpart CC (Petroleum Refinery MACT) since it is permitted to store materials that contain HAPS.  However, since its capacity is less than the 20,000 gallon minimum capacity that defines a Group 1 Storage Vessel in 63.641, S-1564 is a Group 2 Storage Vessel.  All the requirements of 63.660 Storage Vessel Provisions only apply to Group 1 Storage Vessels.  Therefore, S-1564 is exempt from the Storage Vessel Provisions of 40 CFR 63 Subpart CC.

CARB Stationary Diesel Engine Airborne Toxic Control Measure (ATCM)

S-1561 Emergency Diesel Generator is a new standby engine installed after January 1, 2005.  Therefore, S-1561 is required to comply with subsection 93115.6(a)(3) of the ATCM.  The emission standards of this subsection are as follows:

Diesel PM – General Requirements

93115.6(a)(3)(A)1.a		Meet 0.15 g/bhp-hr PM standard

93115.6(a)(3)(A)1.c		Operate 50 hours per year, or less, for maintenance and testing (except emergency use and emissions testing)

HC, NOx, NMHC+NOx, CO

93115.6(a)(3)(B)		Meet standards for off-road engines of the same model year and horsepower rating as specified in the Off-Road Compression Ignition Engine Standards; or if no standards have been established, meet the Tier 1 standards in Title 13, CCR, Section 2423 for off-road engines of the same horsepower rating, irrespective of the new engine’s model year.

S-1561 is an EPA certified Tier 3 engine that complies with these emission standards.

S-1562 and S-1563 Emergency Diesel Engines are new Firewater Pumps installed after January 1, 2005.  Therefore, S-1562 and S-1563 are required to comply with subsection 93115.6(a)(4) of the ATCM.  The emission standards of this subsection are as follows:

93115.6(a)(4)(A)1.a		meet the Tier 2 emission standards specified in the Off-Road Compression Ignition Engine Standards for off-road engines with the same maximum rated power (title 13 CCR, section 2423) until 3 years after the date the Tier 3 standards are applicable for off-road engines with the same maximum rated power. At that time, new direct-drive emergency standby diesel-fueled fire-pump engines (>50 bhp) are required to meet the Tier 3 emission standards, until 3 years after the date the Tier 4 standards are applicable for off-road engines with the same maximum rated power. At that time, new direct-drive emergency standby diesel-fueled fire-pump engines (>50 bhp) are required to meet the Tier 4 emission standards; and

93115.6(a)(4)(A)1.b		not operate more than the number of hours necessary to comply with the testing requirements of the National Fire Protection Association (NFPA) 25 - "Standard for the Inspection, Testing, and Maintenance of Water-Based Fire Protection Systems," 2002 edition, which is incorporated herein by reference. This subsection does not limit engine operation for emergency use and for emission testing to show compliance with 93115.6(a)(4).

S-1562 and S-1563 are EPA certified Tier 3 engines that comply with these emission standards.

PSD

A Prevention of Significant Deterioration (PSD) applicability analysis was performed for the Avon Wharf MOTEMS project. Because this project involves only “new emission units,” Tesoro used the “actual-to-potential test” specified in 40 CFR 52.21(a)(2)(iv)(d) to determine if a “significant emissions increase” and a “significant net emissions increase” of a “regulated NSR pollutant” will occur. The project involves construction of a new Berth 1A, including an emergency standby diesel IC engine, east fire water pump IC engine, west fire water pump IC engine, recovered oil tank, and marine vapor recovery system.

In the BAAQMD, the PSD pollutants are PM, PM10, PM2.5, NO2, CO, and SO2 in this analysis. For this analysis NOx is considered to be equivalent to NO2.  The table below summarizes the resulting project emission increases of each regulated PSD pollutant from the “new emission units” for which there was a calculated emissions increase, the total project emission increase, and the corresponding PSD significant emission rates.

BAAQMD Rule 2-2-215.2 states the emissions associated with the cargo carriers shall not be included when determining applicability of the requirements of Section 2-2-304 (PSD Requirement). This definition is consistent with the definition of a “stationary source” under the PSD program at 40 CFR 52.21(b)(5), which excludes “the activities of any vessel.” The recovered oil tank and marine vapor recovery system have a potential to emit only VOC emissions, and are therefore not considered further.

The following table presents the results of the PSD applicability analysis. The project emission increases are less than the corresponding PSD significant emission rates; therefore, a major modification will not occur and the project is not subject to PSD review. The Project does not trigger PSD review for a criteria pollutant; therefore, GHG is not included within this analysis consistent with EPA guidance on implementation of its PSD regulations in light of the Supreme Court’s decision in UARG v. EPA, 12-1146 (2014).

|  | PM | PM10 | PM2.5 | NO2 | CO | SO2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | tpy | tpy | tpy | tpy | tpy | tpy |
| S-1561 Emergency Standby Diesel IC Engine | 0.00219 | 0.00219 | 0.00219 | 0.0559 | 0.0219 | 0.00004 |
| S-1562 East Fire Water Pump IC Engine | 0.0038 | 0.0038 | 0.0038 | 0.155 | 0.093 | 0.0001 |
| S-1563 West Fire Water Pump IC Engine | 0.0038 | 0.0038 | 0.0038 | 0.155 | 0.093 | 0.0001 |
| Project Emission Increase | 0.0098 | 0.0098 | 0.0098 | 0.3659 | 0.2079 | 0.0002 |
| PSD Significant Emission Rate | 25 | 15 | 10 | 40 | 100 | 40 |

STATEMENT OF COMPLIANCE

S-1560 Avon Wharf Berth 1A will be subject to the same regulations as the existing S-100 Berth 1.  Tesoro is subject to, and will continue to comply with the following applicable requirements:

Regulation 6, Rule 1 (Particulate Matter General Requirements).

Regulation 9, Rule 1 (Sulfur Dioxide).

Regulation 8 Rule 44 (Organic Compounds - Marine Vessel Loading Terminals)

40 CFR 63 Subpart Y (NESHAP for Marine Vessel Loading of Organic Liquids)

40 CFR 63 Subpart CC (NESHAP for Source Categories - Petroleum Refineries)

S-1561 Emergency Standby Diesel IC Engine, S-1562 East and S-1563 West Fire Water Pump IC Engines are subject to, and will comply with, the following applicable requirements:

Regulation 6, Rule 1 (Particulate Matter General Requirements).

Regulation 9, Rule 1 (Sulfur Dioxide).

Regulation 9, Rule 8 (Nitrogen Oxides and Carbon Monoxide from Stationary Internal Combustion Engines)

40 CFR 60 Subpart IIII (NSPS for Stationary Compression Ignition Internal Combustion Engines)

40 CFR 63 Subpart ZZZZ (NESHAP for Stationary Reciprocating Ignition Internal Combustion Engines)

CARB ATCM for Stationary Compression Ignition Engines

S-1564 Recovered Oil Fixed Roof Tank is subject to, and will comply with, the following applicable requirements:

Regulation 8, Rule 5 Organic Compounds – Storage of Organic Liquids

40 CFR 61 Subpart FF (Benzene Waste Operations NESHAPS

S-1564 is exempt from NSPS Kb and MACT CC because of capacity and will be in Group 402A [MACT and NSPS Kb Exempt (size), BWON 61 Subpart FF (Uncontrolled waste streams), Submerged Fill - Top  Fill and Pressure Vacuum Vent] of Table IV-F2 and F3 in the Title V permit.

The project was subject to CEQA review.  An EIR was certified by the State Lands Commission in March 2015.  The District will publish a Notice of Determination that will include the document entitled “Findings and Supporting Facts Regarding the Environmental Impact Report” in support of this CEQA determination.

The project is over 1000 feet from the nearest school and therefore not subject to the public notification requirements of Reg. 2-1-412.

PERMIT CONDITIONS

New Permit Condition 26406 will be imposed for S-1560 Avon Wharf Beth 1A

Permit Condition 23811 will be revised to include S-1561 Diesel Emergency Generator

New Permit Condition 26407 will be imposed for S-1562 and S-1563 Diesel Firewater Pumps

New Permit Condition 26408 will be imposed for S-1564 Recovered Oil Tank

Permit Condition 878 for S-100 Avon Wharf Berth 1 will be deleted.

Permit Condition 23486 for S-1508 and S-1509 Berth 1 and 5 Recovered Oil Tanks will be deleted.

## Condition 23811

Application 14917, September 2006.

Modified by Application 16495, November 2007.

Modified by Application 19330, February 2009.

Modified by Application 21713, May 2010

Modified by Application 22152, October 2010

Modified by Application 25942, February 2014.  Added S-1557.

Modified by Application 27790, January 2017.  Added S-1561

Plant 14628 (B2758) Emergency Diesel Engines S-1518, S-1519 and S-1557

Plant 14629 (B2759) Emergency Diesel Engines S-56 and S-57

Plant 14628 (B2758) Emergency Diesel Engine S-1522

Plant 14629 (B2759) Emergency Diesel Engine S-58

Plant 14628 (B2758) Avon Wharf Berth 1A Emergency Generator Diesel Engine S-1561

1. 	Operating for reliability-related activities is limited to 50 hours per year per engine.

[Basis: "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.6(b)(3)(A)2b and 93115.6(a)(3)(A)1c]

2. 	The owner or operator shall operate each emergency standby engine only for the following purposes: to mitigate emergency conditions, for emission testing to demonstrate compliance with a District, state or Federal emission limit, or for reliability-related activities (maintenance and other testing, but excluding emission testing). Operating hours while mitigating emergency conditions or while emission testing to show compliance with District, state or Federal emission limits is not limited.

[Basis: Regulation 9-8-330, "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.4(29)]

3.	The owner/operator shall operate each emergency standby engine only when a non-resettable totalizing meter (with a minimum display capability of 9,999 hours) that measures the hours of operation for the engine is installed, operated and properly maintained.

[Basis: Regulation 9-8-530, "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.10(d)(1)]

4. 	Records: The owner/operator shall maintain the following monthly records in a District-approved log for at least 60 months from the date of entry. Log entries shall be retained on-site, either at a central location or at the engine's location, and made immediately available to the District staff upon request.

a. Hours of operation for reliability-related activities (maintenance and testing).

b. Hours of operation for emission testing to show compliance with emission limits.

c. Hours of operation (emergency).

d. For each emergency, the nature of the emergency condition.

e. Fuel usage for each engine(s).

[Basis: Regulation 9-8-530, 2-6-501, and "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.10(f)]

## Condition 26406

Application 27790, January 2017.

Avon Wharf MOTEMS Project

S-1560 Avon Wharf Berth 1A, abated by

A-1560 Avon Wharf Berth 1A Marine Vapor Recovery System

1.	The owner/operator shall not operate S-1560 Berth 1A unless the total product transferred at S-1560 does not exceed 30,000,000 barrels in any consecutive 12-month period.  The owner/operator shall not transfer any Crude Oil at S-1560 Berth 1A.  (basis: Cumulative Increase, Offsets)

2.	Emissions for Cargo Carrier (Barges and Tankers) calls to S-1560 Berth 1A shall not exceed the following fully offset limits:

NOx:	188.825 tons/year

CO:	34.425 tons/year

POC: 	10.743 tons/year

PM10: 	4.157 tons/year

SO2: 	9.372 tons/year

(basis: Cumulative Increase, Offsets)

3.	The owner/operator of S-1560 shall demonstrate compliance with the throughput limit in Part 1 by recording the monthly volumes of material loaded and unloaded at S-1560.  Monthly throughput shall be totaled on a consecutive 12-month basis.  The owner/operator shall demonstrate compliance with the emission limits in Part 2 by showing annual loading and unloading throughput, on a calendar year basis, is less than 30,000,000 barrels per year.  (basis: Cumulative Increase, Offsets)

4.	The owner/operator shall operate S-1560 only when POC emissions from product loading operations do not exceed the fully offset limit of 20.00 tons/year.  The owner/operator shall record the quantity of each material loaded onto vessels at S-1560 and perform the emission calculations required to demonstrate compliance using the following emission factors (pounds per 1000 gallons loaded) and assuming a destruction efficiency of 98.5% for controlled loading.  
	Gasoline/Components loaded onto Tanker	1.8

Gasoline/Components loaded onto Barge	3.4

Diesel loaded onto Tanker			0.005

Diesel loaded onto Barge			0.012

Residual Oil loaded onto Tanker		0.00004

Residual Oil loaded onto Barge		0.00009

(basis: Cumulative Increase, Offsets)

5.	 The owner/operator may only use a different methodology and/or different assumptions to demonstrate compliance with Part 4 when approved in advance by the District.  (basis: Cumulative Increase, Offsets)

6.	The owner/operator of S-1560 Berth 1A shall not load any regulated materials, including gasoline, gasoline blending stocks, aviation gas, or JP-4 unless the entire loading operation is abated with A-1560 Marine Vapor Recovery System.  (basis: Cumulative Increase, Offsets)

7.	The owner/operator shall install and maintain a Pressure Recorder/Controller in the vapor recovery system to provide a permanent record of pressure during the loading of vessels. These records shall be maintained for a minimum of 5 years. (basis: Cumulative Increase)

8.	Not less frequently than every six months, the owner/operator shall conduct tests to assess leakage from all relief valves that vent to atmosphere in the marine vapor recovery system.  The owner/operator shall ensure that the testing and record keeping are done in compliance with Regulation 8, Rule 18.  (basis: Cumulative Increase, Regulation 8-18)

9.	Not more than 30 days after the start-up of S-1560 Berth 1A, the owner/operator shall provide the District’s Engineering Division with a final count of fugitive components installed. The owner/operator has been permitted for an increase in the following fugitive components:

12 valves in gas/vapor service

29 valves in light liquid service

17 valves in heavy liquid service

64 flanges/connectors in gas/vapor service

54 flanges/connectors in light liquid service

5 flanges/connectors in heavy liquid service

8 pumps in light liquid service

1 PRV in gas/vapor service

5 PRVs in light liquid service

3 PRVs in heavy liquid service

The total permitted fully offset fugitive POC emissions for the Avon Wharf MOTEMS project are 813 lbs/year.  (basis: Cumulative Increase, Offsets)

10.	If there is an increase in the total fugitive component emissions, the plant’s cumulative emissions for the project shall be adjusted to reflect the difference between emissions based on predicted versus actual component counts. The owner/operator shall provide to the District all additional required offsets at an offset ratio of 1.15:1 no later than 14 days after submittal of the final POC fugitive count. If the actual component count is less than the predicted, the total will be adjusted accordingly and all emission offsets applied by the owner/operator in excess of the actual total fugitive emissions will be credited back to the owner/operator. (basis: Offsets)

11.	The Owner/Operator shall maintain a District-approved record containing all measurements of type of material and quantity of material loaded and unloaded over Avon Wharf Berth 1A.  This information shall be kept available for District inspection for a period of at least 5 years following the date on which such measurements, records or data are made or recorded. (basis: Recordkeeping)

12.	Not more than 30 days after the start-up of S-1560 Berth 1A, the owner/operator shall permanently decommission S-100 Avon Berth 1 and S-1508 Berth 1 Recovered Oil Tank, and surrender the permits for S-100, S-108 Avon Berth 5, S-1508  and S-1509 Berth 5 Recovered Oil Tank.  (basis:  Contemporaneous Emissions Reductions, Cumulative Increase, Offsets)

## Condition 26407

Application 27790, January 2017.

Avon Wharf MOTEMS Project

S-1562 Avon Berth 1A East Diesel Firewater Pump

S-1563 Avon Berth 1A West Diesel Firewater Pump

1. 	Operating for reliability-related activities is limited to 70 hours per year per engine.

[Basis: "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.6(a)(4)(A)1c]

2. 	The owner or operator shall operate each firewater pump only for the following purposes: to mitigate emergency conditions, for emission testing to demonstrate compliance with a District, state or Federal emission limit, or for reliability-related activities (maintenance and other testing, but excluding emission testing). Operating hours while mitigating emergency conditions or while emission testing to show compliance with District, state or Federal emission limits is not limited.

[Basis: Regulation 9-8-330, "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.4(29)]

3.	The owner/operator shall operate each firewater pump only when a non-resettable totalizing meter (with a minimum display capability of 9,999 hours) that measures the hours of operation for the engine is installed, operated and properly maintained.

[Basis: Regulation 9-8-530, "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.10(d)(1)]

4. 	Records: The owner/operator shall maintain the following monthly records in a District-approved log for at least 60 months from the date of entry. Log entries shall be retained on-site, either at a central location or at the engine's location, and made immediately available to the District staff upon request.

a. 	Hours of operation for reliability-related activities (maintenance and testing).

b. 	Hours of operation for emission testing to show compliance with emission limits.

c. 	Hours of operation (emergency).

d. 	For each emergency, the nature of the emergency condition.

e. 	Fuel usage for each engine(s).

[Basis: Regulation 9-8-530, 2-6-501, and "Stationary Diesel Engine ATCM", CA Code of Regulations, Title 17, Section 93115.10(f)]

## Condition 26408

Application 27790, January 2017.

Avon Wharf MOTEMS Project

S-1564 Avon Berth 1A Recovered Oil Fixed Roof Tank 938, 3800 gallons

1.	The owner/operator of S-1564 Tank 938 shall not exceed 250,000 gallons of water runoff and recovered oil in any consecutive 12-month period. (basis: Cumulative Increase, Offsets)

2.	The owner/operator may store alternate liquid(s) other than the materials specified in Part 1 and/or usages in excess of those specified in Part 1, provided that the owner/operator can demonstrate that all of the following are satisfied:

a. 	The true vapor pressure of the organic material is less than 11 psia

b.	The total POC emissions from S-1564 do not exceed 3166 lbs in any consecutive 12-month period.

c.	Toxic emissions do not exceed the following:

Toluene	56.95 lbs/yr or 0.385 lb/hr

Ethylbenzene	3.15 lb/yr

Xylene (Total)	15.73 lbs/yr or 0.106 lb/hr

Benzene	8.67 lbs/yr or 0.0586 lb/hr

Naphthalene	0.06 lbs/yr

n-Hexane	8.16 lb/yr

3.	To determine compliance with the above parts, the owner/operator shall maintain the following records and provide all of the data necessary to evaluate compliance with the above parts, including the following information:

a. 	Quantities of each type of liquid stored at this source on a monthly basis.

b. 	If a material other than those specified in Part 1 is stored, POC and toxic component contents of each material used; and mass emission calculations to demonstrate compliance with Part 2, on a monthly basis;

c. 	Monthly throughput and/or emission calculations shall be totaled for each consecutive twelve-month period.

All records shall be retained on-site for five years, from the date of entry, and made available for inspection by District staff upon request. These recordkeeping requirements shall not replace the recordkeeping requirements contained in any applicable District Regulations. (Basis: Cumulative Increase; Toxics)

RECOMMENDATION

It is recommended that an Authority to Construct be granted  to Tesoro Refining & Marketing Company LLC for the following source:

S-1560	Avon Wharf Berth 1A, abated by

A-1560	Vapor Recovery System for Avon Wharf Berth 1A

S-1561	Emergency Diesel Generator, Caterpillar Model C9, 398 Bhp

S-1562	East Avon Wharf Emergency Diesel Firewater Pump, Caterpillar Model C18, 700 Bhp

S-1563	West Avon Wharf Emergency Diesel Firewater Pump, Caterpillar Model C18, 700 Bhp

S-1564	Recovered Oil Fixed Roof Tank A-938, 3800 gallons

S-1567	East Diesel Tank, 1000 gallons, exempt per Regulation 2-1-123.3.2

S-1568	West Diesel Tank, 1000 gallons, exempt per Regulation 2-1-123.3.2

It is also recommended that exemptions be granted to Tesoro Refining & Marketing Company LLC for the following sources:

S-1567	East Diesel Tank, 1000 gallons, exempt per Regulation 2-1-123.3.2

S-1568	West Diesel Tank, 1000 gallons, exempt per Regulation 2-1-123.3.2

By:

Arthur P Valla

Senior Air Quality Engineer

1/9/2017

### Appendix A

### Tesoro Avon Wharf Marine Vessel 
Emission Calculation Documentation

This document addresses the emissions sources, methodology, and assumptions used within the Avon Wharf Calculation Spreadsheet (spreadsheet) to estimate emissions for marine vessels calling on the proposed Avon Wharf. Emissions are calculated for each marine vessel call at the existing Avon Wharf from August 2013 through July 2016.

### Emissions Sources

Marine vessels calling on the Avon Wharf consist of tanker ships and barges. Tugboats are used to escort and/or push tanker ships and barges to and from the Avon Wharf. Table 1 presents the emissions sources that are evaluated for these marine vessels in the spreadsheet.

Table 1. 
Emissions Sources on Marine Vessels Calling on Avon Wharf

| Marine Vessel | Emissions Sources on Marine Vessels Calling on Avon Wharf |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Main Engine | Auxiliary Engine | Auxiliary Boiler | Onboard Pump Engines | Fugitive Vapor Displacement from Material Loading |
| Tanker ships | Yes | Yes | No 2 | No | Yes |
| Barges | No 1 | Yes | No | Yes | Yes |
| Tugboats (escort/pushing) | Yes | Yes | No | No | No |
| Notes: 1 Only self-propelled barges may have main engines; however, barges calling on Avon Wharf are assumed to not have main engines. 2 Tankers may have auxiliary boilers; however, tankers calling on Avon Wharf are assumed to not use auxiliary boilers. |  |  |  |  |  |

Marine vessels travel to the Avon Wharf using vessel traffic lanes. Emissions are calculated for the round-trip starting from the Pilot Boarding/Sea Buoy location (approximately 11 nautical miles west of the Golden Gate Bridge) to the Avon Wharf. Portions of the trip from the Pilot Boarding/Sea Buoy location to the Avon Wharf are segmented to account for emissions from the marine vessels’ various operating modes, distances, and speeds.

Tugboats escort the ships through tug escort zones. Only tank vessels carrying 5,000 or more long tons of oil are required to have an escort tug (Title 14, CCR, Subdivision 4, Chapter 4, Subchapter 1). The San Francisco Bay Area is divided into 6 tug escort zones. Avon-bound vessels transit through zones 1, 2, 5, and 6. The vessel escort begins and ends at the western end of zone 1, also known as the COLREGS (International Regulations for Preventing Collisions at Sea) Line, approximately 2.2 nautical miles (nm) west of the Golden Gate Bridge. This is the same location as Point Bonita. A vessel must be escorted by tugboats in zones 1, 2, and 6. No escort is required in zone 5, which lies between zones 2 and 6. However, because the tugboats are already escorting the tank vessel, they typically continue to escort the vessel through zone 5. Table 2 presents the vessel travel segments, distances, and tug escort zones.

Table 2. 
Vessel Travel Segments, Distances, and Tug Escort Zones

| Inbound / Outbound | Segment |  | Distance, nm | Tug Escort Zone |
| --- | --- | --- | --- | --- |
|  | From | To |  |  |
| Inbound | Pilot Boarding (Sea Buoy) | COLREGS Line | 8.8 | N/A |
|  | COLREGS Line | Golden Gate Bridge | 2.3 | Zone 1 |
|  | Golden Gate Bridge | Echo Buoy | 16.45 | Zone 2 |
|  | Echo Buoy | SPB Light #15 | 7.7 | Zone 5 |
|  | SPB Light #15 | Near Plains Terminal | 7.9 | Zone 6 |
|  | Near Plains Terminal | Avon Wharf | 0.5 | Zone 6 |
| Outbound | Avon Wharf | Near Plains Terminal | 0.5 | Zone 6 |
|  | Near Plains Terminal | SPB Light #15 | 7.9 | Zone 6 |
|  | SPB Light #15 | COLREGS Line | 26.45 | Zones 1, 2, and 5 |
|  | COLREGS Line | Pilot Boarding (Sea Buoy) | 8.8 | N/A |

#### Tanker Ships

Tanker ships are self-propelled marine vessels that are equipped with a propulsion (main) engine, two to four auxiliary engines, and an auxiliary boiler. The onboard cargo discharge pumps are either steam driven (use steam generated by auxiliary boiler) or electric-driven (use electricity generated by auxiliary engines). Auxiliary engines are typically medium speed, compression ignition engines. Table 3 presents the tanker ship operating mode, speed, duration, and number of tugboats required per segment. Note that ships not carrying product do not require an escort tug. Ships that are carrying product require 1 escort tug, except during maneuvering. Therefore, in Table 3, the number of tugboats required depend on whether the ship is empty or carrying product.

Table 3. 
Tanker Ship Operating Mode, Speed, Duration, and Number of Tugboats Required per Segment

| Inbound / Docked / Outbound | Segment |  | Tanker Ships |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | From | To | Operating Mode | Speed, knots | Duration, hours | Number of Tugboats Required |
| Inbound | Pilot Boarding (Sea Buoy) | COLREGS Line | Transit | 12 | 0.73 | 0 |
|  | COLREGS Line | Golden Gate Bridge | Transit (RSZ) | 10 | 0.23 | 0 or 1 |
|  | Golden Gate Bridge | Echo Buoy | Transit (RSZ) | 10 | 1.65 | 0 or 1 |
|  | Echo Buoy | SPB Light #15 | Transit (RSZ) | 10 | 0.77 | 0 or 1 |
|  | SPB Light #15 | Near Plains Terminal | Transit (RSZ) | 8 | 0.99 | 0 or 1 |
|  | Near Plains Terminal | Avon Wharf | Maneuvering | 5 | 0.10 | 2 |
|  | At Avon Wharf (Docking) |  | Maneuvering | N/A | 0.25 | 2 |
| Docked | At Avon Wharf (Hoteling and Product Discharge) |  | Hoteling | N/A | Varies | N/A |
| Outbound | At Avon Wharf (Undocking) |  | Maneuvering | N/A | 0.25 | 2 |
|  | Avon Wharf | Near Plains Terminal | Maneuvering | 8 | 0.06 | 2 |
|  | Near Plains Terminal | SPB Light #15 | Transit (RSZ) | 10 (loaded); 12 (unloaded) | 0.79 (loaded); 0.66 (unloaded) | 0 or 1 |
|  | SPB Light #15 | COLREGS Line | Transit (RSZ) | 10 (loaded); 12 (unloaded) | 2.65 (loaded); 2.20 (unloaded) | 0 or 1 |
|  | COLREGS Line | Pilot Boarding (Sea Buoy) | Transit | 12 | 0.73 | 0 |
| Note: RSZ = Reduced Speed Zone |  |  |  |  |  |  |

#### Barges

Barges are typically non-self-propelled and therefore aren’t equipped with main engines. Non-self-propelled barges are propelled by tugboats, which are also known as pushing boats. These pushing tugboats accompany the barge throughout the span of the trip. The barges are typically equipped with two small auxiliary engines to generate electricity and two auxiliary engines to run the onboard cargo discharge pumps. Articulated Tug/Barges (ATBs) are barges that are mechanically connected with a dedicated tugboat. The dedicated tugboat is classified as an ocean-going vessel and is equipped with a propulsion engines and auxiliary engines. ATBs do not have auxiliary boilers. For this analysis, it is assumed all barges are non-self-propelled, non-ocean-going barges. Table 4 presents the barge operating mode, speed, duration, and number of tugboats required per segment. As with tankers, barges not carrying product do not require an escort tug. Barges that are carrying product require 1 escort tug, except during maneuvering. Therefore, in Table 4, the number of tugboats required depend on whether the ship is empty or carrying product.

Table 4. 
Barge Operating Mode, Speed, Duration, and Number of Tugboats Required per Segment

| Inbound / Docked / Outbound | Segment |  | Barges |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | From | To | Operating Mode | Speed, knots | Duration, hours | Number of Tugboats Required |
| Inbound | Pilot Boarding (Sea Buoy) | COLREGS Line | Transit | 8 | 1.1 | 1 |
|  | COLREGS Line | Golden Gate Bridge | Transit | 8 | 0.29 | 1 or 2 |
|  | Golden Gate Bridge | Echo Buoy | Transit | 8 | 2.06 | 1 or 2 |
|  | Echo Buoy | SPB Light #15 | Transit | 8 | 0.96 | 1 or 2 |
|  | SPB Light #15 | Near Plains Terminal | Transit | 8 | 0.99 | 1 or 2 |
|  | Near Plains Terminal | Avon Wharf | Maneuvering | 5 | 0.10 | 2 |
|  | At Avon Wharf (Docking) |  | Maneuvering | N/A | 0.25 | 2 |
| Docked | At Avon Wharf (Hoteling and Product Discharge) |  | Hoteling | N/A | Varies | 0 |
| Outbound | At Avon Wharf (Undocking) |  | Maneuvering | N/A | 0.25 | 2 |
|  | Avon Wharf | Near Plains Terminal | Maneuvering | 5 | 0.10 | 2 |
|  | Near Plains Terminal | SPB Light #15 | Transit | 8 | 0.99 | 1 or 2 |
|  | SPB Light #15 | COLREGS Line | Transit | 8 | 3.31 | 1 or 2 |
|  | COLREGS Line | Pilot Boarding (Sea Buoy) | Transit | 8 | 1.1 | 1 |

#### Tugboats

Tugboats are used to escort ships, ATBs, and barges while transiting through the escort zones (only tank vessels carrying 5,000 or more long tons of oil are required to have an escort tug) and assist these vessels while docking and undocking at the port. Non-self-propelled barges require an escort tugboat in addition to the pushing tugboat if the barge is loaded with 5,000 tons of oil as cargo. An escort tugboat is classified as harbor craft, as it operates near the harbor. The number and class of tugboats required to escort a vessel depends on the current velocity, displacement of ships, deadweight tonnage (DWT) of barges, and the bollard pull rating of the available tugboats. However, for this analysis, an escort or pushing tugboat is assumed to typically have two propulsion (main) engines and two auxiliary engines. One of the auxiliary engines is a backup; although two auxiliary engines are present, only one auxiliary engine is used at a time.

Emissions from escort tugboats are estimated for round-trip travel described below:

- Cruise travel from tugboat’s home base to COLREGS Line;

- Escort/assist travel from COLREGS Line to Avon Wharf;

- Cruise travel to and from temporary port (1.7 nautical miles each way);

- Escort/assist travel from Avon Wharf to COLREGS Line;

- Cruise travel from tugboat’s home base.

Emissions from pushing tugboats for barges are estimated for round-trip travel described below:

- Pushing assist travel from the Sea Buoy to Avon Wharf;

- Auxiliary engine use during hoteling;

- Pushing assist travel from Avon Wharf to the Sea Buoy.

### Methodology and Assumptions

The methodology and assumptions used to estimate emissions for the emissions sources associated with tankers, barges, and tugboats are described below.

#### Emissions from Tankers

#### Emissions from Tanker Main Engine

The following equation is used to estimate emissions of a specific pollutant p, in grams, from tanker main (propulsion) engines:

Ep =EFp, T, S,MY, F,om×MCR ×LFom ×Timeom × LLAp × 1-effp, MY  ×SF

Table 5 summarizes the parameters, inputs, default assumptions used, and references for estimating emissions from tanker main engines.

| Table 5. Tanker Main Engine Emissions Calculations Parameters, Data Inputs, Assumptions, and Applicable References |  |  |
| --- | --- | --- |
| Parameter | Description | Data Inputs, Assumptions, and Applicable References |
| T | Engine type | Applicable engine types are motor diesel, diesel-electric, steam turbine, gas turbine, and gas turbine-electric. If data is not available, motor diesel engine type is assumed. |
| EFp, T, S, MY, F | Emission factor, g/kW-hr | Reference: All emission factors for motor engines obtained from California ARB, May 2011, Appendix D, Emissions Estimation Methodology for Ocean-Going Vessels, Tables II-6 through Tables II-8. Criteria pollutant emission factors for steam turbine and gas turbine propulsion engines obtained from Table 2-9, Current Methodologies in Preparing Mobile Source Port-Related Emission Inventories, US EPA, April 2009. Emission factors for Gas Turbine - Electric assumed to be equal to those for Gas Turbine. Currently all ship motor engines are assumed to be motor diesel. |
| p | Pollutant | NOX, CO, VOC, PM10, PM2.5, SO2 |
| S | Engine speed, rpm | If data is not available, slow speed engine (129 rpm) is assumed. Main diesel engines are typically two-stroke, slow-speed engines with speed less than 130 rpm. |
| MY | Engine model year | If vessel-specific engine data is not available, model year is assumed to be equal to ship built year. |
| MCR | Maximum continuous rating (max power) of the engine, kW | If tanker-specific kW is not known, maximum continuous rating is based on ship’s deadweight tonnage (DWT), based on equation below: MCR, kW=0.746×9070+0.101×DWT Reference: USEPA, Analysis of Commercial Marine Vessels Emissions and Fuel Consumption Data, (EPA420-R-00-002, February 2000), Table 4-5. |
| LLAp | Low-load adjustment multiplier | LLA is only applied if main engine operates below 20% load factor. Reference: USEPA, Current Methodologies in Preparing Mobile Source Port-Related Emission Inventories, April 2009, Table 2-15 |
| LFom | Load factor of the engine by operating mode, % | LFom= ASomMS3 For docking and undocking operating mode, load factor assumed to be 2%. On a vessel with a steam engine for a main engine with no auxiliary boilers, a minimum load factor of 2% or an equivalent load factor for the same power consumption as required for an auxiliary boiler on a motor diesel main engine (3,000 kW / MCR) or diesel-electric (346 kW / MCR) was assumed. Reference: SF Bay Area Seaports Air Emissions Inventory, Port of Richmond 2005 Emissions Inventory, June 2010 |
| effp, MY | Control efficiency | Control efficiencies for NOX (30%) and PM (25%) assumed for main motor diesel or diesel-electric engines built after 2004. This control efficiency was removed from the emission calculations since subsequent Port of Los Angeles Inventory documents changed and did not apply these control efficiencies to all tanker engines. Reference: Port of Los Angeles Inventory of Air Emissions 2012, July 2013, Section 3.5.12. |
| ASom | Actual vessel speed in operating mode, knots | Assumed speeds are as presented in Table 3. Reference: SF Bay Area Seaports Air Emissions Inventory, Port of Richmond 2005 Emissions Inventory, June 2010; Title 14, CCR, Subdivision 4. Office of Spill Prevention and Response, Chapter 4, Subchapter 1, Sections 851.1 through 851.10.1 |
| MS | Maximum vessel speed, knots | If data is not available, 15 knots is assumed. Reference: CARB, Emissions Estimation Methodology for Ocean Going Vessels, May 2011 |
| Timeom | Duration in operating mode, hours | Timeom= DistanceomASom Distanceom is the distance travelled by the vessel by operating mode in nautical miles. Reference: Pilot Guidebook provided by Tesoro, Distances from the Sea Buoy, pg 76 Docking and undocking activities at Wharf take 15 minutes each. Hoteling time is equal to the vessel departure time subtracted by the vessel arrival time. Reference: SF Bay Area Seaports Air Emissions Inventory, Port of Richmond 2005 Emissions Inventory, June 2010 |
| F | Fuel type | Applicable fuel types for tanker ship main engines include 2.7% sulfur heavy fuel oil [HFO], 0.5% sulfur marine diesel oil [MDO]/marine gas oil [MGO], and 0.1% sulfur MDO/MGO. MDO is assumed to be used in the engines. Since July 1, 2009, sulfur content for ocean-going vessels was reduced to 0.5% sulfur for MDO. Since January 1, 2014, sulfur content for ocean-going vessels has been reduced to 0.1% sulfur for MDO. No heavy fuel oil is used. Emissions factors are converted from 2.7% sulfur content heavy fuel oil to 0.5% and 0.1% sulfur content marine diesel oil using the fuel correction factors in the following equation: EFp, T, S,MY, x% MDO= EFp, T, S,MY, 2.7% HFO × FCFx % MDO |
| om | Operating mode | Tanker ships have cruise, maneuvering, and hoteling operating modes. Main engines are assumed to be operated in cruise, slow cruise in RSZ, and maneuvering operating modes. |
| SF | Shared emissions factor | Ocean going vessels may service multiple refineries. If an ocean going vessel services multiple refineries, the emissions attributed to transit is apportioned evenly to all the refineries that are serviced by the ocean going vessel. The emissions attributed to at-berth activities (maneuvering, hoteling, etc.) during the ship call are not considered shared. Data from the Marine Exchange of the San Francisco Bay Region (SFMX) was used for information on ship calls. |

#### Emissions from Tanker Auxiliary Engines

The following equation is used to estimate emissions of a specific pollutant p, in grams, from tanker auxiliary engines:

Ep =EFp, T, S,MY, F×MCR ×LFom ×Timeom ×SF

Table 6 summarizes the parameters, inputs, default assumptions used, and references for estimating emissions from tanker auxiliary engines.

Table 6. 
Tanker Auxiliary Engine Emissions Calculations Parameters, Data Inputs, Assumptions, and Applicable References

| Parameter | Description | Data Inputs, Assumptions, and Applicable References |
| --- | --- | --- |
| T | Engine type | If data is not available, motor diesel engine type is assumed. |
| EFp, T, S, MY, F | Emission factor, g/kW-hr | Reference: All emission factors for motor engines obtained from California ARB, May 2011, Appendix D, Emissions Estimation Methodology for Ocean-Going Vessels, Tables II-8. |
| p | Pollutant | NOX, CO, VOC, PM10, PM2.5, SO2 |
| S | Engine speed, rpm | Engine speed of 900 rpm assumed. |
| MY | Engine model year | If vessel-specific engine data is not available, model year is assumed to be equal to ship built year. |
| MCR | Maximum continuous rating (max power) of the engine, kW | If tanker-specific kW is not known, maximum continuous rating based on ship’s maximum continuous rating of the main engine, based on equation below: MCR, kW=0.211×MCR of Main Engine Reference: USEPA, Current Methodologies in Preparing Mobile Source Port-Related Emission Inventories, April 2009, Table 2-4. |
| LFom | Load factor of the engine by operating mode, % | Load factors are based on the values below: Operating Mode; Tanker Ship; ATB. Cruise; 24%; 17%. Reduced Speed Zone (RSZ) Transit; 28%; 27%. Maneuvering; 33%; 45%. Hoteling; 26%; 22%. Reference: Current Methodologies in Preparing Mobile Source Port-Related Emission Inventories, US EPA, April 2009, Table 2-7 |
| Timeom | Duration in operating mode, hours | Auxiliary engines are assumed to be running for the entire hoteling duration. Hoteling time is equal to the vessel departure time subtracted by the vessel arrival time. Reference: SF Bay Area Seaports Air Emissions Inventory, Port of Richmond 2005 Emissions Inventory, June 2010 |
| F | Fuel type | Applicable fuel types for tanker ship auxiliary engines include 2.7% sulfur heavy fuel oil [HFO], 0.5% sulfur marine diesel oil [MDO]/marine gas oil [MGO], 0.1% sulfur MDO/MGO. Since January 1, 2014, sulfur content for ocean-going vessels has been reduced to 0.1% sulfur for MDO/MGO. No heavy fuel oil is used. Emissions factors are converted from 2.7% sulfur content heavy fuel oil to 0.5% and 0.1% sulfur content marine diesel oil using the fuel correction factors in the following equation: EFp, T, S,MY, x% MDO= EFp, T, S,MY, 2.7% HFO × FCFx % MDO |
| om | Operating mode | Tanker ships have cruise, slow cruise in reduced speed zone [RSZ], maneuvering, and hoteling operating modes. Auxiliary engines are assumed to be operated in all operating modes. |
| SF | Shared emissions factor | Ocean going vessels may service multiple refineries. If an ocean going vessel services multiple refineries, the emissions attributed to transit is apportioned evenly to all the refineries that are serviced by the ocean going vessel. The emissions attributed to at-berth activities (maneuvering, hoteling, etc.) during the ship call are not considered shared. Data from the Marine Exchange of the San Francisco Bay Region (SFMX) was used for information on ship calls. |

#### Emissions from Tanker Auxiliary Boilers

According to Tesoro, tankers visiting the Avon Berth have onboard pumps that are typically driven by hydraulic or electric power generated by auxiliary engines. Therefore, emissions from tanker auxiliary boilers would not occur. Emissions from the auxiliary engines that drive the onboard pumps have been accounted for.

#### Emissions from Barges

Emissions from barges are calculated for auxiliary engines used to generate electricity and auxiliary engines used to operate cargo discharge pumps. The following equation is used to estimate emissions of a specific pollutant p, in grams, from barge engines:

Ep =EF0 MY, HP, U,om×FMY, HP × 1+ DHP×AULU ×HPU×n×LFU, om ×Timeom×SF

Table 7 summarizes the parameters, inputs, default assumptions used, and references for estimating emissions from barges.

| Table 7. Barge Emissions Calculations Parameters, Data Inputs, Assumptions, and Applicable References |  |  |
| --- | --- | --- |
| Parameter | Description | Data Inputs, Assumptions, and Applicable References |
| T | Engine type | If data is not available, motor diesel engine type is assumed. |
| EF0, MY, HP, U | Zero- hour emission factor based on engine model year, horsepower and engine use, g/HP-hr | Reference: Criteria air pollutant emission factors from CARB, California Barge and Dredge Emissions Inventory Database, http://www.arb.ca.gov/msei/categories.htm. Zero-hour emission factor for PM2.5 assumed to be equal to the zero-hour emission factor for PM10. Zero-hour emission factor for SO2 was estimated using the 15 ppm fuel sulfur content in ultra-low sulfur diesel and the following equation: EF0, SO2= 15106 × MWSO2MWS ×BSFC Where MWSO2 is the molecular weight of SO2 (64 g/mole), MWS is the molecular weight of sulfur (32 g/mole), and BSFC is the brake-specific fuel consumption in g/HP-hr. |
| p | Pollutant | NOX, CO, VOC, PM10, PM2.5, SO2 |
| MY | Engine model year | Where engine-specific information is not known, the engine model year is assumed to be the latest of either the barge build year, or the earliest year for which Tier 3 was required for an engine of that horsepower. (The shipping operators replace Tier 1 or 2 engines with Tier 3 in lieu of rebuild for compliance with ARB fleet rules.) |
| LFU, om | Load factor of the engine by engine use and operating mode, % | Load factors are based on the values below: Engine; Transit; Hoteling; Discharge. Main (propulsion); 45%; 0%; 0%. Auxiliary engine (generator); 75%; 75%; 75%. Auxiliary engine (pumps); 0%; 71%; 71%. Reference: CARB, Appendix C Updates on the Emissions Inventory for Commercial Harbor Craft Operating in California, Table 3. As noted above, it is assumed that none of the barges are self-propelled, therefore no emissions are calculated from main propulsion engines. |
| Timeom | Duration in operating mode, hours | Transit time is equal to distance traveled in nautical miles divided by speed in knots. For transit mode, barges are assumed to travel at 8 knots. For maneuvering mode, barges are assumed to travel at 5 knots. For docking and undocking, it is assumed to take 15 minutes for each (30 minutes total). Hoteling time is equal to the vessel departure time subtracted by the vessel arrival time. Discharge time is equal to the quantity of cargo discharged divided by the discharge rate of the pumps, provided in the barge database. Reference: Title 14, CCR, Subdivision 4. Office of Spill Prevention and Response, Chapter 4, Subchapter 1, Sections 851.1 through 851.10.1 (for transit speed) |
| FMY, HP | Fuel correction factor based on engine model year and horsepower | Reference: CARB, Proposed OFFROAD Diesel Fuel Correction Factors, Table 7: http://www.arb.ca.gov/msei/offroad/techmemo/arb_offroad_fuels.pdf Fuel correction factor for PM2.5 assumed to be equal to fuel correction factor for PM10. For all other pollutants, fuel correction factor assumed to be equal to 1. |
| DHP | Deterioration factor based on horsepower | Reference: CARB, California Barge and Dredge Emissions Inventory Database. http://www.arb.ca.gov/msei/categories.htm Deterioration factor for PM2.5 assumed to be equal to deterioration factor for PM10. For SO2, deterioration factor assumed to be equal to 0. |
| A | Age of the engine | Age of the engine equals year of call subtracted by engine model year. |
| ULU | Useful life based on the engine use | For main (propulsion) engines, useful life is assumed to be equal to 17 years. For auxiliary engines (generators), useful life is assumed to be equal to 22.5 years. For auxiliary engines (pumps), useful life is assumed to be equal to 21 years. Reference: CARB, Appendix C, Updates on the Emissions Inventory for Commercial Harbor Craft Operating in California, Table 3. |
| HPU | Total horsepower rating of the engines by use | Total horsepower rating is equal to the power rating per engine multiplied by the number of engines. For unknown engine horsepower ratings, the average horsepower of barge engines by engine use was used, based on available barge data. |
| n | Number of engines by use on the barge | All barges are assumed to be non-self-propelled. Therefore, the number of main (propulsion) engines is assumed to be 0. The number of auxiliary engines (generators) is assumed to be 2, and the number of auxiliary engines (pumps) is assumed to be 2, if vessel-specific information is not available. In general, only one auxiliary generator engine is needed at a time. For discharging, it is assumed all pumps on vessel are used, and one auxiliary generator engine, unless the barge operates an inert gas system during discharge of gasoline, which necessitates the use of a second auxiliary generator engine. For loading, no barge pumps are used to assist the onshore pump(s) and only one generator (auxiliary engine) is used. |
| U | Engine use | Main or auxiliary engine. |
| om | Operating mode | Barges have cruise and escort/assist operating modes. |
| SF | Shared emissions factor | Ocean going vessels may service multiple refineries. If an ocean going vessel services multiple refineries, the emissions attributed to transit is apportioned evenly to all the refineries that are serviced by the ocean going vessel. The emissions attributed to at-berth activities (maneuvering, hoteling, etc.) during the ship call are not considered shared. Data from the Marine Exchange of the San Francisco Bay Region (SFMX) was used for information on ship calls. |

#### Emissions from Tugboats

The following equation is used to estimate emissions of a specific pollutant p, in grams, from tugboat engines:

Ep =EF0 MY, HP, U×FMY, HP × 1+ DHP×AULU ×HPU×n×LFU, om ×Timeom×SF×N

Table 8 summarizes the parameters, inputs, default assumptions used, and references for estimating emissions from escort tugboats.

| Table 8. Tugboat Emissions Calculations Parameters, Data Inputs, Assumptions, and Applicable References |  |  |
| --- | --- | --- |
| Parameter | Description | Data Inputs, Assumptions, and Applicable References |
| EF0, MY, HP, U | Zero-hour emission factor based on engine model year, horsepower and engine use, g/HP-hr | Reference: Criteria air pollutant emission factors from CARB, Appendix B Emissions Estimation Methodology for Commercial Harbor Craft Operating in California, 2012. Zero-hour emission factor for PM2.5 assumed to be equal to the zero-hour emission factor for PM10. Zero-hour emission factor for SO2 was estimated using the 15 ppm fuel sulfur content in ultra-low sulfur diesel and the following equation: EF0, SO2= 15106 × MWSO2MWS ×BSFC Where MWSO2 is the molecular weight of SO2 (64 g/mole), MWS is the molecular weight of sulfur (32 g/mole), and BSFC is the brake-specific fuel consumption in g/HP-hr. |
| p | Pollutant | NOX, CO, VOC, PM10, PM2.5, SO2 |
| MY | Engine model year | Tugboat built year assumed to be equal to the average built year of all tugboats operating in the Bay Area, by tug class (Class A or B). Engine model year assumed to be the same as the built year, unless specific information on engine replacements is known. |
| LFU, om | Load factor of the engine by engine use and operating mode, % | Load factors are based on the values below: Operating Mode; Main; Auxiliary. Escort cruise; 50%; 43%. Escort assist; 31%; 43%. Escort hotel; 0%; 0%. Pushing cruise; 31%; 17%. Pushing assist (Reduced Speed Zone); 31%; 27%. Pushing assist (maneuvering); 31%; 45%. Pushing hotel; 0%; 22%. Reference: SF Bay Area Seaports Air Emissions Inventory, Port of Richmond 2005 Emissions Inventory, June 2010, Appendix A, Table 2; Load factor for pushing tugboat auxiliary engine from Current Methodologies in Preparing Mobile Source Port-Related Emission Inventories, US EPA, April 2009, Table 2-7. |
| Timeom | Duration in operating mode, hours | Distances and speeds are outlined in Tables 3 and 4, except for light trips (trips when a tug is not escorting/ assisting a vessel) outlined below. Distance from tugboat home base to COLREGS Line is average of distances between the three major tugboat companies operating in Bay Area (obtained from Google Earth). Cruise time is equal to the duration it takes for the tugboat to arrive at the COLREGS Line from its home base (round-trip), and also takes into account travel to temporary base (1.7 nautical miles each way) while escort vessel is hoteling. Tugs used for maneuvering assist but not for escorting are assumed to originate at and/or return to the temporary base. Assist/escort time is the time taken by the vessel to travel to and from the COLREGS Line and docking/undocking at the Berth. Average tugboat speed assumed to be 13 knots when not escorting or pushing. |
| FMY, HP | Fuel correction factor based on engine model year and horsepower | Reference: CARB, Appendix B, Emissions Estimation Methodology for Commercial Harbor Craft Operating in California, 2012, Table II-4; Port of Los Angeles Inventory of Air Emissions 2012, July 2013, Table 4.5. Fuel correction factor for PM2.5 assumed to be equal to fuel correction factor for PM10. For all other pollutants, fuel correction factor assumed to be equal to 1 (SO2 emission factor already calculated for 15ppm sulfur content). |
| DHP | Deterioration factor based on horsepower | Reference: CARB, Appendix B, Emissions Estimation Methodology for Commercial Harbor Craft Operating in California, 2012, Table II-5 Deterioration factor for PM2.5 assumed to be equal to deterioration factor for PM10. For SO2, deterioration factor assumed to be equal to 0. |
| A | Age of the engine | Age of the engine equals year of call subtracted by engine model year. |
| ULU | Useful life based on the engine use | For tugboat main (propulsion) engines, useful life is assumed to be equal to 21 years. For auxiliary engines, useful life is assumed to be equal to 23 years. Reference: CARB, Appendix B Emissions Estimation Methodology for Commercial Harbor Craft Operating in California, 2012, Table II-2. |
| HPU | Horsepower rating of the engine by use | Average of all tugboats operating in Bay Area by tug class (Class A or B). Tugboats escorting barges are assumed to be Class A. Pushing tugboats assisting barges are assumed to be Class A. Tugboats escorting tanker ships are assumed to be Class A. |
| n | Number of engines by use | Each tugboat is assumed to have 2 main engines and 2 auxiliary engines. It is assumed that one of the auxiliary engines is a backup engine, and that only one auxiliary engine is operated at any time. |
| U | Engine use | Main or auxiliary engine. |
| om | Operating mode | Tugboats have cruise, escort/assist, and pushing/assist operating modes. |
| N | Number of tugboats required to escort and dock/undock the vessel | Number of tugboats required per ship varies by currents, ship’s displacement, and the Kips rating of the available tugboats. For barges, one pushing tugboat is assumed to be required. One escort tugboat is required based on whether the vessel contains cargo or not. During maneuvering, it is assumed that two tugboats must be present (two escort tugboats for tankers; one pushing tugboat and one escort tugboat for barges). Reference: Title 14, CCR, Subdivision 4. Office of Spill Prevention and Response, Chapter 4, Subchapter 1, Sections 851.1 through 851.10.1. For docking and undocking: San Francisco Harbor Safety Plan Addendum 3 - Tesoro Terminal Regulations for Ships and Barges - Tug Requirement. |
| SF | Shared emissions factor | Ocean going vessels may service multiple refineries. If an ocean going vessel services multiple refineries, the emissions attributed to transit is apportioned evenly to all the refineries that are serviced by the ocean going vessel. The emissions attributed to at-berth activities (maneuvering, hoteling, etc.) during the ship call are not considered shared. Data from the Marine Exchange of the San Francisco Bay Region (SFMX) was used for information on ship calls. |

#### Emissions from Loading

Fugitive VOC emissions due to vapor displacement during cargo loading into the marine vessel cargo holds were estimated using the emissions factors provided in AP-42, Chapter 5.2, Transportation and Marketing of Petroleum Liquids, June 2008. Table 5.2-2 of this AP-42 document provides uncontrolled VOC emission factors for gasoline loading into marine vessels for various loading scenarios. These emission factors are summarized in Table 9 below. Control efficiency applied only to loading gasoline or other regulated materials onto vessels. Capture and control efficiency of 98.5% was assumed for ships controlled with the A-1560 vapor recovery system.

Table 9. 
Uncontrolled Emission Factors for Gasoline Loading

| Vessel Tank Condition | Previous Cargo | VOC Emissions, lb/1,000 gallons loaded |  |
| --- | --- | --- | --- |
|  |  | Ships / Ocean Barges | Barges |
| Typical overall situation | Any cargo | 1.8 | 3.4 |
| Source: EPA AP-42, Chapter 5.2, Transportation and Marketing of Petroleum Liquids, June 2008, Table 5.2-2. |  |  |  |

While the previous cargo for each call is known, the vessel tank condition is unknown. Emissions during cargo loading assume an unknown/overall typical vessel tank condition for both ships/ocean barges and barges.

Table 5.2-6 of this AP-42 document provides uncontrolled Total Organic Compound (TOC) emission factors for loading other petroleum products into marine vessels for various loading scenarios. It is assumed VOC is equal to TOC. These emission factors are summarized in Table 10 below.

Table 10. 
Uncontrolled Emission Factors for Petroleum Liquid Loading

| Petroleum Liquid | VOC Emissions, lb/1,000 gallons loaded |  |
| --- | --- | --- |
|  | Ships / Ocean Barges | Barges |
| Diesel | 0.005 | 0.012 |
| Jet naptha (JP-4) | 0.5 | 1.2 |
| Jet kerosene | 0.005 | 0.013 |
| Residual oil | 0.00004 | 0.00009 |

### Future Projected Emissions

Per the BAAQMD’s direction, a baseline period from August 2013 through July 2016 was used to calculate an average emission factor in pounds per 1000 barrels. This best represents Tesoro’s current fleet and operations. No change in our operations are anticipated for the future. Therefore, the emission factor represents actual volumes loaded or unloaded from ships, the typical fleet use of barges versus tankers, and the typical pattern with respect to sharing emissions due to multiple stops in the Bay. Therefore, this factor is used to calculate projected emissions.  This approach is more conservative than assuming that ships will be larger and more completely loaded in the future.

The baseline emissions calculated are contemporaneous emission reductions due to the shutdown and replacement of the existing berth. Therefore, the future emissions projections are adjusted for the contemporaneous emission reduction. The net increase will be offset at the appropriate ratio.