# II	EQUIPMENT

#### Table II A1A – Permitted Sources (New Source Review)

Plant #B2758 - Tesoro Refining and Marketing Company - Martinez Refinery

Each of the following sources has been issued a permit to operate pursuant to the requirements of BAAQMD Regulation 2, Permits. The capacities in this table are the maximum allowable capacities pursuant to 2-1-301. Throughput limits function as reporting thresholds as described in Standard Conditions J.1.

| S-# | Description | Make or Type | Model | Capacity | Grandfathered Limit, or Firm Limit and Basis |
| --- | --- | --- | --- | --- | --- |
| 709 | Tank 113-A-709, Green Crude Oil, Waste Oil | External floating roof |  | 4,746K gal 4934.1K bbl/yr 5,302 lbs total organic emissions per year | 1978 Application 26763 New Source Review |

# IV	SOURCE-SPECIFIC APPLICABLE REQUIREMENTS

## Section F Tanks

### Section F.1: Tanks – Source Listing and Applicable Permit Conditions

#### Table IV – F.1

#### Source-specific Applicable Requirements

Tanks – Source Listing and Applicable Permit Conditions

| S-# | Description | Group | BAAQMD Cond # | Condition Description | FE |
| --- | --- | --- | --- | --- | --- |
| 709 | Tank 113-A-709, Green Crude Oil, Waste Oil | 203B | None |  |  |

### Section F.2: Tanks – Groups And Group Descriptions

#### Table IV – F.2

#### Source-specific Applicable Requirements

Tanks – Groups and Group Descriptions

| Tank Group | Tank Type | Group Description | Sources |
| --- | --- | --- | --- |
| 203 | 8-5-304 EFR | MACT EEEE | This group includes sources from 203A, 203B, 203C, and 203D |
| 203B | 8-5-304 EFR | NSPS Ka, MACT EEEE EFR | S705, S706, S707, S708, S709 |

### Section F.3: Tanks – Tank Group Applicable Requirements

#### Table IV – F.3

#### Source-specific Applicable Requirements

Tank Group Applicable Requirements

| Regulation | Description | FE Y/N | 101 AB | 102 ABCD | 201 | 202 ABCD | 203 ABCD | 204 ABC | 301 | 302 | 303 AB | 401 | 402 | 403 ABCD | 404 ABC | 405 (Reserved) | 501 | 502 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8-5-100 | General | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-101 | Description | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111 | Limited Exemption, Tank Removal From and Return to Service | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.1 | Limited Exemption, Tank Removal From and Return to Service, Notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.1.1 | Limited Exemption, Tank Removal From and Return to Service, Notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.1.2 | Limited Exemption, Tank Removal From and Return to Service, Notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.2 | Limited Exemption, Tank Removal From and Return to Service; Tank in compliance at time of notification | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.3 | Limited Exemption, Tank Removal From and Return to Service; Filling, emptying, refilling floating roof tanks | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-111.5 | Limited Exemption, Tank Removal From and Return to Service; Minimize emissions and, if required, degas per 8-5-328 | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.6 | Limited Exemption, Tank Removal From and Return to Service; Self report if out of compliance during exemption period | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.1 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.1.1 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.1.2 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.2 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Tank in compliance at time of notification | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.3 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; No product movement, Minimize emissions | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.4 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Not to exceed 7 days | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.5 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Self report if out of compliance during exemption period | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.6 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Keep records for each exemption | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.6.1 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Keep records for each exemption | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.6.2 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Keep records for each exemption | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.6.3 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Keep records for each exemption | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.6.4 | Limited Exemption, Preventative Maintenance and Inspection of Tanks in Operation; Keep records for each exemption | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-117 | Limited Exemption, Low Vapor Pressure | N |  | X | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-119 | Limited Exemption, Repair Period - Optional | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-119.1 | Limited Exemption, Repair Period - Optional | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-119.2 | Limited Exemption, Repair Period - Optional | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-119.3 | Limited Exemption, Repair Period - Optional | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-301 | Storage Tank Control Requirements | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-304 | Requirements for External Floating Roof Tanks | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.1 | Requirements for External Floating Roofs; Tank fittings | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.2 | Requirements for External Floating Roofs; Primary seal (8-5-321) | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.3 | Requirements for External Floating Roofs; Secondary seal (8-5-322) | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.4 | Requirements for External Floating Roofs; Floating roof | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.5 | Requirements for External Floating Roofs; Tank shell | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.6 | Requirements for External Floating Roofs; Pontoons – no leaks | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.6.1 | Requirements for External Floating Roofs; Pontoons – make gas tight if leaking | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.6.2 | Requirements for External Floating Roofs; Pontoons-repair all leaks at next removal from service | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-320 | Floating Roof Tank Fitting Requirements | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.2 | Floating Roof Tank Fitting Requirements; Projection below liquid surface | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.3 | Floating Roof Tank Fitting Requirements; Gasketed covers, seals, lids | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.3.1 | Floating Roof Tank Fitting Requirements; Gasketed covers, seals, lids - Gap requirements | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.4 | Floating Roof Tank Fitting Requirements; Solid sampling or gauging wells - | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-320.4.1 | Floating Roof Tank Fitting Requirements; Solid sampling or gauging wells - projection below liquid surface | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-320.4.2 | Floating Roof Tank Fitting Requirements; Solid sampling or gauging wells - cover, seal, or lid | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-320.4.3 | Floating Roof Tank Fitting Requirements; Solid sampling or gauging wells - total secondary seal gap must include well gap | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-320.5 | Floating Roof Tank Fitting Requirements; Slotted sampling or gauging wells | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.5.1 | Floating Roof Tank Fitting Requirements; Slotted sampling or gauging wells -projection below liquid surface | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.5.2 | Floating Roof Tank Fitting Requirements; Slotted sampling or gauging wells -cover, gasket, pole sleeve, pole wiper for EFR wells | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.5.3 | Floating Roof Tank Fitting Requirements; Slotted sampling or gauging wells-total secondary seal gap must include well gap | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321 | Primary Seal Requirements | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321.1 | Primary Seal Requirements; No holes, tears, other openings | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321.2 | Primary Seal Requirements; The seal shall be metallic shoe or liquid mounted except as provided in 8-5-305.1.3 | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321.3 | Primary Seal Requirements; Metallic-shoe-type seal requirements | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321.3.1 | Primary Seal Requirements; Metallic-shoe-type seal requirements - geometry of shoe | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321.3.2 | Primary Seal Requirements; Metallic-shoe-type seal requirements - welded tanks | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322 | Secondary Seal Requirements | N |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322.1 | Secondary Seal Requirements; No holes, tears, other openings | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322.2 | Secondary Seal Requirements; Insertion of probes | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322.4 | Secondary seal requirements; Riveted tanks seal requirements | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322.5 | Secondary Seal Requirements; Gap requirements for welded external floating roof tanks with seals installed after 9/4/1985 | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322.6 | Secondary Seal Requirements; Extent of seal | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-328 | Tank Degassing Requirements | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-328.1 | Tank Degassing Requirements; Tanks > 75 cubic meters | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-328.2 | Tank Degassing Requirements; Ozone Excess Day Prohibition | Y |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-328.3 | Tank Degassing Requirements; BAAQMD notification required | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-331 | Tank Cleaning Requirements | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-331.1 | Tank Cleaning Requirements; Cleaning material properties | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-331.2 | Tank Cleaning Requirements; Steam cleaning prohibition | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-331.3 | Tank Cleaning Requirements; Steam cleaning exceptions | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-401 | Inspection Requirements for External Floating Roof Tanks | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-401.1 | Inspection Requirements for External Floating Roof Tanks; Primary and Secondary Seal Inspections | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-401.2 | Inspection Requirements for External Floating Roof Tanks; Tank Fittings Inspections | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-404 | Inspection, Abatement Efficiency Determination, and Source Test Reports | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-411 | Enhanced Monitoring Program (Optional) | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-411.3 | Enhanced Monitoring Program (Optional); Performance requirements | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-412 | Monitoring of Leaking Pontoons | N |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-501 | Records | N |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-501.1 | Records; Type and amounts of liquid, type of blanket gas, TVP - Retain 24 months | Y |  |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 8-5-501.2 | Records; Internal and External Floating Roof Tanks, Seal Replacement Records - Retain 10 years | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-501.3 | Records; Retention | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-502.2 | Source Test Requirements; 12-month source test for approved emission control systems and abatement devices for 8-5-328.1 or 331. | N |  |  | X | X | X | X | X | X | X | X | X | A B D | X |  | X | X |
| 8-5-602 | Analysis of Samples, True Vapor Pressure | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-604 | Determination of Applicability Based on True Vapor Pressure | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-605 | Measurement of Leak Concentration and Residual Concentrations | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-605.1 | Measurement of Leak Concentration and Residual Concentrations; EPA Method 21 Instrument | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-605.2 | Measurement of Leak Concentration and Residual Concentrations; Test Methods | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-606 | Analysis of Samples, Tank Cleaning Agents | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-606.1 | Analysis of Samples, Tank Cleaning Agents; IBP | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-606.2 | Analysis of Samples, Tank Cleaning Agents; TVP | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-606.3 | Analysis of Samples, Tank Cleaning Agents; VOC | N |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111 | Limited Exemption, Tank Removal From and Return to Service | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.2 | Limited Exemption, Tank Removal From and Return to Service, Tank in compliance prior to notification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.5 | Limited Exemption, Tank Removal From and Return to Service, Minimize emissions | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.6 | Limited Exemption, Tank Removal From and Return to Service, Notice of completion not required | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-111.7 | Limited Exemption, Tank Removal From and Return to Service, Satisfy requirements of 8-5-328 | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112 | Limited Exemption, Tanks in Operation | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.2 | Limited Exemption, Tanks in Operation, Tank in compliance prior to start of work. Certified per 8-5-404 | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-112.4 | Limited Exemption, Tanks in Operation, Not to exceed 7 days | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-117 | Exemption, Low Vapor Pressure | Y |  | X | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-301 | Storage Tank Control Requirements | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-304 | Requirements for External Floating Roofs; Floating roof requirements | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-304.4 | Requirements for External Floating Roofs; Floating roof requirements | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-320 | Tank Fitting Requirements | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.2 | Tank Fitting Requirements – Floating roof tanks, Gasketed covers, seals, lids – Projection below surface except p/v valves and vacuum breaker vents | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.3 | Tank Fitting Requirements; Gasketed covers, seals, lids | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.5 | Tank Fitting Requirements; Slotted sampling or gauging wells | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-320.5.2 | Tank Fitting Requirements; Slotted sampling or gauging wells -cover, gasket, pole sleeve, pole wiper for EFR wells | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321 | Primary Seal Requirements | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-321.3 | Primary Seal Requirements; Metallic-shoe-type seal requirements | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-322 | Secondary Seal Requirements | Y |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| 8-5-328 | Tank degassing requirements | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-328.1 | Tank degassing requirements; Tanks > 75 cubic meters | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-328.1.2 | Tank degassing requirements; Concentration of <10,000 ppm as methane after degassing | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-401 | Inspection Requirements for External Floating Roof Tanks |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-401.1 | Inspection Requirements for External Floating Roof Tanks; Primary and Secondary Seal Inspections | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-401.2 | Inspection Requirements for External Floating Roof Tanks; Tank Fittings Inspections | Y |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 8-5-404 | Certification | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-405 | Report | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-405.1 | Information required | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-405.2 | Information required | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-405.3 | Information required | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-501 | Records | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| 10-16 | Subpart Ka – Standards of Performance for Storage Vessels for Petroleum Liquids for which Construction, Reconstruction, or Modification Commence After June 11, 1973 and Prior to May 19, 1978 | Y |  |  |  |  | B | B |  |  |  |  |  |  |  |  |  |  |
| 60.110a(a) | Applicability and Designation of Affected Facility; Volatile organic liquid storage vessels ≥ to 40,000 gallons, after 5/18/1978 | Y |  |  |  |  | B | B |  |  |  |  |  |  |  |  |  |  |
| 63.1060 | Applicability: These requirements apply to storage tanks subject to control requirements under 40 CFR Part 63 Subparts EEEE and FFFF, and achieve compliance with those subparts by meeting the floating roof storage tank requirements of Subpart WW. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1063 | Standards: Floating Roof Storage Tanks | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1063(a)(1)(ii) | Floating roof rim seal standards for external floating roof storage tanks. | Y |  | C |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| 63.1063(a)(2) | Floating roof rim seal standards for both internal and external floating roof tanks. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1063(b) | Operational requirements for all floating roof storage tanks. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1063(c)(2) | External floating roof tank inspection and seal gap measurement frequency standards. | Y |  | C |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| 63.1063(d)(1) | Complete inspection procedure requirements for all floating roof tanks when emptied and degassed. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1063(d)(3) | Seal gap measurement procedures for external floating roof tank. | Y |  | C |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| 63.1063(e) | Repair requirements for deficiencies identified during inspections of floating roof storage tanks. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1065 | Recordkeeping requirements | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1065(a) | Maintain records of all storage tanks' dimensions, capacity, and stored liquid(s). | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1065(b) | Maintain records of all storage tank inspections. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1065(c) | Maintain records of all floating roof landing and refloating events. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1065(d) | Maintain records of all delay-of-repair extensions utilized. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1066(a) | Submit notification of initial startup of a tank. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1066(b)(1) | Submit notification at least 30 days prior to conducting seal gap measurements or an emptied and degassed complete inspection. If extenuating circumstances arise such that the inspection could not have been known 30 days in advance, notification shall be made at least 7 days prior to inspection. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1066(b)(2) | Submit a copy of inspection records that identify deficiencies. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.1066(b)(3) | Submit documentation of delay-of-repair allowances utilized for a particular tank. | Y |  | C |  | X | X |  |  | X |  |  |  |  |  |  |  |  |
| 63.2338 | Applicable equipment | Y | A | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2338(b)(1) | Applicable equipment: all storage tanks storing "organic liquids" as defined within 40 CFR 63.2406. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(a) | Standards: Storage tanks | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(a)(3) | Standards: Storage tanks: Comply with 40 CFR part 63, subpart WW (control level 2). | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(a)(6) | Standards: Storage tanks: Beginning no later than July 7, 2023, during storage tank shutdown operations (i.e., emptying and degassing of a storage tank) you must comply with paragraphs (a)(6)(i) through (iii) until the vapor space concentration in the tank is less than 10 percent of the lower explosive limit (LEL). Determine the LEL using process instrumentation or portable measurement devices and follow procedures for calibration and maintenance according to manufacturer's specifications. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(a)(6)(i) | Standards: Storage tanks: During tank emptying and degassing, remove organic liquids from the storage tank as much as practicable. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(a)(6)(ii) | Standards: Storage tanks: During tank emptying and degassing, route emissions through a closed-vent system to a control device capable of reducing total organic HAP by 95 weight percent or to an exhaust concentration of less than 20 ppmv corrected to 3% Oxygen for combustion devices using supplemental air, or route the emissions to a fuel gas system or other process. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(a)(6)(iii) | Standards: Storage tanks: During tank emptying and degassing, comply with the general duty to minimize emissions from as specified within 40 CFR 63.2350(d). Maintain records necessary to demonstrate the general duty standard was met including, if appropriate, records of procedures used to empty and degas equipment. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(k) | Standards: Storage tanks: Beginning no later than July 7, 2023, for each storage tank vented through a closed vent system to a flare you must comply with the requirements specified in §63.2380 instead of the requirements in §63.987 and the provisions regarding flare compliance assessments at §63.997(a), (b), and (c). | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2346(l) | Standards: Storage tanks: Startup, shutdown, and malfunction: Beginning no later than July 7, 2023, the allowance for non-compliance during startup, shutdown, and malfunction specified within 40 CFR Part 63 subpart SS are no longer applicable. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2378 | Continuous compliance demonstrations | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2378(c) | Continuous compliance demonstrations: Except as specified in paragraph (e) of this section, periods of planned routine maintenance of a control device used to control storage tanks, during which the control device does not meet the relevant emission standards, must not exceed 240 hours per year. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2378(e) | Continuous compliance demonstrations: Beginning no later than July 7, 2023, §63.2378(c) no longer applies. Instead, you must be in compliance with each emission limitation, operating limit, and work practice standard specified in paragraph (a) of this section at all times, except during periods of nonoperation of the affected source (or specific portion thereof) resulting in cessation of the emissions to which this subpart applies and must comply with the requirements specified in paragraphs (e)(1) through (5) of this section, as applicable. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2378(e)(1) | Continuous compliance demonstrations: Except as specified in paragraphs (e)(3) through (5), bypassing a control device is a deviation from the emission standards of Subpart EEEE. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2378(e)(3) | Continuous compliance demonstrations: Periods of planned routine maintenance of a control device during which the control device is bypassed or does not meet the the requirements of Subpart EEEE, must not exceed 240 hours per year. The level of material in the storage vessel shall not be increased during this time. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2382(a) | Submit notifications required by 63.2382 and Table 12 to Subpart EEEE. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| 63.2386(c-d) | Submit initial and subsequent compliance reports containing the required information relevant to storage tanks. | Y |  | C |  |  | X |  |  |  |  |  |  |  | X |  |  |  |

# APPLICABLE LIMITS & COMPLIANCE MONITORING REQUIREMENTS

## Section F Tanks

#### Table VII – F.3

Applicable Limits and Compliance Monitoring Requirements

Tank Group Applicable Limits & Compliance Monitoring Requirements

| Type | Citation | FE Y/N | Future Effective Date | Description | Citation | Frequency (P/C/N) | Type | Source # | 101 AB | 102 ABCD | 201 | 202 ABCD | 203 ABCD | 204 ABC | 301 | 302 | 303 AB | 401 | 402 | 403 ABCD | 404 ABC | 405 (Res.) | 501 | 502 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVP | BAAQMD 8-5-501.1 | Y |  | True vapor pressure | BAAQMD 8-5-501.1 | P/E initially and upon change of service | Look up table or sample analysis; Records |  |  | X | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| VOC | BAAQMD 8-5-412 | N |  | EFR leaking pontoons gas tight requirements | BAAQMD 8-5-412 | P/Q until repaired | Method 21 portable hydrocarbon detector |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| VOC | BAAQMD 8-5-401.2 SIP 8-5-401.2 | Y |  | EFR floating roof fitting closure standards; includes gasketed covers | BAAQMD 8-5-401.2 SIP 8-5-401.2 | P/SA | Measurement and visual inspection |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| VOC | BAAQMD 8-5-401.1 SIP 8-5-401.1 | Y |  | EFR primary rim-seal standards; includes gap criteria | BAAQMD 8-5-401.1 SIP 8-5-401.1 | P/SA and every time a seal is replaced | Seal inspection |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| VOC | BAAQMD 8-5-401.1 SIP 8-5-401.1 | Y |  | EFR secondary rim-seal standards; includes gap criteria | BAAQMD 8-5-401.1 SIP 8-5-401.1 | P/SA and every time a seal is replaced | Seal inspection |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| VOC | BAAQMD 8-5-401.1 8-5-401.2 8-5-411.3 (optional) | N |  | EFR floating roof fitting, primary and secondary seal standards | BAAQMD 8-5-401.1 8-5-401.2 8-5-411.3 (optional) | P/Q (optional) | Seal and fitting inspection; (enhanced monitoring) |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| VOC | BAAQMD 8-5-328.1 | N |  | Tanks > 75 m3 residual organic concentration of < 10,000 ppm as methane after degassing | BAAQMD 8-5-328.1 | P/each time emptied & degassed; 4 consecutive measure-ments at 15 minute intervals | Method 21 portable hydrocarbon detector |  |  |  | X | X | X | X | X | X | X | X |  | X | X |  | X | X |
| VOC | SIP 8-5-503 | Y |  | Tanks > 75 m3 concentration of < 10,000 ppm as methane after degassing | SIP 8-5-503 | P/each time emptied & degassed | Portable hydrocarbon detector |  |  |  | X | X | X | X | X | X | X | X |  | X | X |  | X | X |
| VOC | BAAQMD 8-5-501 | Y |  | Tanks > 75 m3 tank degassing control by liquid balancing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |  |  |  | X | X | X | X | X | X | X | X |  | X | X |  | X | X |
| VOC | BAAQMD 8-5-502 and 8-5-603.2 SIP 8-5-502 | Y |  | Tank degassing control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 SIP 8-5-502 | P/A | Source test |  |  |  | X | X | X | X | X | X | X | X |  | X | X |  | X | X |
| VOC | BAAQMD 8-5-404 SIP 8-5-404 SIP 8-5-405 | Y |  | Certification reports on tank inspections and source tests | BAAQMD 8-5-404 SIP 8-5-404 SIP 8-5-405 | P/ after each tank inspection and source test | Certification report |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| VOC | BAAQMD 8-5-501.2 | Y |  | Records of tank seal replacement | BAAQMD 8-5-501.2 | P/ for each tank seal replacement | Records (retain 10 years) |  |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  |
| VOC | BAAQMD 8-5-604 | Y |  | Determination of applicability | BAAQMD 8-5-604 | P/E | Look-up table or sample analysis |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X |
| HAP | 63.1063(c)(2)(ii) 63.1063(d)(3) | Y |  | EFR primary rim-seal standards; includes gap criteria | 63.1063(c)(2)(ii) 63.1063(d)(3) | P/ at 5 year intervals | Measurement and visual inspection |  |  | C |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| HAP | 63.1063(c)(2)(ii) 63.1063(d)(3) | Y |  | EFR secondary rim-seal standards; includes gap criteria | 63.1063(c)(2)(ii) 63.1063(d)(3) | P/A | Measurement and visual inspection |  |  | C |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| HAP | 63.1063(c)(1)(i)(B) 63.1063(c)(2)(iii) 63.1063(d)(2) | Y |  | Rim-seal standards, deck fitting standards, operational requirements, inspection and repair requirements | 63.1063(c)(1)(i)(B) 63.1063(c)(2)(iii) 63.1063(d)(2) | P/A or periodic each time emptied & degassed, at least every 10 years | Visual inspection |  |  | C |  | X | X |  | X |  |  |  |  |  |  |  |  |  |
| HAP | 63.1065(a) | Y |  | Recordkeeping | 63.1065(a) | P/E, Tank dimensions, capacity, and stored liquids | Records |  |  | C |  | X | X |  | X |  |  |  |  |  |  |  |  |  |
| HAP | 63.1065(b) | Y |  | Recordkeeping | 63.1065(b) | P/ Tank inspection records | Records |  |  | C |  | X | X |  | X |  |  |  |  |  |  |  |  |  |
| HAP | 63.1065(c) | Y |  | Recordkeeping | 63.1065(c) | P/ Floating roof landing and refloating events | Records |  |  | C |  | X | X |  | X |  |  |  |  |  |  |  |  |  |
| HAP | 63.1065(d) | Y |  | Recordkeeping | 63.1065(d) | P/ Delay of repair extensions | Records |  |  | C |  | X | X |  | X |  |  |  |  |  |  |  |  |  |
| HAP | 63.2346(a)(6) | Y |  | Control emissions from tank cleaning and degassing until LEL ≤ 10% | 63.2346(a)(6) | P/ each time emptied & degassed | Monitor LEL using process instrument or portable device |  |  |  |  |  | X |  |  |  |  |  |  |  | X |  |  |  |
| BAAQMD Regulation 8, Rule 5 Organic Compounds - Storage of Organic Liquids SIP Regulation 8, Rule 5 Organic Compounds - Storage of Organic Liquids |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NSPS 40 CFR 60 Subpart Kb Volatile Organic Liquid Storage Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 CFR 61 Subpart FF – Benzene Waste Operations NESHAP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 CFR 63 Subpart R Gasoline Storage Tanks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 CFR 63 Subpart WW Volatile Organic Liquid Storage Tanks – Incorporated by Reference by 40 CFR Part 63 Subparts EEEE and FFFF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 CFR 63 Subpart EEEE NESHAP for Organic Liquids Distribution (MACT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 CFR 63 Subpart FFFF NESHAP for Miscellaneous Organic Chemical Manufacturing (MACT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAAQMD Permit Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAAQMD Permit Conditions (Throughputs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |