## DRAFT ENGINEERING EVALUATION

Tesoro Refining & Marketing Company LLC

Plant Nos. 14628

Application No. 31991_like_demo

This document is the Air District’s Engineering Evaluation Report for the Tesoro Refining & Marketing Company LLC application for an Authority to Construct for its Use Existing LPG Tanks S-646 and S-647 to Store Renewable Propane, Application No. 31991_like_demo.

# 1.0 - Background

Tesoro Refining & Marketing Company LLC has requested a Certificate of Exemption for two existing pressurized LPG tanks, S-646 and S-647, at Plant 14628. Under this project, the tanks would be available for storage of renewable propane associated with the facility's renewable fuels operations.

The project does not include installation of new tanks, an increase in tank size, or a change to refinery production capacity. Instead, the project is a change in how two existing vessels may be used within the broader renewable fuels configuration. The project description provided by the applicant states that renewable propane production will not increase as a result of this action.

The tanks are existing pressure vessels that have historically been outside the evaluated scope of the renewable fuels process. The project therefore requires the engineering evaluation to confirm that S-646 and S-647 continue to qualify for exemption and to determine whether existing permit condition language should be revised so the tanks may participate in renewable propane storage service.

Based on the project specification, the permit action is narrow in scope. It applies only to S-646 and S-647 and does not authorize new process equipment, additional throughput, or any physical modification that would reasonably be expected to increase emissions.

# 2.0 Emissions Calculation

Pressurized LPG tanks are closed systems and are not expected to have routine direct emissions from storage service. For this project, S-646 and S-647 remain existing pressure tanks, and the proposed action only changes the permitted service to include renewable propane.

Any potential emissions associated with these tanks would be limited to minor fugitive losses from associated components, and those losses are expected to be negligible for the purposes of this permit action. Because the applicant is not proposing a production increase, a capacity increase, or a physical modification to the tanks, there is no basis to calculate an increase in direct tank emissions.

Table 1 - Estimated Emissions for Renewable Propane Storage Tanks

| Source No. | Description | Pre-Project POC (lb/day) | Pre-Project POC (tpy) | Post-Project POC (lb/day) | Post-Project POC (tpy) |
| --- | --- | --- | --- | --- | --- |
| S-646 | Tank A-646, pressurized renewable propane storage tank | 0 | 0 | 0 | 0 |
| S-647 | Tank A-647, pressurized renewable propane storage tank | 0 | 0 | 0 | 0 |

For this first-pass draft, the project is therefore evaluated as having no direct emissions increase.

# 3.0 Cumulative Increase

There is no emissions increase associated with the proposed action. The project adds no new source, does not physically modify S-646 or S-647, and does not increase renewable propane production at the facility. Accordingly, cumulative increase does not apply for this permit action.

# 4.0 Toxic Screening Analysis

This project does not create an increase in toxic air contaminant emissions. Because the proposed action is limited to use of existing pressurized tanks with no expected direct emissions increase, a health risk assessment is not triggered.

# 6.0 Offsets

Offsets are not required. The project does not result in an increase in emissions and does not constitute a new or modified source requiring offsets under Regulation 2, Rule 2.

# 7.0 Statement of Compliance

S-646 and S-647 are existing pressurized LPG tanks. Based on the project specification, the tanks are expected to remain exempt from permitting requirements under Regulation 2-1-123.3.1, provided that the exemption criteria continue to be satisfied. The project also relies on continued compliance with Regulation 2-1-319 for exempt equipment.

The evaluation assumes that renewable propane stored in S-646 and S-647 will not alter the exempt status of the tanks and that the tanks will continue to be maintained and operated as pressure vessels in accordance with applicable requirements. To the extent Regulation 8, Rule 5 applies to pressure tanks, the sources must continue to comply with the general standards for storage tanks, including maintenance of the tank shell, pressure integrity, and pressure relief device sealing mechanisms.

No information in the project specification indicates that this action would trigger Best Available Control Technology, offsets, PSD review, or a toxic risk assessment. The project is therefore considered consistent with continued exempt operation, subject to any conforming updates needed in permit-condition language and Title V documentation.

# 8.0 Permit Conditions

Permit Condition 27583 should be revised, as necessary, so that S-646 and S-647 are no longer included among the sources excluded from the renewable fuels process.

The revised condition should continue to prohibit use of the remaining listed sources in renewable fuels operations unless a permit application is submitted and approval is received from the Air District. The revision should be limited to recognizing S-646 and S-647 as renewable propane storage tanks within the project scope.

All other parts of Permit Condition 27583 should remain unchanged unless additional conforming edits are needed for clarity.

# 9.0 Title V Permit

The facility operates pursuant to a Title V permit. Any change to the authorized service of S-646 and S-647 and any associated revision to Permit Condition 27583 should be carried forward into the Title V permit record so the engineering evaluation, permit condition text, and major facility review documentation remain aligned.

# 10.0 Recommendation

I recommend that the Air District issue a Certificate of Exemption for S-646 and S-647 for renewable propane storage service, based on the project description provided and the determination that the tanks remain exempt pressure vessels.

I also recommend that Permit Condition 27583 be revised as needed so the condition language is consistent with the intended use of S-646 and S-647 in the renewable fuels process.

See July 11, 2022, Health Risk Assessment. An initial HRA was conducted on December 16, 2021 and updated on April 4, 2022. The April 4, 2022, update was conducted to include sulfuric acid mist emissions for S-1600 (via A-2000), which were inadvertently omitted in the initial health risk assessment. The inclusion of the sulfuric acid mist emissions did not increase the risk impacts because sulfuric acid mist is not a carcinogen and maximally impacted target organ groups for the hazard indices (chronic and acute) do not include sulfuric acid mist as a risk contributor. Therefore, the results of the December 16, 2021 HRA report are unchanged. During internal review of the permit application, additional sources have been identified with increases in TAC emissions and should have been included in the HRA. Those sources are S-1560, S-692, and S-819. Emissions from S-1560 are routed to the facility's fuel gas system and combusted at process heaters. Emissions from S-1560 are distributed to the emission points of each process heaters. In addition, there are also increases to one or more TAC pollutants for S-830, S-831, S-842, and S-1026; however, these sources are exempt from Regulation 2-5-401, Health Risk Assessment Requirements, per limited exemption 2-5-114 since the toxicity weighted emissions will not be increasing. Therefore S-830, S-831, S-842, and S-1026 are not included in the HRA. The health risk assessment was updated on July 11, 2022 to include emissions from S-1560, S-692, and S-819. The results of the July 11, 2022 HRA report supersede the previous versions dated December 16, 2021 and April 4, 2022.