Attachment C - Best Available Control Technology Analysis

Notice of Construction and Application for Approval

Marine Operations Voluntary Emission Control Project

Anacortes Refinery

Prepared for

Tesoro Refining & Marketing Company LLC

Prepared by
Barr Engineering Co.

March 2026

Attachment C
Anacortes Refinery

March 2026

Contents

1	Executive Summary	1

2	BACT Methodology	3

2.1	Top-Down BACT Approach	3

Step 1 - Identify all Available Control Technologies	3

Step 2 - Eliminate Technically Infeasible Control Technologies	4

Step 3 - Rank Technically Feasible Technologies by Control Effectiveness	4

Step 4 - Evaluate Technically Feasible Control Technologies	4

Step 5 - Select BACT	4

2.2	Constraints on BACT Emission Limits	5

2.3	Compliance with BACT during Startup, Shutdown and Malfunction	5

2.4	Emission Unit Parameters Used in the BACT Evaluation	5

3	Overview of Available Control Technologies	6

4	BACT for Vapor Combustion Unit	7

4.1	Vapor Combustion Unit PM10/PM2.5 Emissions	7

4.1.1	Step 1 – Identify All Available Control Technologies	7

4.1.2	Step 2 – Technical Feasibility of Control Technologies	7

4.1.3	Step 3 – Effectiveness of Feasible Control Technologies	8

4.1.4	Step 4 - Evaluation of Feasible Control Technologies	8

4.1.5	Step 5 – BACT Selection	9

4.2	Vapor Combustion Unit NOx Emissions	9

4.2.1	Step 1 – Identify All Available Control Technologies	9

4.2.2	Step 2 – Technical Feasibility of Control Technologies	9

4.2.3	Step 3 – Effectiveness of Feasible Control Technologies	10

4.2.4	Step 4 – Evaluation of Feasible Control Technologies	10

4.2.5	Step 5 – BACT Selection	10

4.3	Vapor Combustion Unit CO Emissions	12

4.3.1	Step 1 – Identify All Available Control Technologies	12

4.3.2	Step 2 – Technical Feasibility of Control Technologies	12

4.3.3	Step 3 – Effectiveness of Feasible Control Technologies	12

4.3.4	Step 4 – Evaluation of Feasible Control Technologies	13

4.3.5	Step 5 – BACT Selection	13

4.4	Vapor Combustion Unit SO2 Emissions	13

4.4.1	Step 1 – Identify All Available Control Technologies	13

4.4.2	Step 2 – Technical Feasibility of Control Technologies	13

4.4.3	Step 3 – Effectiveness of Feasible Control Technologies	14

4.4.4	Step 4 – Evaluation of Feasible Control Technologies	14

4.4.5	Step 5 – BACT Selection	14

5	BACT for MVEC Storage Tank	16

5.1	EFRT VOC Emissions	16

5.1.1	Step 1 – Identify All Available Control Technologies	16

5.1.2	Step 2 – Technical Feasibility of Control Technologies	16

5.1.3	Step 3 – Effectiveness of Feasible Control Technologies	17

5.1.4	Step 4 - Evaluation of Feasible Control Technologies	17

5.1.5	Step 5 – BACT Selection	18

6	BACT for Fugitive Components	19

6.1	Fugitive Component Organic TAP Emissions	19

6.1.1	Step 1 – Identify All Available Control Technologies	19

6.1.2	Step 2 – Technical Feasibility of Control Technologies	19

6.1.3	Step 3 – Effectiveness of Feasible Control Technologies	19

6.1.4	Step 4 - Evaluation of Feasible Control Technologies	19

6.1.5	Step 5 – BACT Selection	20

Tables

Table 1-1	Summary of Toxic Air Pollutants subject to tBACT	1

Table 1-2	BACT Analysis Summary	2

Table 2-1	Standards Establishing BACT “Floor”	5

Table 3-1	Available Emission Control Technologies	6

Table 4-1	Technical Feasibility of PM10/PM2.5 Control Technologies for the Vapor Combustion Unit	8

Table 4-2	Control Effectiveness Ranking of PM10/PM2.5 Control Technologies for the Vapor Combustion Unit	8

Table 4-3	Technical Feasibility of NOx Control Technologies for Vapor Combustion Unit	9

Table 4-4	Control Effectiveness Ranking of NOx Control Technologies for the Vapor Combustion Unit	10

Table 4-5	Technical Feasibility of CO Control Technologies for the Vapor Combustion Unit	12

Table 4-6	Control Effectiveness Ranking of CO Control Technologies for the Vapor Combustion Unit	12

Table 4-7	Technical Feasibility of SO2 Control Technologies for the Vapor Combustion Unit	14

Table 4-8	Control Effectiveness Ranking of SO2 Control Technologies for the Vapor Combustion Unit	14

Table 5-1	Technical Feasibility of VOC Control Technologies for the EFRT	17

Table 5-2	Control Effectiveness Ranking of VOC Control Technologies for the EFRT	17

Table 5-3	Cost-Effectiveness of VOC Control Technologies for the EFRT	17

Table 5-4	Environmental Impacts of VOC Control Technologies for the EFRT	18

Table 5-5	Energy Impacts of VOC Control Technologies for the EFRT	18

Table 6-1	Technical Feasibility of Organic TAP Control Technologies for Fugitive Components	19

Table 6-2	Control Effectiveness Ranking of Organic TAP Control Technologies for Fugitive Components	19

Appendices

Appendix A	RACT/BACT/LAER Clearinghouse Summary

Appendix B	Control Cost Calculations for Domed External Floating Root Tank

Abbreviations

BACT		Best Available Control Technology

CO		Carbon Monoxide

LAER		Lowest Achievable Emission Rate

MVEC		Marine Vapor Emission Control

NOx		Nitrogen Oxides

NWCAA	Northwest Clean Air Agency

PM10		Particulate Matter less than 10 microns in diameter

PM2.5		Particulate Matter less than 2.5 microns in diameter

RACT		Reasonably Available Control Technology

SO2		Sulfur Dioxide

TAPs		Toxic Air Pollutants

tBACT		Best Available Control Technology for Toxic Air Pollutants

VCU		Vapor Combustion Unit

VOCs		Volatile Organic Compounds

# Executive Summary

Tesoro Refining & Marketing Company LLC (Tesoro) is proposing to voluntarily install a new Marine Vapor Emission Control (MVEC) System utilizing a vapor combustion unit (VCU) that will reduce emissions of volatile organic compounds (VOCs) and toxic air pollutants (TAPs) displaced from marine vessels during loading operations. While emissions from marine vessel loading will decrease, emissions associated with combustion of the displaced vapors and fugitive leaks from the MVEC closed vent system are new emissions that are subject to review. This section addresses Best Available Control Technology requirements of Northwest Clean Air Agency (NWCAA) Regulation section 300.9.

To address NWCAA Regulation Section 300.9, Tesoro is providing a Best Available Control Technology (BACT) review for each emissions unit at which an emissions increase will occur above de minimis thresholds as a result of a physical change or change in the method of operation of the unit. For the proposed Marine Operations Voluntary Emission Control Project (Project), BACT is required for emissions of PM10, PM2.5, CO, NOx, and SO2. and certain toxic air pollutants (TAPs) associated with combustion from the MVEC System.

BACT for TAPs (tBACT) is required for emissions above the small quantity emission rates. Table 1-1 summarizes the TAPs subject to tBACT review and how those pollutants are addressed in this report.

Table 1-1	Summary of Toxic Air Pollutants subject to tBACT

| Pollutant | Surrogate Pollutant or Grouping |
| --- | --- |
| Nitrogen Dioxide | N/A – directly addressed |
| Sulfur Dioxide | N/A – directly addressed |
| Arsenic | PM10/PM2.5 |
| Cadmium | PM10/PM2.5 |
| Chromium (Hexavalent) | PM10/PM2.5 |
| Formaldehyde | Organic TAPs |
| 7,12-Dimethylbenz(a)anthracene | Organic TAPs |

This BACT analysis follows EPA’s five-step top-down approach, as specified in the U.S. EPA’s draft New Source Review Workshop Manual, (October 1990).

Step 1 – Identify All Available Control Technologies

Step 2 – Eliminate Technically Infeasible Options

Step 3 – Rank Remaining Control Technologies by Control Effectiveness

Step 4 – Evaluate Most Effective Control Technologies and Document Results

Step 5 – Select BACT

Table 1-2 below summarizes the determinations of the five step BACT analysis for each project-related emissions unit and pollutant subject to NWCAA BACT requirements.

Table 1-2	BACT Analysis Summary

| Source | Pollutant | Proposed BACT | Rationale |
| --- | --- | --- | --- |
| MVEC System [A] | PM10/PM2.5 | Clean Fuels Good Design Methods Good Operating Practices 0.0075 lb/MMBtu, 3-hr avg | Proposed PM10/PM2.5 performance is consistent with other applicable BACT determinations as well as AP-42 guidance for natural gas combustion, and there are no demonstrated and technically feasible combustion technologies applied to a similar marine loading operation which achieves better PM10/PM2.5 performance. |
|  | SO2 | Clean Fuels | There are no demonstrated SO2 control technologies for marine loading. |
|  | NOx | Clean Fuels Good Design Methods Good Operating Practices 0.138 lb/MMBtu, 3-hr avg | Proposed NOx performance is consistent with other applicable BACT determinations, and there are no demonstrated and technically feasible combustion technologies applied to a similar marine loading operation which achieves better NOx performance. |
|  | CO | Clean Fuels Good Design Methods Good Operating Practices 0.2 lb/MMBtu, 3-hr avg | Proposed CO performance is consistent with other applicable BACT determinations, and there are no demonstrated and technically feasible combustion technologies applied to a similar marine loading operation which achieves better CO performance. |
| External Floating Roof Storage Tank | VOC | No numeric limit | Proposed VOC performance is consistent with other applicable BACT determinations. |
| Fugitive Components | Organic TAPs | Leak Detection and Repair Program | Consistent with NSPS requirements, MACT requirements, and other applicable BACT determinations. |

[A]	Installation of the MVEC System as a control device is inherent to the Project scope and there will be no increase in VOC emissions from the Marine Terminal. Therefore, this unit is not required to undergo BACT review for VOC emissions.

# BACT Methodology

BACT is defined as an emission limitation based on the maximum emission reduction achievable after a case-by-case review of potential emission controls which takes into account energy, environmental and economic impacts. This emissions limit may be achieved by a variety of means, such as control technologies, clean fuels, inherently lower polluting processes or alternative operating practices.

## Top-Down BACT Approach

This BACT analysis has been conducted in accordance with NWCAA 300.9(B). Proposed BACT emissions limits have been selected using the “top-down” approach specified in U.S. EPA’s draft New Source Review Workshop Manual, (October 1990), using the five-step process.

### Step 1 - Identify all Available Control Technologies

All available control technologies are identified for each emission unit. A control technology is considered available for a specific pollutant if it could practically be applied to the specific emission unit. To identify all available control technologies, the following sources were consulted:

U.S. EPA’s RACT/BACT/LAER Clearinghouse (RBLC)

U.S. EPA draft permit review comments on recent PSD permits

State/local agency air quality permits and the associated agency review documents

Permit applications and BACT reports for recent projects

Air pollution control technology vendors and consultants

Manufacturer’s recommendations

Technical journals, reports, webinars, conferences and seminars

Certain court and regulatory agency determinations have held that “clean fuels” must be considered as one of the emission control technologies in a BACT analysis. The fuels analysis is based on pollutant emissions directly associated with use of a particular fuel. EPA has recognized that the initial list of control technologies for a BACT analysis does not need to include “clean fuel” technologies that would fundamentally redefine the source. Such technologies include those that would require a permit applicant to switch to a primary fuel type (i.e., coal, natural gas, or biomass) other than the type of fuel that an applicant proposes to use for its primary combustion process.

### Step 2 - Eliminate Technically Infeasible Control Technologies

Each control technology identified in Step 1 is evaluated, using source-specific factors, to determine if it is technically feasible. If physical, chemical or engineering principles demonstrate that a technology could not be successfully, reliably, and safely used on the emission unit, then that technology is determined to be technically infeasible. Economics are not considered in the determination of technical feasibility. Technologies which are determined to be infeasible are eliminated from further consideration.

### Step 3 - Rank Technically Feasible Technologies by Control Effectiveness

All technically feasible technologies are ranked in order of overall control effectiveness. Rankings are based on the level of emission control expressed as emissions per unit of production, emissions per unit of energy used, the concentration of a pollutant emitted from the source, control efficiency, or a similar measure. The control effectiveness listed will be representative of the level of emission control which can be achieved by the control technology at the operating conditions of the emission unit being reviewed. If the most effective control technology is selected as BACT, then Step 4 need not be completed.

### Step 4 - Evaluate Technically Feasible Control Technologies

The economic, environmental, and energy impacts of each control technology are evaluated. Step 4 is only required if the most effective control technology is not proposed as BACT. Tesoro did not complete Step 4 impacts analyses to support any of the BACT determinations in this report.

### Step 5 - Select BACT

Based on technical considerations and economic, environmental, and energy impacts, the proposed BACT for each emissions unit will include:

A pollutant-specific emission control technology as BACT, or a combination of controls when appropriate

A BACT emission limit or work practice standard, as appropriate

A BACT limit compliance demonstration method where an emission limit is stated, as appropriate

Summary tables of relevant RBLC determinations used in establishing proposed BACT limitations are located in Appendix A.

## Constraints on BACT Emission Limits

BACT emission limits for sources subject to New Source Performance Standards (NSPS) under 40 CFR Part 60, National Emission Standards for Hazardous Air Pollutants (NESHAPs) under 40 CFR Part 61, or Maximum Achievable Control Technology (MACT) standards issued under 40 CFR Part 63 cannot be less stringent than the applicable standards. In other words, these applicable standards provide a “floor” for BACT emission limits.

The review of NSPS, NESHAP, and MACT standards within Section 4.0 of the permit application indicates that the standards identified in Table 2-1 are applicable to the new equipment associated with the Project. There are no Part 61 NESHAPs applicable to new equipment associated with the Project.

Table 2-1	Standards Establishing BACT “Floor”

| Subpart | Affected Source | Pollutants Regulated |
| --- | --- | --- |
| NSPS Subpart GGGa | Fugitive Components | VOC |
| MACT Subpart CC | Fugitive Components | VOC as surrogate for organic HAPs |

The standards listed above for Parts 60 and 63 are discussed in the following sections of the BACT report as appropriate.

## Compliance with BACT during Startup, Shutdown and Malfunction

The MVEC will be operated intermittently depending on vessel loading activity and operational needs. Routine startup and shutdown of the MVEC is not expected to exceed proposed BACT standards and no separate startup / shutdown limitations are considered necessary. Good combustion practices will be followed at all times. Tesoro will execute industry-standard maintenance practices and respond to malfunctions that present excess emissions by shutting the MVEC down for repair. No alternative limits for malfunction conditions are deemed necessary.

The EFRT will be maintained and operated in compliance with 40 CFR 63, Subpart Kc—Standards of Performance for Volatile Organic Liquid Storage Vessels (Including Petroleum Liquid Storage Vessels) for Which Construction, Reconstruction, or Modification Commenced After October 4, 2023 (NSPS Kc), including the emission control requirements for degassing activities.

## Emission Unit Parameters Used in the BACT Evaluation

The BACT determinations in this report are based on projected emission unit operating parameters. See Section 3.3 of the permit application for estimated emissions utilized in the BACT evaluation. Note: emissions are based on potential to emit.

# Overview of Available Control Technologies

Available emission control technologies for the pollutants evaluated in this report are listed below in Table 3-1.

Table 3-1	Available Emission Control Technologies

| Pollutant | Control Technology |
| --- | --- |
| PM10/PM2.5 | Clean Fuels |
|  | Good Design Methods and Operating Practices |
|  | Fabric Filter (Baghouse) |
|  | Dry Electrostatic Precipitator |
|  | Wet Electrostatic Precipitator |
|  | Wet Scrubber |
| NOx | Selective Catalytic Reduction (SCR) |
|  | Selective Non-Catalytic Reduction (SNCR) |
|  | Low NOx Burners with Flue Gas Recirculation |
|  | Ultra Low NOx Burners |
|  | Clean Fuels |
|  | Good Design Methods and Operating Practices |
| CO | Clean Fuels |
|  | Good Design Methods and Operating Practices |
|  | Catalytic Oxidation |
|  | Thermal Oxidation |
| SO2 | Clean Fuels |
|  | Dry Scrubbing |
|  | Absorber (Wet Scrubber) |
|  | Dry Sorbent Injection |
| Organic TAPs | Leak Detection and Repair (LDAR) Monitoring |
|  | Optical Gas Imaging LDAR Monitoring |
| VOC - Tank | Good Design Methods and Operating Practices |
|  | Internal or External Floating Roof Tank (IFRT/EFRT) |
|  | Optical Gas Imaging (OGI) |
|  | Domed External Floating Roof Tank (DEFRT) |
|  | Routing to Closed Vent System (CVS) and Control Device (CD) |

# BACT for Vapor Combustion Unit

The new MVEC System will be used to control hydrocarbon emissions from marine vessels during loading operations at the wharf.

The displaced vapors will be collected by vapor hoses and routed to a Dock Safety Unit (DSU) that will be stationed on the wharf structure. The DSU is an essential piece of the overall MVEC System to ensure the safety for the ships, vessels and the overall MVEC System by safeguarding against over-pressure and flammable conditions in the vessels or vapor lines. A new 3” natural gas supply line to the DSU will be installed to supply enrichment gas needed when loading non-inerted vessels. The enrichment gas will be sourced from an existing (purchased) natural gas circuit within the refinery. The vapors exiting the DSU will be routed through an existing line available on the wharf/causeway structure to a Vapor Blower Unit (VBU) to provide motive force and then to the new Vapor Combustion Unit (VCU) to be located in the refinery. Natural gas will also be added as support gas to assist combustion (assist gas) in the VCU during loading of inerted vessels and to maintain adequate combustion zone temperature during low-flow and start-up conditions.

Pollutant emissions from the VCU include combustion products of both the natural gas and marine loading vapors. Proper design and operation of the VCU is intended to minimize the quantity of vapors that escape destruction. Good combustion practices and the use of clean natural gas as enrichment/assist gases, are intended to minimize the production of criteria pollutant emissions.

## Vapor Combustion Unit PM10/PM2.5 Emissions

Particulate matter emissions from combustion of gaseous fuels are typically low and consist of filterable and condensable fractions. The particulate matter emitted from natural gas and hydrocarbon vapor combustion processes is primarily condensable, consisting of larger molecular weight hydrocarbons which are not fully combusted. Condensable particulate matter exists as a gas in the stack but condenses in the cooler ambient air to form PM2.5.

### Step 1 – Identify All Available Control Technologies

Potential control technologies for PM emissions from a review of available information are listed in Table 3-1.

### Step 2 – Technical Feasibility of Control Technologies

The technical feasibility of potential control options for PM10/PM2.5 emissions is summarized in Table 4-1.

Table 4-1	Technical Feasibility of PM10/PM2.5 Control Technologies for the Vapor Combustion Unit

| Technology | Demonstrated In Practice? | Technically Feasible? |
| --- | --- | --- |
| Clean Fuels | Yes | Yes |
| Good Design Methods and Operating Practices | Yes | Yes |
| Fabric Filter (Baghouse) | No | No |
| Dry ESP | No | No |
| Wet ESP | No | No |
| Wet Scrubber | No | No |

The VCU will be an intermittently operated combustion device combusting gaseous fuels of variable nature depending on the product loaded. Particulate from natural gas and other vapors are primarily condensable particulates, which are not readily removed by filterable particulate control devices. At very low particulate concentrations, as is the case here, the use of add-on control technologies such as a baghouse, an electrostatic precipitator (ESP), or a wet scrubber is technically infeasible due to the capture limitations of these control technologies. These add-on control devices remove filterable particulate matter from solid and liquid fuel combustion processes. However, the particulate loading produced by vapor combustion is lower than what these controls are capable of effectively capturing. As a result, the application of post-combustion control devices is not technically feasible due to the minimal particulate emissions from the VCU.

### Step 3 – Effectiveness of Feasible Control Technologies

The technically feasible control options are ranked in Table 4-2, according to their control effectiveness.

Table 4-2	Control Effectiveness Ranking of PM10/PM2.5 Control Technologies for the Vapor Combustion Unit

| Rank | Technology | Emission Control Effectiveness | Basis for Listed Performance |
| --- | --- | --- | --- |
| 1 | Clean Fuels | 0.0075 lb/MMBtu | AP-42 Section 1.4 |
| 2 | Good Design Methods and Operating Practices | 0.0075 lb/MMBtu | AP-42 Section 1.4 |

### Step 4 - Evaluation of Feasible Control Technologies

The economic, environmental and energy impacts of technically feasible control options are not required, as the top feasible control option is selected. All feasible technologies have no difference in environmental or energy impact, since natural gas, as a clean fuel, has inherently low PM10/PM2.5 emission levels which make add-on control infeasible.

### Step 5 – BACT Selection

The proposed BACT for the MVEC System is clean fuels, combined with good design methods and operating practices.

The PM10/PM2.5 emissions performance of the MVEC System VCU will be 0.0075 lb/MMBtu on a 3-hour average. The MVEC to be installed will be a proven, commonly installed, and safe marine vapor control technology. Tesoro reviewed permitting determinations in Washington and the RBLC to confirm that this performance meets BACT, summarized as follows:

The proposed emissions performance is consistent with the total PM emission factor of 0.0075 lb/MMBtu from AP-42 Chapter 1.4: Natural Gas Combustion, Table 1.4-2.

The proposed emissions performance is consistent with the limit of 0.0074 lb/MMBtu from Venture Global Calcasieu Pass, LLC’s Calcasieu Pass marine loading flare for liquid natural gas loading. This value results from rounding down the PM emission factor from AP-42 Chapter 1.4: Natural Gas Combustion, Table 1.4-2.

Compliance will be demonstrated with performance testing. During unit startup or shutdown, good combustion practices will be followed in order to minimize emissions of PM.

## Vapor Combustion Unit NOx Emissions

There are three mechanisms by which NOx production occurs during combustion including thermal, fuel, and prompt NOx formation. For combustion of natural gas and other fuels with insignificant nitrogen content, the primary mechanism of NOx production is through thermal NOx formation. Natural gas and hydrocarbon vapors from vessel loading have minimal fuel bound nitrogen which eliminates fuel bound NOx as a major concern. Thermal and prompt NOx control is generally achieved by controlling combustion conditions (i.e. burners designed for lower flame temperature or via optimized combustion tuning) or via add-on emission control devices.

### Step 1 – Identify All Available Control Technologies

Potential control technologies for NOx emissions from a review of available information are listed in Table 3-1.

### Step 2 – Technical Feasibility of Control Technologies

The technical feasibility of potential control options for NOx emissions is summarized in Table 4-3.

Table 4-3	Technical Feasibility of NOx Control Technologies for Vapor Combustion Unit

| Technology | Demonstrated In Practice? | Technically Feasible? |
| --- | --- | --- |
| Clean Fuels (Natural Gas) with Good Design and Operating Practices | Yes | Yes |
| Selective Catalytic Reduction (SCR) | No | No |
| Selective Non-Catalytic Reduction (SNCR) | No | No |
| High-Efficiency/Low Emitting Design | No | No |

Add-on control devices such as SCR and SNCR generally require continuously stable flue gas conditions and are not technically feasible for the intermittently operated VCU. In most cases, the gas stream controlled by the VCU is of variable composition and concentration and presents a wide range of combustion conditions. Review of available information does not indicate any marine loading vapor combustion units installed that are controlled via SCR or SNCR.

Tesoro previously considered BACT for NOx using a high-efficiency, low-emitting burner design. However, subsequent evaluations revealed concerns regarding the technical feasibility of these units for marine loading operations, including design limitations such as the inability to handle multiple product loadings and operational issues with multi-stage designs. Safety issues were also identified, evidenced by an explosion with the first installation of one vendor’s low-NOx thermal oxidizer technology. Later installations by the same vendor have failed to consistently demonstrate low NOx performance levels. Therefore, Tesoro concludes that high-efficiency, low-emitting design vapor combustors are not a technically feasible control option for marine loading operations.

Good design practices include premix combustion technology to burn the VOC-laden gases and use of natural gas as assist and enrichment gas. Minimizing NOx emissions while maintaining an acceptable VOC destruction efficiency is considered part of good combustion practices.

### Step 3 – Effectiveness of Feasible Control Technologies

The technically feasible control options are ranked in Table 4-4, according to their control effectiveness.

Table 4-4	Control Effectiveness Ranking of NOx Control Technologies for the Vapor Combustion Unit

| Rank | Technology | Emission Control Effectiveness | Basis for Listed Performance |
| --- | --- | --- | --- |
| 1 | Clean Fuels (Natural Gas) with Good Design and Operating Practices | 0.138 lb/MMBtu | RBLC ID TX-0879 VCU determination for Motiva Enterprises, LLC Port Arthur Terminal |

### Step 4 – Evaluation of Feasible Control Technologies

The economic, environmental and energy impacts of technically feasible control options are not required, as the top technically feasible control option is selected.

### Step 5 – BACT Selection

The proposed BACT for the MVEC System is clean fuels, combined with good design methods and operating practices.

The NOx emissions performance of the MVEC System Vapor Combustion Unit will be 0.138 lb/MMBtu on a 3-hour average. The MVEC to be installed will be a proven, commonly installed, and safe marine vapor control technology. Tesoro reviewed permitting determinations in Washington and the RBLC to confirm that this performance meets BACT, summarized as follows:

The MVEC at BP Cherry Point permitted by NWCAA does not have any BACT NOx emission limits. Additionally, the MVECs at US Oil and Targa permitted by PSCAA in 2013 and 2017, respectively, do not have any BACT NOx limits and the agency thereby concluded conventional VCU technology presents BACT-level NOx performance.

The proposed emissions performance is consistent with the emissions limit for Motiva Enterprises, LLC Port Arthur Terminal for petroleum product loading.

A portable VCU permitted at Motiva Enterprises, LLC Port Arthur Terminal has a NOx BACT limit of 0.12 lb/MMBtu. However, this limit was permitted for VOC emission control during startup, shutdown, maintenance, or malfunction and is therefore not applicable to a system intended for standard operation.

A marine loading flare installed at the Venture Global Calcasieu Pass, LLC’s Calcasieu Pass LNG Project (RBLC ID LA-0331) has a NOx BACT limit of 0.068 lb/MMBtu. Upon review, the BACT determination does not establish a performance limit but instead establishes proper burner design and good combustion practices for the unit. However, the design of this unit is inherently different than the proposed MVEC System due to the significant difference in materials loaded (i.e., gasoline vs. natural gas). Additionally, this emission factor is set because it is equivalent to the AP-42 emission factor for industrial flares (AP-42 Section 13.5), which is not appropriate for a vapor combustor.

VCUs installed at the Oxy Ingleside Energy Center (RBLC ID TX-0844) have a NOx LAER limit of 0.06 lb/MMBtu. LAER is more stringent than BACT so the limit is not applicable to this project.

VCUs permitted for Jupiter Brownsville, LLC Centurion Brownsville (RBLC ID TX-0930) have a NOx BACT limit of 0.06 lb/MMBtu. However, this project was never constructed. Based on the PACAN Decision made by the Texas Supreme Court, a permitted limit that has not been demonstrated should not be considered BACT. Since the project at Centurion Brownsville was never constructed, the NOx limit of 0.06 lb/MMBtu for a VCU has not been demonstrated and is not applicable to the MVEC System.

TCEQ does not specify NOx BACT for VCUs. TCEQ guidelines indicate a NOx BACT limit of 0.06 lb/MMBtu for thermal oxidizers; however, this limit is not applicable to the VCU. Consistent with controls installed at most hydrocarbon loading operations, Tesoro is installing a vapor combustor, not a thermal oxidizer. Vapor combustors provide control for high Btu and variable flow hydrocarbon streams, whereas thermal oxidizers are typically characterized by separate enclosed combustion chambers designed for continuous low-Btu waste gas streams.

Bay Area Air Quality Management District (BAAQMD) and South Coast Air Quality Management District (SCAQMD) do not specify NOx BACT for marine loading. SCAQMD does regulate organic liquid loading operations controlled via flares at sources other than refineries at Rule 1118.1. Rule 1118.1 specifies a NOx limit of 0.034 lb/Mgal loaded. The proposed NOx performance meets this limit.

Compliance will be demonstrated through performance testing. During unit startup or shutdown, good combustion practices will be followed in order to minimize the amount of NOx generated.

## Vapor Combustion Unit CO Emissions

CO emissions from the VCU are a result of incomplete combustion of natural gas and displaced hydrocarbons. In fuel-rich mixtures, CO occurs as a direct product of combustion. In fuel-lean mixtures, CO can still occur due to poor air-fuel mixtures causing localized fuel-rich regions in the combustion zone.

### Step 1 – Identify All Available Control Technologies

Potential control technologies for CO emissions from a review of available information are listed in Table 3-1.

### Step 2 – Technical Feasibility of Control Technologies

The technical feasibility of potential control options for CO emissions is summarized in Table 4-5.

Table 4-5	Technical Feasibility of CO Control Technologies for the Vapor Combustion Unit

| Technology | Demonstrated In Practice? | Technically Feasible? |
| --- | --- | --- |
| Clean Fuels (Natural Gas) with Good Design and Operating Practices | Yes | Yes |
| Catalytic Oxidation | No | No |
| Thermal Oxidation | No | No |
| High-efficiency/Low Emitting Design | No | No |

Add-on control devices such as catalytic and thermal oxidation devices are not technically feasible for the intermittently operated VCU. In most cases, the VOC stream that a VCU controls is of variable composition and concentration. As a result, the associated burner must be designed to handle a wide range of combustion conditions and cannot be optimized. The high-efficiency/low-emitting design is not technically feasible as discussed in Section 4.2.2.

Clean fuels and good design and operating practices include premix combustion technology to burn the VOC-laden gases and use of natural gas as the assist or enrichment gas. Minimizing CO emissions while maintaining acceptable destruction efficiency is considered part of good combustion practices.

### Step 3 – Effectiveness of Feasible Control Technologies

The technically feasible control options are ranked in Table 4-6, according to their control effectiveness.

Table 4-6	Control Effectiveness Ranking of CO Control Technologies for the Vapor Combustion Unit

| Rank | Technology | Emission Control Effectiveness | Basis for Listed Performance |
| --- | --- | --- | --- |
| 1 | Clean Fuels (Natural Gas) with Good Design and Operating Practices | 0.2 lb/MMBtu | Vendor information |

### Step 4 – Evaluation of Feasible Control Technologies

The economic, environmental and energy impacts of technically feasible control options are not required, as the top feasible control option is selected.

### Step 5 – BACT Selection

Tesoro proposes that BACT for CO emissions from the MVEC System is clean fuels, combined with good design methods and operating practices. There are no feasible add-on control devices for CO.

The CO emissions performance by the MVEC System will be 0.2 lb/MMBtu on a 3-hour average. Tesoro reviewed final BACT determinations in the RBLC and found that performance limits ranged primarily from 0.2 to 0.376 lb/MMBtu CO, with a single lower determination at 0.004 lb/MMBtu for the VCUs permitted for Jupiter Brownsville, LLC Centurion Brownsville (RBLC ID TX-0930). However, this project was never constructed. Based on the PACAN Decision made by the Texas Supreme Court, a permitted limit that has not been demonstrated should not be considered BACT6. Since the project at Centurion Brownsville was never constructed, the CO limit of 0.004 lb/MMBtu for a VCU has not been demonstrated and is not applicable to the MVEC System. The expected performance of the MVEC system is consistent with the best-performing, demonstrated units listed in the RBLC.

No additional Washington state minor NSR BACT emission limits were identified. No CO limits were stipulated for the BP Cherry Point, Targa, or US Oil MVECs referenced in the NOx BACT.

Compliance will be demonstrated through performance testing. During unit startup or shutdown, good combustion practices will be followed in order to minimize the emissions of CO.

## Vapor Combustion Unit SO2 Emissions

SO2 emissions are due to the intermittent combustion of potential sulfur-containing compounds in the gaseous vapors from the marine vessels. Sulfur containing compounds may be present in the vapors from material loading, which would be oxidized to SO2 at the VCU. In a review of the RBLC, Tesoro did not identify any SO2 control technologies applied to VCU at other permitted facilities.

### Step 1 – Identify All Available Control Technologies

Potential control technologies for SO2 emissions from a review of available information are listed in Table 3-1.

### Step 2 – Technical Feasibility of Control Technologies

The technical feasibility of potential control options for SO2 emissions are summarized in Table 4-7. The following sections provide additional detail.

Table 4-7	Technical Feasibility of SO2 Control Technologies for the Vapor Combustion Unit

| Technology | Demonstrated In Practice? | Technically Feasible? |
| --- | --- | --- |
| Clean Fuels (Natural Gas) for Assist and Enrichment Gases | Yes | Yes |
| Spray Dryer Absorber | No | No |
| Absorber (Wet Scrubber) | No | No |
| Dry Sorbent Injection | No | No |

All add-on control technologies are infeasible. Flue gas desulfurization is not demonstrated on MVEC Systems. The outlet concentration of SO2 will be below the exhaust concentrations that can be achieved with add-on controls during typical operations. As such, coupled with the fact that the VCU is an intermittently operated combustion device, these add-on controls are not feasible control technologies for SO2.

### Step 3 – Effectiveness of Feasible Control Technologies

The technically feasible control options are ranked in Table 4-8, according to their control effectiveness.

Table 4-8	Control Effectiveness Ranking of SO2 Control Technologies for the Vapor Combustion Unit

| Rank | Technology | Emission Control Effectiveness | Basis for Listed Performance |
| --- | --- | --- | --- |
| 1 | Clean Fuels (Natural Gas) for Assist and Enrichment Gases | 0.0025 lb/MMBtu for Assist and Enrichment Gases | Site-Specific Natural Gas Emission Factor (Williams Natural Gas Data) |

### Step 4 – Evaluation of Feasible Control Technologies

The economic, environmental and energy impacts of technically feasible control options are not required, as the top feasible control option is selected.

### Step 5 – BACT Selection

The proposed BACT for the MVEC System is the use of clean fuels (natural gas) as assist and enrichment gases. There are no feasible add on control devices for SO2. Tesoro reviewed permitting determinations in Washington and the RBLC to confirm that this performance meets BACT, summarized as follows:

A marine loading flare installed at the Venture Global Calcasieu Pass, LLC’s Calcasieu Pass LNG Project has an SO2 BACT limit of 4 ppmv. However, the only displaced vapors being combusted at this flare are from liquified natural gas loading which has inherently low sulfur content. Therefore, the limit of 4 ppmv is not reasonable for systems loading higher sulfur-containing products.

VCUs permitted for Jupiter Brownsville, LLC Centurion Brownsville (RBLC ID TX-0930) have a SO2 BACT limit of 0.028 lb/MMBtu. However, this project was never constructed. Based on the PACAN Decision made by the Texas Supreme Court, a permitted limit that has not been demonstrated should not be considered BACT6. Since the project at Centurion Brownsville was never constructed, the NOx limit of 0.2 gr/dscf for a VCU has not been demonstrated and is not applicable to the MVEC System.

During unit startup or shutdown, good combustion practices will be followed in order to minimize emissions of SO2

# BACT for MVEC Storage Tank

The Project will include the installation of a new organic liquid storage tank to store finished products, primarily gasoline, prior to loading onto marine vessels.

## Storage Tank VOC Emissions

The new organic liquid storage tank will generate VOC and TAP emissions primarily through diffusion and minor leakage at the roof-to-shell interface, driven by product vapor pressure and influenced by temperature gradients and wind effects. Additional VOC emissions occur during normal operating conditions as the floating roof moves during filling or emptying, which displaces vapors trapped beneath the roof and within seal cavities.

### Step 1 – Identify All Available Control Technologies

Potential control technologies for VOC and TAP emissions from a review of available information are listed in Table 3-1.

For purposes of BACT evaluation, an EFRT that meets the applicable NSPS Kc design and operating requirements provides VOC emission control that is functionally equivalent to an IFRT. In the October 4, 2023 proposed NSPS Kc rule, the United States Environmental Protection Agency (EPA) determined that IFRTs represent the Best System of Emission Reduction (BSER) for volatile organic liquid storage vessels. However, in the same document, EPA explicitly states that EFRTs achieving equivalent emissions performance may be used as an alternative compliance option. This determination is reaffirmed in the following response made by the EPA to a public comment regarding the NSPS Kc 2023 proposed rule:

“We determined that IFRs are the BSER for storage vessels with vapor pressures

less than 11.1 psia. The options provided for EFRs are those that provide the same

emissions reduction as an IFR.”

Section IV.B of the preamble to the NSPS Kc final rule confirms that EPA considers EFRTs to be an approved alternative compliance option to IFRTs. Given that NSPS Kc establishes the BACT floor for this source category and EPA has determined that compliant EFRTs achieve emissions reductions equivalent to IFRTs, these technologies may be considered functionally equivalent for BACT evaluation. Therefore, it is unnecessary to assess both options, and this analysis will proceed with EFRTs.

Please also note that NSPS Kc requires controlling tank cleaning and degassing emissions and is included under the umbrella of “good design methods and operating practices” identified below.

### Step 2 – Technical Feasibility of Control Technologies

The technical feasibility of potential control options for VOC emissions from the EFRT is summarized in Table 5-1.

Table 5-1	Technical Feasibility of VOC Control Technologies for the EFRT

| Technology | Demonstrated In Practice? | Technically Feasible? |
| --- | --- | --- |
| Good Design Methods and Operating Practices | Yes | Yes |
| OGI | Yes | Yes |
| DEFRT | Yes | Yes |
| DEFRT + Routing to CVS and CD | Yes | Yes |

### Step 3 – Effectiveness of Feasible Control Technologies

The technically feasible control options are ranked in Table 5-2, according to their control effectiveness.

Table 5-2	Control Effectiveness Ranking of VOC Control Technologies for the EFRT

| Rank | Technology | Emission Control Effectiveness | Basis for Listed Performance |
| --- | --- | --- | --- |
| 1 | DEFRT + Routing to CVS and CD | 99.7% | DEFRT level of control plus additional 98% control from CVS and CD |
| 2 | DEFRT | 83% | AP-42 Methodology |
| 3 | OGI | Inherently lower emissions | N/A – early leak detection |
| 4 | Good Design Methods and Operating Practices | Inherently lower emissions | N/A – base case |

### Step 4 - Evaluation of Feasible Control Technologies

Tesoro evaluated the economic feasibility, as well as environmental and energy impacts of the VOC control technologies.

#### Economic Feasibility

Tesoro evaluated the cost-effectiveness of technically feasible VOC controls for the EFRT, summarized in Table 5-3. Details regarding economic feasibility for VOC controls are summarized in Appendix B.

Table 5-3	Cost-Effectiveness of VOC Control Technologies for the EFRT

| Technology | Cost Effectiveness ($/ton) | Economically Feasible? |
| --- | --- | --- |
| DEFRT + Routing to CVS and CD | $26,500 | No |
| DEFRT | $34,500 | No |
| OGI | N/A | No |
| Good Design Methods and Operating Practices | N/A | Yes |

The cost-effectiveness of a DEFRT with or without also routing emissions to a CVS and CD is well above what is considered economically feasible and is therefore eliminated from further consideration.

The cost-effectiveness of OGI cannot be evaluated on a $/ton basis since it is not possible to quantify the controlled emissions from the application of this technology. OGI has been demonstrated for storage tank monitoring and is required by the California South Coast Air Quality Management District (SCAQMD) and the Utah Administrative Code (UAC) for petroleum storage tanks meeting specific criteria. It is also a requirement included in the 2025 consent decree for the HF Sinclair Artesia refinery. However, EPA states in section IV.D of the NSPS Kc final rule that OGI requirements for storage vessels have been reviewed in the past, were reviewed again for NSPS Kc, and were determined to be not cost-effective. Therefore, OGI is eliminated from further consideration.

#### Environmental Impacts

Tesoro evaluated the environmental impacts of technically feasible VOC controls for the EFRT, summarized in Table 5-4.

Table 5-4	Environmental Impacts of VOC Control Technologies for the EFRT

| Technology | Collateral Air Emissions | Non-Air Environmental Impacts |
| --- | --- | --- |
| DEFRT + Routing to CVS and CD | NOx and CO if combustion control is used | Solid waste if carbon adsorption is used |
| DEFRT | N/A | N/A |
| OGI | N/A | N/A |
| Good Design Methods and Operating Practices | N/A | N/A |

There are no substantial adverse environmental effects from the technically feasible VOC control technologies.

#### Energy Impacts

Tesoro evaluated the energy impacts of technically feasible VOC controls for the EFRT, summarized in Table 5-5.

Table 5-5	Energy Impacts of VOC Control Technologies for the EFRT

| Technology | Energy Impacts |
| --- | --- |
| DEFRT + Routing to CVS and CD | Supplemental fuel if combustion control is used |
| DEFRT | N/A |
| OGI | N/A |
| Good Design Methods and Operating Practices | N/A |

There are no substantial adverse energy effects from the technically feasible VOC control technologies.

### Step 5 – BACT Selection

Tesoro proposes good design methods and operating practices via compliance with NSPS Kc as BACT for VOC emissions from the EFRT.

Tesoro reviewed permitting determinations in the RBLC and is not proposing any numeric emission limits for the EFRT. There is a single determination in the RBLC with a true performance limit for VOC emissions of 100 ppmv at Buckeye South Texas Gateway Terminal (RBLC ID TX-0855). However, this limit was permitted for a frac tank used during MSS activities, and so is not applicable to the EFRT being evaluated. Furthermore, the same facility permitted 30 IFRTs with no numeric emission limits and no add-on controls.

# BACT for Fugitive Components

## Fugitive Component Organic TAP Emissions

Control strategies for organic TAP emissions from fugitive components are based on LDAR program work practice requirements, which identify and then reduce emissions from process equipment components.

### Step 1 – Identify All Available Control Technologies

Potential control technologies for organic TAP emissions from a review of available information are listed in Table 3-1.

### Step 2 – Technical Feasibility of Control Technologies

The technical feasibility of potential control options for organic TAP emissions are summarized in Table 6-1. The following sections provide additional detail.

Table 6-1	Technical Feasibility of Organic TAP Control Technologies for Fugitive Components

| Technology | Demonstrated In Practice? | Technically Feasible? |
| --- | --- | --- |
| LDAR Monitoring | Yes | Yes |
| OGI LDAR Monitoring | Yes | Yes |

### Step 3 – Effectiveness of Feasible Control Technologies

The technically feasible control options are ranked in Table 6-2, according to their control effectiveness. In the case of the competing LDAR programs, the most effective control measures are fundamentally a matter of leak detection threshold. As such, the ranking for these technologies is as follows.

Table 6-2	Control Effectiveness Ranking of Organic TAP Control Technologies for Fugitive Components

| Rank | Technology | Emission Control Effectiveness | Basis for Listed Performance |
| --- | --- | --- | --- |
| 1 | LDAR Monitoring | Standard leak rates (generally based on 500 ppmv VOC) | 40 CFR 60.592a(a) and 63.658(c)(1)(ii) |
| 2 | Optical Gas Imaging LDAR Monitoring | Camera leak detection is on a qualitative basis; additional Method 21 monitoring required to quantify leak rate | EPA Handbook: Optical Remote Sensing for Measurement and Monitoring of Emissions Flux |

### Step 4 - Evaluation of Feasible Control Technologies

The economic, environmental and energy impacts of technically feasible control options are not required, as the top feasible control options are selected.

### Step 5 – BACT Selection

Tesoro proposes to utilize an LDAR monitoring program in accordance with the applicable requirements. The transfer operations are subject to LDAR requirements under 40 CFR 60 Subpart GGGa and 40 CFR 63 Subpart CC.

Appendix A

RACT/BACT/LAER Clearinghouse Summary

Appendix A	RACT/BACT/LAER Clearinghouse Summary

Appendix B

RACT/BACT/LAER Clearinghouse Summary

Appendix B	Control Cost Calculations for Domed External Floating Root Tank

The workshop manual can be found at U.S. EPA’s website http://www.epa.gov/NSR/ttnnsr01/gen/wkshpman.pdf.

.“BEST AVAILABLE CONTROL TECHNOLOGY (BACT) - An emission limitation based on the maximum degree of reduction for each air pollutant subject to regulation under chapter 70.94 RCW emitted from or which results from any new or modified stationary source, which the NWCAA, on a case-by-case basis, taking into account energy, environmental, and economic impacts, and other costs, determines is achievable for such stationary source or modification through application of production processes and available methods, systems, and techniques, including fuel cleaning, clean fuels, or treatment or innovative fuel combustion techniques for control of each such pollutant. In no event shall application of the “Best Available Control Technology” result in emissions of any pollutants which will exceed the emissions allowed by any applicable standard under 40 CFR Parts 60, 61, and 63. Emissions from any stationary source utilizing clean fuels, or any other means, to comply with this paragraph shall not be allowed to increase above levels that would have been required under the definition of BACT in the Federal Clean Air Act as it existed prior to enactment of the Clean Air Act Amendments of 1990.”

The workshop manual can be found at U.S. EPA’s website http://www.epa.gov/NSR/ttnnsr01/gen/wkshpman.pdf.

Northern Michigan University Ripley Heating Plant – PSD Appeal No. 08-02 before the US EPA Environmental Appeals Board and Petition Numbers IV-2008-1 and IV-2008-2 to the US EPA for re-consideration of Title V/PSD Air Quality Permit #V-07-017 for Cash Creek Generation, LLC facility located in Henderson, Kentucky.

Louisiana EDMS DocID: 11322607, p. 136

Supreme Court of Texas Case Summaries, February 14, 2025

The limit was permitted at 0.2 gr/dscf which is equivalent to 0.028 lb/MMBtu assuming the heating value of natural gas.

88 Federal Register 68,542 & 68,543 (October 4, 2023)

NSPS for Volatile Organic Liquid Storage Vessels Background Information for Final Rule - Summary of Public Comments and Responses (September 2024)

89 Federal Register 83,301 (October 15, 2024)

SCAQMD - Rule 1178

UAC - R311-203

HF Sinclair Navajo Refining LLC, Consent Decree

89 Federal Register 83,311 (October 15, 2024)