Technical Memorandum

UNIVERSAL MEMO TEMPLATE—READ BEFORE USING!

- There are no longer individual memo templates; instead, select Memorandum or Technical Memorandum from the Choose an item drop-down at the top

- If you don’t need the certification box, click on the border and delete it.

- If it’s an Attorney-Client Privilege memo, press return after the memo type and the header on the next page add:
Attorney-Client Privileged Communication:  Do Not Distribute without Consent of Counsel

- Include the full name of the Barr “entity” you’re representing in the text of the memo itself 
(Barr Engineering Co., Barr Engineering Michigan LLC, or Barr Engineering and Environmental Science Canada Ltd.; all should be shortened to Barr, not Barr Engineering).

- Select the appropriate address in the footer. Right-click anywhere at the bottom of the page and choose Edit Footer; then left-click in the “Choose an item” field to select an address from the dropdown list.

- To ensure that the header on succeeding pages functions correctly, follow these instructions: Click ONCE in the To, From, Subject, and Date fields and manually type pertinent information. It’s important to NOT copy and paste text into either of those two fields. If you follow this procedure, the headers on the following pages will automatically populate and update when you print, preview, or generate a PDF of your letter.

- Delete this box when you’re done.

Certification

Click or tap here to enter text.

MM/DD/YYYY

Click or tap here to enter text.		Date

PE #: Click or tap here to enter text.

To:	Project File

From:	Regulatory Analysis Support

Subject:	Offshore Loading Under 40 CFR 63 Subpart Y

Date:	March 28, 2026

Project:	MACT Y Rule History Review

# Purpose

This memorandum summarizes the regulatory history for offshore marine tank vessel loading operations under 40 CFR 63 Subpart Y and reviews whether the 2025 proposed rule reexamined whether add-on controls should be required for loading berths more than 0.5 miles from shore.

# Key Conclusions

Under the current rule, existing offshore loading terminals are not required to install the same MACT or RACT control devices that apply to onshore loading terminals.

EPA historically treated offshore loading differently because offshore vapor control was much more expensive than onshore control, had poor cost effectiveness, and raised technical and environmental concerns.

The 2025 proposal reviews the offshore issue again, but the proposal is framed as a clarification and reaffirmation of the existing offshore treatment rather than a proposal to newly require offshore controls.

# History Under the Current Rule

In the original Subpart Y rulemaking, EPA established separate offshore-terminal and Valdez subcategories. EPA explained that the MACT floor for existing offshore sources was no control of HAP emissions, while the MACT floor for new offshore sources was 95 percent reduction.

"For existing offshore terminals, the MACT floor is no control of HAP emissions. For new offshore terminals, the MACT floor is 95 percent reduction of HAP emissions."

EPA also concluded that offshore controls would cost roughly two to five times more than onshore controls and would not be reasonable as RACT because of the combination of cost, cost effectiveness, and offshore implementation challenges.

The rule also excluded offshore lightering and vessel-to-vessel bulk liquid transfer operations from the source category. Existing offshore terminals instead remained subject to submerged-fill requirements tied to Coast Guard rules.

Later amendments retained that structure. The 2011 residual risk and technology review kept the offshore subcategory outside the MACT control-device standards in 40 CFR 63.562(b) and relied on submerged loading requirements for existing offshore sources and certain lower-emitting sources.

# 2025 Proposed Rule Review

The 2025 proposed technology review expressly discusses the offshore definition, which remains tied to loading terminals with all loading berths at least 0.81 kilometer, or 0.5 miles, from shore.

The proposal does not identify a new offshore control technology basis for requiring add-on controls. Instead, EPA proposes to clarify that 40 CFR 63.560(d)(6) excludes existing offshore loading terminals from the MACT standards in 40 CFR 63.562(b), and EPA further proposes to clarify that the RACT provisions do not apply offshore.

The redline confirms that existing offshore terminals would continue to meet submerged-fill requirements under 46 CFR 153.282 rather than a broader vapor control requirement, and the supporting technology review tables continue to show submerged loading as the operative requirement for existing offshore categories.

# Why Controls Were Not Required Again

The current docket does not show EPA reversing the original offshore rationale. The 2025 record instead repeats the same core basis: offshore controls were previously found unreasonable because they were substantially more expensive and less cost-effective than onshore controls, and existing submerged-fill requirements already address loading practices for the offshore subcategory.

As a result, the better explanation of the proposal is that EPA reviewed offshore loading again and chose to preserve the existing approach. Existing offshore loading terminals remain outside the main MACT and RACT control-device requirements, while new offshore major sources remain subject to the 95 percent reduction standard identified in the original rulemaking.

EPA-HQ-OAR-2025-0207-0001, National Emission Standards for Marine Tank Vessel Loading Operations Technology Review, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0001\national-emission-standards-for-marine-tank-vessel-loading-operations-technology-review.pdf.

EPA-HQ-OAR-2025-0207-0010, Redline of 40 CFR Part 63 Subpart Y Showing Proposed Amendments, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0010\redline-of-40-cfr-part-63-subpart-y-showing-proposed-amendments.pdf.

EPA-HQ-OAR-2025-0207-0030, NESHAP Subpart Y Technical Support for Final Standards: Summary of Public Comments and Responses, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0030\neshap-subpart-y-technical-support-for-final-standards-summary-of-public-comments-and-responses.pdf.

EPA-HQ-OAR-2025-0207-0051, Technology Review for National Emission Standards Marine Tank Vessel Loading Operations, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0051\technology-review-for-national-emission-standards-marine-tank-vessel-loading-operations.pdf.

EPA-HQ-OAR-2025-0207-0030, NESHAP Subpart Y Technical Support for Final Standards: Summary of Public Comments and Responses, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0030\neshap-subpart-y-technical-support-for-final-standards-summary-of-public-comments-and-responses.pdf.

EPA-HQ-OAR-2025-0207-0030, NESHAP Subpart Y Technical Support for Final Standards: Summary of Public Comments and Responses, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0030\neshap-subpart-y-technical-support-for-final-standards-summary-of-public-comments-and-responses.pdf.

EPA-HQ-OAR-2025-0207-0030, NESHAP Subpart Y Technical Support for Final Standards: Summary of Public Comments and Responses, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0030\neshap-subpart-y-technical-support-for-final-standards-summary-of-public-comments-and-responses.pdf.

EPA-HQ-OAR-2025-0207-0010, Redline of 40 CFR Part 63 Subpart Y Showing Proposed Amendments, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0010\redline-of-40-cfr-part-63-subpart-y-showing-proposed-amendments.pdf.

76 FR 22595 and related 2011 residual risk and technology review materials at C:\Users\aaw\Codex\MACT Y\data\fr\76-fr-22595\final-rule.xml.txt.

EPA-HQ-OAR-2025-0207-0001, National Emission Standards for Marine Tank Vessel Loading Operations Technology Review, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0001\national-emission-standards-for-marine-tank-vessel-loading-operations-technology-review.pdf.

EPA-HQ-OAR-2025-0207-0001, National Emission Standards for Marine Tank Vessel Loading Operations Technology Review, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0001\national-emission-standards-for-marine-tank-vessel-loading-operations-technology-review.pdf.

EPA-HQ-OAR-2025-0207-0010, Redline of 40 CFR Part 63 Subpart Y Showing Proposed Amendments, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0010\redline-of-40-cfr-part-63-subpart-y-showing-proposed-amendments.pdf.

EPA-HQ-OAR-2025-0207-0051, Technology Review for National Emission Standards Marine Tank Vessel Loading Operations, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0051\technology-review-for-national-emission-standards-marine-tank-vessel-loading-operations.pdf.

EPA-HQ-OAR-2025-0207-0001, National Emission Standards for Marine Tank Vessel Loading Operations Technology Review, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0001\national-emission-standards-for-marine-tank-vessel-loading-operations-technology-review.pdf.

EPA-HQ-OAR-2025-0207-0030, NESHAP Subpart Y Technical Support for Final Standards: Summary of Public Comments and Responses, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0030\neshap-subpart-y-technical-support-for-final-standards-summary-of-public-comments-and-responses.pdf.

EPA-HQ-OAR-2025-0207-0030, NESHAP Subpart Y Technical Support for Final Standards: Summary of Public Comments and Responses, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0030\neshap-subpart-y-technical-support-for-final-standards-summary-of-public-comments-and-responses.pdf.

EPA-HQ-OAR-2025-0207-0051, Technology Review for National Emission Standards Marine Tank Vessel Loading Operations, docket file at C:\Users\aaw\Codex\MACT Y\data\regulations_gov\EPA-HQ-OAR-2025-0207\EPA-HQ-OAR-2025-0207-0051\technology-review-for-national-emission-standards-marine-tank-vessel-loading-operations.pdf.