Technical Memorandum

UNIVERSAL MEMO TEMPLATE—READ BEFORE USING!

- There are no longer individual memo templates; instead, select Memorandum or Technical Memorandum from the Choose an item drop-down at the top

- If you don’t need the certification box, click on the border and delete it.

- If it’s an Attorney-Client Privilege memo, press return after the memo type and the header on the next page add:
Attorney-Client Privileged Communication:  Do Not Distribute without Consent of Counsel

- Include the full name of the Barr “entity” you’re representing in the text of the memo itself 
(Barr Engineering Co., Barr Engineering Michigan LLC, or Barr Engineering and Environmental Science Canada Ltd.; all should be shortened to Barr, not Barr Engineering).

- Select the appropriate address in the footer. Right-click anywhere at the bottom of the page and choose Edit Footer; then left-click in the “Choose an item” field to select an address from the dropdown list.

- To ensure that the header on succeeding pages functions correctly, follow these instructions: Click ONCE in the To, From, Subject, and Date fields and manually type pertinent information. It’s important to NOT copy and paste text into either of those two fields. If you follow this procedure, the headers on the following pages will automatically populate and update when you print, preview, or generate a PDF of your letter.

- Delete this box when you’re done.

Certification

Click or tap here to enter text.

MM/DD/YYYY

Click or tap here to enter text.		Date

PE #: Click or tap here to enter text.

To:	Project File

From:	Barr Engineering Co.

Subject:	Proposed MACT Y Technology Review Impacts for Avon Wharf and Amorco Wharf

Date:	March 29, 2026

Project:	MACT Y Rule Review

# Purpose

This memorandum evaluates the proposed amendments to 40 CFR Part 63, Subpart Y for marine tank vessel loading operations and summarizes the likely impacts for the Avon Wharf and Amorco Wharf loading activities reflected in the current Title V permit record.[FN1][FN2][FN4]

# Executive Summary

EPA published the Subpart Y technology review proposal on March 4, 2026. As of March 29, 2026, the action remains proposed, and comments are due April 20, 2026.[FN1]

The proposal is primarily a compliance, monitoring, and reporting update rather than a rewrite of core applicability thresholds. EPA proposes periodic performance testing, electronic reporting, enhanced flare monitoring, removal of startup, shutdown, and malfunction exemptions, and tighter leak monitoring and repair provisions.[FN1][FN3]

Avon Wharf appears to be the primary directly affected facility because the current permit identifies Source S1560/A1560 as a marine bulk plant with a vapor recovery system and Title V applicable requirements under 40 CFR Part 63 Subpart Y, plus BAAQMD Condition 26406 requiring operation when loading regulated material and 98.5 percent VOC control.[FN2]

Amorco Wharf appears to be lower impact on the current permit record. The permit excerpt identifies wharf support equipment and an Amorco marine loading activity limited to renewable diesel, but it does not show a current Subpart Y vapor recovery system or Subpart Y applicable requirements comparable to Avon.[FN4]

For Avon, the likely compliance impact is moderate and mostly administrative and testing related unless EPA's flare provisions apply to an existing marine-loading flare or combustion device. For Amorco, direct impact appears limited unless the terminal loads regulated materials above 1.5 psia or has marine loading equipment not reflected in the permit extract reviewed here.[FN2][FN3][FN4][FN5]

# Detailed Proposed Changes

Remove startup, shutdown, and malfunction exemptions and related maintenance and affirmative defense provisions. EPA's direction is that Subpart Y standards must apply continuously, with comment requested on whether separate maintenance work practices are needed.[FN1][FN3]

Require electronic reporting through CDX/CEDRI for notifications and reports, with ERT-based submittals for performance tests and CEMS performance evaluations.[FN3]

Require periodic performance testing at least once every 60 calendar months for control devices currently subject only to an initial performance test. EPA also proposes a 180-day post-promulgation deadline for the first required test, with credit for tests conducted up to two years before the final rule.[FN1][FN3]

Require annual relative accuracy test audits for VOC CEMS where VOC CEMS are used as the compliance monitoring approach.[FN1][FN3]

Apply enhanced flare monitoring provisions to marine loading flares. EPA's proposal is patterned after refinery flare requirements and could require additional instrumentation, demonstrations, and ongoing monitoring at any Subpart Y flare-based control device.[FN1][FN3]

Clarify that annual Method 21 monitoring for vapor collection systems and control devices must be performed during active loading of regulated materials subject to 40 CFR 63.562(b), (c), or (d).[FN1][FN3]

Tighten leak repair requirements so repairs must be completed within 15 days of identification or prior to the next loading operation, whichever is later, and verified by re-monitoring.[FN1][FN3]

Clarify offshore treatment rather than expanding it. EPA continues to show submerged loading as the operative requirement for existing offshore terminals and proposes to clarify that the RACT provisions do not apply offshore.[FN1][FN3][FN5]

EPA discussed the existing 10/25-ton structure, the 1.5 psia trigger, and offshore treatment, but the proposal reviewed here does not appear to expand core Subpart Y applicability to lower-vapor-pressure diesel or renewable diesel loading.[FN1][FN3][FN5]

# Avon Wharf

The current Title V permit identifies Avon Wharf Berth No. 1A, Source S1560, as a marine bulk plant with A1560 vapor recovery. The permit lists loading of gasoline, blendstocks, diesel, distillate, residual oil, renewable diesel, and renewable naphtha, and the applicable requirements table includes 40 CFR Part 63 Subpart Y for S1560/A1560.[FN2]

Direct applicability risk is high relative to the other reviewed facilities because Avon already has an identified Subpart Y source and control device in the permit record.[FN2]

Periodic performance testing is likely the most concrete new obligation for Avon. If recent representative testing has not been performed within two years before a final rule, Avon would likely need a new test within 180 days after the effective date of the final rule.[FN1][FN3]

Electronic reporting would likely require process changes even if the technical control configuration stays the same. Test reports, recurring reports, and notifications would need to move into EPA's CDX/CEDRI workflow.[FN3]

The proposed active-loading Method 21 clarification and 15-day repair deadline would directly affect Avon if the vapor collection system or control device has leaking components. This is a tighter and more explicit federal compliance expectation than a simple first-attempt repair standard.[FN1][FN3]

The permit already requires A1560 to operate when loading regulated material and identifies a 98.5 percent VOC control expectation under BAAQMD Condition 26406. The proposal therefore looks more like a tightening of federal compliance demonstrations than a requirement to install a fundamentally new control system at Avon.[FN2]

Flare-related impacts are potentially significant but fact dependent. The reviewed permit language shows Avon primarily routed to vapor recovery and fuel gas handling rather than a dedicated Subpart Y flare. If Avon uses a flare or flare-like combustion device as the marine-loading compliance device in any operating mode, EPA's flare revisions could create the largest incremental capital and monitoring burden in this rulemaking.[FN2][FN3]

Removal of SSM exemptions could increase compliance risk during startup, shutdown, maintenance, or upset conditions affecting the vapor recovery and control system because EPA is moving toward continuous enforceability rather than special treatment for those periods.[FN1][FN3]

# Amorco Wharf

The permit record reviewed for Plant B2759 is materially different from Avon. The wharf-specific permitted sources identified are the Amorco Wharf slop tank and emergency diesel equipment, and the only loading activity expressly called out is Source S55, 'Amorco Terminal, Renewable Diesel Loading Only,' listed as exempt from permitting under BAAQMD Regulation 2-1-123.3.2 with a firm throughput limit.[FN4]

On the permit record reviewed here, there is no identified Amorco source equivalent to Avon S1560/A1560 and no clear Subpart Y applicable requirements table tied to a marine vapor recovery system.[FN4]

If Amorco is limited in practice to renewable diesel or other low-vapor-pressure diesel-range material, the current proposal does not appear to create a new direct Subpart Y control requirement because EPA did not propose changing the 1.5 psia trigger or extending the rule to lower-vapor-pressure diesel loading.[FN1][FN3][FN5]

If Amorco is treated as an offshore terminal for any berth configuration, EPA's proposal appears to preserve rather than eliminate the current offshore structure. Existing offshore terminals would remain tied to submerged-loading requirements, and EPA proposes to clarify that RACT does not apply offshore.[FN1][FN3][FN5]

The main risk at Amorco is factual rather than regulatory: if the terminal loads any regulated materials above 1.5 psia, or if the current permit record omits a marine loading control system that is actually in service, then Amorco could move closer to the Avon-style impact profile and would need a separate applicability review.[FN4][FN5]

Based on the materials reviewed, Amorco's near-term implications are best characterized as low direct impact but high need for confirmation of actual commodities, berth configuration, and whether any non-diesel marine loading remains authorized or active.[FN4][FN5]

# Recommended Next Steps

Confirm the current commodity slate at Avon and Amorco, including whether either terminal loads any material above 1.5 psia and whether Amorco operates as an offshore terminal under the Subpart Y definition.[FN2][FN4][FN5]

Map each marine-loading control device and compliance monitoring approach now in use, including whether Avon uses VOC CEMS, a flare, or another combustion device in any Subpart Y compliance scenario.[FN2][FN3]

Compare existing leak monitoring, repair, and reporting practices against the proposal so implementation gaps can be identified before rule finalization.[FN1][FN3]

If client comments are being considered, focus comments on offshore applicability clarity, flare applicability for intermittent or alternate marine-loading control paths, and the practicality of the proposed testing and reporting deadlines for refinery-integrated marine systems.[FN1][FN3][FN5]