Tesoro Refining & Marketing Company LLC

Martinez Renewable Fuels Facility

150 Solano Way

Martinez CA  94553

February 27, 2025

Mr. Bao Trinh

Permit Services Division

Bay Area Air Quality Management District

375 Beale Street, Suite 600

San Francisco, CA 94105-2066

###### Subject:  	Response to Incompleteness Letter (Application No. 720986)

###### Application for Foul Water Strippers (FWS) Permit Modifications

###### Martinez Renewable Fuels Facility; Facility Number B2758

Dear Mr. Trinh:

Tesoro Refining & Marketing Company LLC applied for an Authority to Construct requesting alternative control scenarios for the Foul Water Stripper (FWS) to route the gas stream to the existing flare header system when the Foul Water Stripper Off-Gas Thermal Oxidizer in not available as a control device at Martinez Renewable Fuels Facility (Facility Number B2758). In a letter dated January 14, 2025, the Bay Area Air Quality Management District deemed the application incomplete with a request for additional information. This letter provides responses to each of the numbered items from the incompleteness letter.

District Comments (bolded)

- Project Information – Scenario 2a – Flare Combustion

  - While Marathon expects to primarily send FWS offgas to the fuel gas system, how does Marathon decide when to reroute FWS offgas to the flare header?

  - Per Regulation 12-12, flaring needs to be minimized or not occur at all unless there’s an emergency. Marathon’s proposed routine flaring scenario is inconsistent with the intent of 12-12, so we believe that scenario 2a is not permittable.

- Tesoro expects to primarily send FWS offgas to the A-2000 thermal oxidizer (TO); however, similar to any other process unit at the facility, the TO system is subject to failures and malfunctions. As such, to ensure safe and effective management of the FWS offgas, Tesoro is proposing to route the FWS offgas to the flare header when the TO is not available. Once in the flare header, the gases will either be recovered at the flare gas recovery (FGR) compressors and utilized in the fuel gas system or sent to the flare tip if there is insufficient FGR capacity.  Examples of periods when the TO is not available include when the TO is unstable and either shuts down (i.e., trips) or results in incomplete combustion.

- Tesoro is not requesting approval to flare routinely or as part of normal operations but rather only as a contingency measure to ensure safe operations; as such, Tesoro’s proposed alternative scenario during emergencies is consistent with Regulation 12-12. When considering safety risks, Tesoro does not consider the TO in isolation but as a fully integrated system that includes upstream and downstream equipment. Additionally, any emergency flaring would only occur consistent with the site’s flare management plan (FMP) and Section 12-12-301, which states that flaring is allowable when “the flaring is caused by an emergency and is necessary to prevent an accident, hazard or release of vent gas directly to the atmosphere.”

Tesoro is in the process of updating its FMP to address the FWS offgas alternative scenario, pursuant to Section 12-12-404 requirements. The facility has sufficient flare gas recovery compressor capacity to compress the additional FWS offgas flow during the proposed emergency scenarios. Tesoro will continue to comply with the reporting and recordkeeping requirements of Regulation 12-12, including Sections 12-12-405 and 12-12-406, and report all flaring events over 500,000 standard cubic feet per calendar day.

- Project Information – Scenario 2b – Process Heater Combustion

  - Provide the list of sources in the fuel gas system and their corresponding permitted limits. Daily and annual limits may need to be imposed for sources that don’t have existing daily and annual limits.

The following table lists the fuel gas combustion devices along with their permitted daily and annual heat input limits in million British Thermal Units (MMBtu) per day and year, respectively:

| Source Number | Daily Heat Input Limit (MMBtu/day) | Annual Heat Input Limit (MMBtu/year) |
| --- | --- | --- |
| 919 | 2,664 | 972,360 |
| 920 | 1,512 | 551,880 |
| 928 | 480 | 175,200 |
| 929 | 480 | 175,200 |
| 930 | 480 | 175,200 |
| 931 | 480 | 175,200 |
| 932 | 480 | 175,200 |
| 933 | 480 | 175,200 |
| 934 | 3,240 | 1,182,600 |
| 937 | 17,832 | 6,508,680 |
| 973 | 2,640 | 963,600 |
| 1511 | 5,520 | 2,014,800 |
| 1512 | 5,520 | 2,014,800 |
| Total | 39,144 | 15,259,920 |

The fugitive component emissions associated with the fuel gas system are included under unit A-1584 and the associated POC permitted limits are 336.3 lb/day and 122,570 lb/yr.

Tesoro does not believe that this project requires any changes to the daily or annual limits. The FWS offgas represents a negligible fraction of total fuel gas usage. The maximum thermal oxidizer (TO) duty is 2.2 MMBtu/hr, or 52.8 MMBtu/day, which is 0.13% of the total 39,144 MMBtu/day permitted capacity. Additionally, the FWS offgas would only be fired in heaters for a maximum of 500 hours per year or 0.0072% of the total annual permitted fuel gas usage, further limiting its impact. Given these factors, FWS offgas use would not materially affect fuel gas flow, composition, or permitted operations, making additional restrictions unwarranted.

  - Provide a process flow diagram of all labeled sources downstream in the fuel gas system, per alternative control scenario 2b – process heater combustion. Depending on permitted limits, downstream sources may be modified or altered if FWS offgas is routed to the combustion equipment.

  - In the emission calculations, footnote 1 says “…FWS offgas is sufficiently similar to the existing fuel gas”. The calculated post project PTE shows an increase in ammonia vs. normal operation which would constitute a modification in sources in the fuel gas system since there is an increase in ammonia at those sources. Please submit the toxics profile and heating value for the FWS offgas and the existing renewable fuel gas.

The fuel gas drum is a dynamic system, continuously blending offgases from multiple process units with varying compositions and flow rates. The toxics profile and heating value of the FWS offgas are inconsequential compared to the overall fuel gas characteristics because the flow is negligible (significantly less than 0.5%), as noted in the response to Question 2.a above. Refer to Attachment A for the estimated combined composition.

This project does not increase the ammonia generated from the FWS (A-1600) but only changes the emission point from the FWS TO to downstream fuel gas combustion devices or the flare in limited emergency scenarios. The ammonia is not generated by these devices; rather, the devices serve as alternative control options when the primary control device is unavailable.

The increase in ammonia PTE under the fuel gas and flare scenarios is due to a conservative destruction/conversion factor assumption of 99% from Texas Commission on Environmental Quality (TCEQ) guidance rather than the higher vendor guarantee used for the FWS TO.

Even using the more conservative TCEQ destruction factor, the A-1600 ammonia post-project PTE remains 0.880 lb/hr and 1,348 lb/yr—significantly below the acute (1,400 lb/hr) and chronic (7,770 lb/yr) trigger levels in Table 2-5-1 of Regulation 2-5. Pursuant to Section 2-1-234.1.3, Rule 2-5 requirements are satisfied, as the project does not result in emissions exceeding the trigger levels in Table 2-5-1.

  - How did Marathon quantify 500 hours/year as the estimate for the amount of time A-2000 will be taken offline?

Tesoro has conservatively assumed 500 hours per year for A-2000 downtime based on past emergency downtime events and a safety factor. The 500 hours per year estimate also aligns with the industry-standard approach used for other emergency equipment, such as engines, ensuring sufficient time for necessary repairs to the primary control device while maintaining operational flexibility and safety.

  - Is the FWS offgas treated to meet the 162 ppmv H2S limit of Permit Condition #27604 and Subpart Ja prior to distributing FWS offgas at the inlet of each process heater?

    - How does Marathon verify the H2S concentration at the inlet of each process heater?

Yes, the FWS offgas is treated to meet the 162 ppmv H2S limit prior to distributing FWS offgas at the inlet of each process heater. For BAAQMD’s understanding, the FWS offgas does not remain a separate stream until it reaches the inlet of each downstream unit. Rather, the FWS offgas is fully integrated into the fuel gas system at the No. 5 Gas Plant and treated by the A-2001 H2S Adsorption Vessels. The treated fuel gas stream is monitored by an H2S CEMS prior to distribution to combustion devices to demonstrate compliance with Permit Condition #27604.

Note that Subpart Ja does not apply to the process heaters because the facility is no longer a petroleum refinery. Additionally, Tesoro has applied to revise its Title V permit to revise conditions relating to Subpart J and its EPA consent decree. Tesoro agrees that a 162 ppmv H2S limit applies under its EPA consent decree.

  - Provide proposed permit condition language for routing emissions to the fuel gas system. Only proposed permit condition language for routing emissions to flare header was provided.

The proposed permit condition language allows for the rerouting of FWS offgas to the flare header, where it may either:

- Be combusted in a flare connected to the flare gas header (Scenario 2a), or

- Be captured by the flare gas recovery unit, integrated into the fuel gas system, and combusted in downstream fuel gas combustion devices (Scenario 2b).

Since both Scenario 2a (flaring) and Scenario 2b (process heater combustion) are already included in the proposed permit condition language, no additional permit modifications are necessary.

  - Is there an increase in fugitive component count? If so, provide new and replaced quantities of each component.

There is no increase in fugitive component count. This project utilizes existing piping, which is already represented in the existing component counts that form the PTE basis, and no new fugitive components are being added or replaced.

- A-2000 Thermal Oxidizer

  - Is it feasible for Marathon to replace the current Zeeco thermal oxidizer with a larger thermal oxidizer or add a larger thermal oxidizer to run parallel based on quantity of FWS gas?

We are studying the underlying issues that have been contributing to the instability of the thermal oxidizer.  This includes evaluating both the thermal oxidizer and upstream improvements to prevent hydrocarbons from reaching the Foul Water Stripper.  At this time it is not clear if replacing the thermal oxidizer with a larger version would fully resolve the issues. Regardless, a larger TO would not eliminate the need to reroute the FWS offgas during TO downtime, and a larger TO would still require maintenance and experience unplanned outages.

  - A-2000 was permitted with a 1.91 MMBtu/hr firing rate in Application #30768. What is the basis of increasing the permitted 1.91 MMBtu/hr firing rate to proposed 2.2 MMBtu/hr?

A-2000 was originally permitted at 1.91 MMBtu/hr based on the best available information at the time of Application #30768. The latest data sheet specifies a maximum heat input capacity of 2.2 MMBtu/hr. Refer to the data sheet in Attachment B.

  - Does the manufacturer specification sheet support the proposed firing rate increase? If so, please provide an updated manufacturer specification sheet with that detail. The least manufacturer vendor sheet was provided on 7/14/23 and that vendor sheet did not list a firing rate.

Refer to the attached data sheet. Note, the maximum total heat release listed on the sheet is on a LHV basis.

  - Why is Marathon requesting a large increase in hourly CO emission rates (from permitted value of 10 ppmv to requested 350 ppmv @ 15% O2)?

  - The manufacturer specification sheet, provided on 7/14/23 after issuance of the A/C for Application #30768, provided an emissions guarantee of 100 ppmv @ 15% O2 for CO (equal to 0.22 lb/MMBtu). The RACT CO values Marathon requested were from an Air District policy dated 1999, so those proposed RACT values are outdated. Is Marathon willing to accept 107 ppmv @ 15% O2 as the CO emission limit?

Tesoro will accept 107 ppmv CO at 15% O2 emission limit (equal to 0.24 lb/MMBtu) on a daily basis. The updated PTE calculations are included in Attachment C.

  - The HRA was evaluated using max hourly acute emissions. Emission limits in the permit condition can’t be revised from a max hourly limit to a daily average as that’s not representative of what was modeled. Instead of average, please provide proposed max hourly limits for ammonia and max hourly and annual H2S limits. An HRA will be reconducted with the proposed max limits.

The HRA modeling assumed all sources emit 24 hours per day, 7 days a week, with hourly emission rates derived by dividing the annual emission rates by 8,760 hours per year. The daily averages use these modeled hourly emission rates.

  - Once available, please provide Source Test Section reviewed and approved source test values that contain the revised minimum destruction efficiency of ammonia.

Refer to the attached stack test report in Attachment D.

- A-2002 H2S Adsorption Vessels

  - What is the bases for the 140 dscfm flowrate? Based on previous understanding of the A-2002 system, the outlet of S-1600/inlet of A-2002 must be 140 dscfm to achieve the residence time to adsorb H2S emissions effectively.

  - Please propose a continuous flow probe monitor at the inlet of A-2002 to demonstrate compliance with the 140 dscfm flowrate limit. Submit manufacturer specifications for the proposed continuous flow probe monitor. The current method of using valve curves, based upon pressure and valve position, to estimate the flowrate is not acceptable.

The H2S adsorber was designed based on a 140 dscfm flow rate with a margin of safety. The design was primarily intended to meet the SO2 emission limit at the thermal oxidizer stack at all times. However, the new CEMS (Attachment E) will include a stack flow meter which will allow us to more accurately calculate the emissions from the FWS.  As such, a flow meter at the inlet  of A-2002 is no longer necessary to demonstrate compliance with emission limitations and we request removal of the 140 dscfm requirement.

Refer to the attached flow meter specifications in Attachment E.

- Form HRA

  - Submit Form HRA and HRA map for A-2000 Thermal Oxidizer.

Refer to Attachment F for the Form HRA and HRA map.

- CEMS

  - Please provide a copy of the manufacturer’s specifications for the CEMS.

Refer to Attachment E for CEMS manufacturer’s specifications.

- CEQA

  - Has an NOE/Neg. dec/NOD been filed with the County of Contra Costa (County)?

  - Provide a copy of the project application submitted to the County for review and the County’s response.

Yes, an NOE was submitted to the County on February 8, 2025. Refer to Attachment G for the submitted project application. Tesoro will forward to the District the response from the County when it is received.

  - Please clarify why the emissions analysis in the EIR wouldn’t need to be revised to reflect the proposed increase in TAC emissions limits.

An updated emissions analysis was provided to the County.

If you have any questions, please feel free to call me at (925) 708-0159.

Sincerely,

Sharon Lim

Senior Environmental Engineer

Attachments:

Attachment A		Fuel Gas and FWS Offgas Estimated Stream Composition Calculations

Attachment B		A-2000 Data Sheet

Attachment C		Updated PTE Calculations

Attachment D		Foul Water Stripper (S-1600) Compliance Source Testing

Attachment E		Flow Meter and CEMS Specifications

Attachment F		HRA Form and Map

Attachment G		CEQA Application

The specification sheet states a total heat release value of 2.0 MMBtu/hr on a low heating value (LHV) basis. The HHV basis is 10% higher at 2.2 MMBtu/hr.