Tesoro Refining & Marketing Company LLC

Martinez Renewable Fuels Facility

150 Solano Way

Martinez CA  94553

April XX, 2025

Mr. Bao Trinh

Permit Services Division

Bay Area Air Quality Management District

375 Beale Street, Suite 600

San Francisco, CA 94105-2066

###### Subject:  	Response to Incompleteness Letter (Application No. 720986)

###### Application for Foul Water Strippers (FWS) Permit Modifications

###### Martinez Renewable Fuels Facility; Facility Number B2758, Plant 14628

Dear Mr. Trinh:

Tesoro Refining & Marketing Company LLC applied for an Authority to Construct requesting alternative control scenarios for the Foul Water Stripper (FWS) to route the gas stream to the existing flare header system when the Foul Water Stripper Off-Gas Thermal Oxidizer (TO) is not available as a control device at Martinez Renewable Fuels Facility (Facility Number B2758). The Bay Area Air District (BAAD) deemed the application incomplete in a letter dated January 14, 2025. Tesoro responded to the January incompleteness letter in a letter dated February 27, 2025. BAAD sent a second incompleteness letter dated March 26, 2025. This letter provides responses to each numbered item from the March incompleteness letter.

District Comments (bolded)

- Project Information – Scenario 2a – Flare Combustion

  - The Air District will not incorporate changes to permit conditions that allow flaring. If flaring happens during emergencies, then the remedial measures should align with the approved FMP.

Tesoro is not requesting a permit condition that would allow flaring, and we will continue to comply with our Flare Management Plan. The primary request is to remove the prohibition to abate the FWS by the TO at all times and replace it with an allowance of 500 hours for periods of startup, shutdown, or malfunctions. The discussion regarding the destination of the offgas during the 500 hours (i.e., fuel gas system or flare system) was provided to BAAD to determine the emissions impacts during those hours.

- Project Information – Scenario 2b – Process Heater Combustion

  - See attached calcs spreadsheet for existing PTE limits for the process heaters. Please confirm if Tesoro is willing to accept the NOx and SO2 limit per process heater and accept the facility wide Reg 9-10 NOx bubble cap of 834 lb/day.

Tesoro will accept the NOx and SO2 mass emission limits on each process heater, in addition to the existing facility-wide Reg 9-10 NOx bubble cap of 834 lb/day.

  - S-937 Hydrogen Plant Heater (F37) has a current daily limit of 1,430 lb/day of NOx from permit condition #677 which exceeds the bubble cap. Please propose a new daily limit and emission factor that will allow routine operation of S-937 without exceeding the bubble cap limit. Permit condition #677 will be amended to reflect S-973’s new daily NOx limit.

Tesoro proposes a new daily limit of 834 lb/day of NOx emissions at S-937, with compliance demonstrated by the existing continuous emission monitoring system consistent with how compliance is demonstrated with permit condition #677. Proposed revisions to Condition #677, part 1 are as follows, and justification for the proposed limit is included in the response to item (c) below.

S937 Hydrogen Plant Heater

Administratively Revised via Application 19647 (March 2009) Consolidation of Bubble Condition 4357 with Condition 8077  
Administratively Revised by Application 19874 (July 2009) Updates for Combustion Sources

Administratively Revised by Application 720986 (Month Year) for Foul Water Stripper Offgas Process Heater Combustion

1. Permittee/Owner/Operator shall ensure that the mass emissions of nitrogen oxides (NOx), calculated as NO2, from furnace, S-937 do not exceed 834 1430 lb/stream day or 1089 lb/calendar day. (basis: cumulative increase, Bubble Condition 4357/8077 via Application 19647)

  - Provide a basis for the proposed daily emission limit for S-937 (i.e. how the value was derived, historically high records obtained from renewables operation, etc.).

Tesoro recognizes that the current daily limit of 1,430 lb/day required by Condition #677 is less stringent than the Reg 9-10 NOx bubble cap of 834 lb/day, which applies to S-937 and additional refinery process heaters. The proposed daily limit of 834 lb/day at S-937 will be a legally enforceable limitation that has the effect of limiting emissions and thus may be taken into account when determining the source’s PTE, consistent with Rule 2-1-234.1.1.

S-937 is listed in Title V Table II A1B as having a firm limit on firing at 743 MMBtu/hr per Condition #16685, part 1. Additionally, S-937 can physically operate to the full extent of the limitation. Low NOx burners rated at 743 MMBtu/hr were installed in 2002 via Application 2721. Additionally, emissions performance has been 0.047 lb/MMBtu or greater during a calendar day. Therefore, the physical capability to emit NOx from the source is greater than the proposed limitation of 834 lb/day.

Reducing this limit from 1,430 lb/day to 834 lb/day ensures that there will be no increase in the potential to emit (PTE) from S-937 and that no “modification” will occur as a result of this project.

  - If the existing NOx and SO2 limits are declined,

    - Tesoro will need to quantify NOx and SO2 emissions increase at each process heater from the FWS offgas to demonstrate altered/modified status.

    - Provide an unlocked excel workbook that quantifies the NOx and SO2 increase at each process heater.

Tesoro will accept the existing NOx and SO2 limits as described in the responses above.

- A-2000 Thermal Oxidizer

  - Instead of average hourly emissions, please provide proposed max hourly limits for ammonia and max hourly and annual H2S limits using the attached “Guidance for Calculating Maximum Hourly Toxic Air Contaminant Emission Rates”. Max hourly emissions * 24 hours can then be used as max daily limits.

Tesoro confirmed that the hourly and annual emission rates represented are acceptable as limits. The prior request to change the averaging period was intended to provide additional flexibility with the compliance demonstration.

  - Instead of a 3 ppmv @ 15% O2 increase in H2S emissions, will Tesoro accept a 2 ppmv @ 15% O2 limit based on previous vendor guarantee? 2 ppmv was evaluated in Application 30768, but a zero emission limit was used because it was incorrectly assumed that all H2S emissions would be converted to SO2 emissions post combustion. The Air District will allow this change without constituting it as a modification. If 3 ppmv is used, then there is an increase in H2S emissions which would make A-200 modified.

Tesoro will accept a 2 ppmv @ 15% O2 limit for A-2000 if this results in a change that is not considered a modification.

  - Provide and unlocked excel workbook containing the following emissions calculations:

    - Increase in criteria pollutant emissions from increase from 1.91 MMBtu/hr firing rate to 2.2 MMBtu/hr.

    - CO emissions calculations for revised CO limit based on 107 ppmv @ 15% O2 RACT limit.

    - Increase in TAC pollutants from combustion.

Refer to the attached workbook.

- GHG Emissions

  - Provide an unlocked excel workbook containing the GHG emissions calculated in the original application submittal.

Refer to the attached workbook.

- Application Fees

  - Pay revised application fees in the amount of $119,039 (Invoice #T180735).

Application fees will be paid under separate cover.

If you have any questions, please feel free to call me at (925) 708-0159.

Sincerely,

Sharon Lim

Senior Environmental Engineer

Attachments:

Emission Calculation Workbook

Permit Evaluation and Statement of Basis for Renewal of the Title V permit, April 2011, page 716

For example, this was exceeded on January 1, 4, 5, and 10 of 2024 during renewables operation. A threshold of 0.047 lb/MMBtu was evaluated because 0.047 lb/MMBtu * 743 MMBtu/hr * 24 hr/day > 834 lb/day.