Project Description

Tesoro is seeking to approval from the District for alternative control scenarios for the FWS S-1600 when the A-2000 Thermal Oxidizer (TO) is not available. During times when the TO is not available, Tesoro plans to route gases through the existing ductwork located after the A-2002 H2S Adsorption Vessels to the flare header system. From the flare header system, the exhaust gases may either be combusted in one of the flares connected to the flare gas header (herein referred to as “Scenario 2a – Flare Combustion”) or captured in the flare gas recovery unit for combustion in one of the downstream fuel gas combustion devices (herein referred to as “Scenario 2b – Process Heater Combustion”). Scenario 2b – Process Heater Combustion is expected to be the primary scenario because the facility has sufficient flare gas recovery compressor capacity to recover the additional flow rate from the FWS during the alternative control scenarios. Scenario 2a – Flare Combustion is only expected during certain shutdown events or malfunctions. Tesoro proposes to use these two alternative control scenarios not more than 500 hours per year total for both scenarios.

Tesoro proposes to update the following representations:

- Increased A-2000 firing rate from 1.91 million British Thermal Units per hour (MMBtu/hr) to 2.2 MMBtu/hr, and update the associated description from “Sour Water Stripper” to “Foul Water Stripper” to be consistent with the site’s nomenclature.

A process flow diagram focused on the foul water system depicts Scenarios 2a and 2b and is included below.

New Source Review (NSR) Applicability as a “Modification” (Rule 2-1-234)

For purposes of this ATC application, Tesoro has evaluated whether the proposed changes result in an increase in potential to emit (PTE) as defined under District’s Rule 2-1-234.1 and/or an increase over actual emissions baseline as defined under Section 2-1-234.2.

PTE: Pre-project PTE under Section 2-1-234.1 is determined by existing permit limits, where available, and past permit application representations. The updated S-1600/A-2000 PTE reflects the following changes:

- Increased maximum firing rate from 1.91 million British Thermal Units per hour (MMBtu/hr) to 2.2 MMBtu/hr,

- Increased hourly CO emission rate from 0.006 pound per hour (lb/hr) to 1.760 lb/hr based on BAAQMD’s Reasonable Available Control Technology (RACT) CO emission factor (0.8 lb/MMBtu) for thermal oxidizers with a firing rate of 7.5 MMBtu/hr or less,

- Newly represented hydrogen sulfide (H2S) emission factor based on 3 ppmv H2S at 15% (oxygen) O2 engineering estimate, and

- GWPs used by CARB in the current GHG inventories.

These updated calculations show an increase in daily and annual emission levels for all regulated pollutants except NOx and SO2, and, therefore, a “modification” will occur as a result of the project.

Actual Emissions: To determine an increase in emissions under Section 2-1-234.2, the District Rule incorporates U.S. EPA’s federal “major modification” definition, which applies to changes at a source that will cause projected future emissions to be higher than historical emission levels (known as an “actual-to-projected-actual” test). Even when conservatively considering baseline actual emissions as zero, and assuming post-project PTE as representative of projected actual emissions, the emission increases are less than the significance threshold for all regulated pollutants.

A “modification” will occur for S-1600/A-2000 because the project results in an operation change that increases the daily and annual emission levels for all regulated pollutants except NOx and SO2. As such, the project will require an Authority to Construct.

New Source Review Requirement (Section 2-1-301)

This rule applies to modified sources subject to the requirements of Section 2-1-301 or 2-1-302. The purpose of this rule is to implement the NSR provisions of the federal and California Clean Air Acts. This section describes how the project complies with the substantive requirements of Best Available Control Technology (BACT), Offsets, Prevention of Significant Deterioration (PSD), and other applicable air quality impact analyses. Each of the elements of NSR is discussed below.

BACT

BACT is required for modified sources meeting the conditions specified in Section 2-2-301.2. BACT requirements are considered for each of the operating scenarios as follows:

- The S-1600/A-2000 post-project PTE for all regulated pollutants, except CO, are less than 10 lb/day and do not trigger BACT, as presented in Attachment C (Emission Calculations). The updated S-1600/A-2000 post-project CO PTE (refer to the discussion above for more information) exceeds 10 lb/day. Since the CO emissions are secondary pollutants that are a direct result of the use of an abatement device for the control of another pollutant, Reasonable Available Control Technology (RACT) is required pursuant to Section 2-2-102. BAAQMD defined CO RACT for thermal oxidizers with a firing rate of 7.5 MMBtu/hr or less as 0.8 lb/MMBtu.1 A-2000 meets the CO RACT based on the initial performance test.

- Alternative control Scenario 2a (flaring) exceeds 10 lb/day for NOx and CO. RACT is required pursuant to Section 2-2-102 because NOx and CO emissions are secondary pollutants that are is a direct result of the use of an abatement device for the control of another pollutant. The flares are already subject to 40 CFR 63 Subpart FFFF, National Emission Standards for Hazardous Pollutants (NESHAP) for Miscellaneous Organic Chemical Manufacturing (commonly referred to as “MON”), and Tesoro has elected to comply with the 40 CFR 63 Subpart CC, NESHAP from Petroleum Refineries (commonly referenced to as “Maximum Achievable Control Technology (MACT) CC”) flaring requirements, pursuant to §63.2450(e)(5). Compliance with the applicable requirements for flaring satisfies RACT for NOx and CO.

- Alternative control Scenario 2b (process heater combustion) is also conservatively assumed to trigger RACT requirements. The process heaters already comply with Regulation 9, Rule 10, which meets RACT.

Offsets

Sections 2-2-302 and 2-2-303 provide the emission offset requirements for POC, NOx, particulate matter less than 2.5 microns (PM2.5), particulate matter less than 10 microns (PM10), and SO2. Offsets are triggered based on the Cumulative Increase Calculations Procedures described in Sections 2-2-607 and 2-2-608. As demonstrated in the analysis provided in Attachment C (Emission Calculations), the project requires the following emission reduction credits (ERCs) as provided in Table 1 below. Certificates 1144 and 1838 can be used for offsets.

Table 1 - ERC Summary

| Pollutant | Cumulative Increase (tpy) | Offset Ratio | Offsets Required (tpy) |
| --- | --- | --- | --- |
| POC | 0.063 | 1.15 | 0.073 |
| NOx | 0.040 | 1.15 | 0.046 |
| PM2.5 | 0.003 | 1.00 | 0.003 |
| PM10 | 0.003 | 1.00 | 0.003 |
| SO2 | 0 | 1.00 | 0 |

PSD

Major facility review is triggered if the emissions increase or net emissions increase is above the significance thresholds of PSD for attainment pollutants. The permitting analysis included in Attachment C (Emission Calculations) confirms that the proposed project is not a PSD project and is therefore exempt from the PSD BACT requirements of Section 2-2-304 and exempt from the PSD source impact analysis requirements of Sections 2-2-305 and 2-2-306.

New or Modified Sources of Toxic Air Contaminants or Hazardous Air Pollutants (2-1-316)

This section applies to new or modified sources that emit one or more toxic air contaminants (TACs) in quantities that exceed the trigger levels listed in Table 2-5-1 of Regulation 2-5. Emission calculations are provided in Attachment C (Emission Calculations) which show that emissions do not exceed trigger any levels.

New Source Review of Toxic Air Contaminants (Rule 2-5)

Tesoro has evaluated changes to levels of TAC as a result of the proposed project under Section 2-5-214 and concluded there will be increases in daily and annual emissions of TACs. Therefore, S-1600/A-2000 is considered “modified” under Rule 2-5.

Health Risk Assessment (HRA) Requirements (District Rule 2-5-401)

BAAQMD has requested an updated HRA reflecting the revised emission rates at A-2000 in the HRA completed for the Martinez Renewable Fuels project. Revised emission rates are included in Attachment C (Emission Calculations). Tesoro will submit an updated Health Risk Assessment (HRA) under a separate cover letter after alignment with BAAQMD staff.

California Environmental Quality Act Applicability

Tesoro believes this permitting action is exempt from CEQA review because the project has no potential for causing a significant adverse environmental impact.

New Source Performance Standards (NSPS) Applicability

The project does not change NSPS applicability for the A-2000 Thermal Oxidizer, flares, or downstream fuel gas combustion devices.

National Emission Standards for Hazardous Air Pollutants (NESHAP) Applicability

The project does not change 40 CFR Part 61 or Part 63 NESHAP applicability for the A-2000 Thermal Oxidizer, flares, or downstream fuel gas combustion devices.

https://www.baaqmd.gov/~/media/files/engineering/policy_and_procedures/bart_ract/noxandcoractforthermaloxidizers.pdf

https://ww2.arb.ca.gov/ghg-gwps