Memorandum

To:	Kirby Dahlquist, Kristin Heutmaker, Calvin Johnson (St. Paul Park Refining Co. LLC (SPPRC)), Haley Ilg, Andrew True, and Dan Hido (Marathon Petroleum Company)

From:	Phil Fish, Sara Wandrei, and Emily Byrd (Barr Engineering Co. (Barr))

Subject:	Final Title V Permit (-102) Changes Summary

Date:	November 11, 2022

Project:	23821187.03

This memorandum summarizes the pertinent permit changes (structure, condition language, and unit IDs) and PTE calculation changes in the final Title V permit (16300003-102) issued in November 2022 compared to the existing SPPRC’s existing Title V permit (16300003-101).

This memorandum is meant to facilitate the internal SPPRC and Marathon review processes, and it is organized into the following sections.

- Permit-wide Changes

- Total Facility Section (TFAC)

- Heaters and Boilers

- Meters

- Wastewater Treatment Plant (WWTP)

- Equipment Leaks

- Flare

- Tanks

- FCC

- Engines

- Loading

- Sulfur Recovery

- Catalyst

- Reformer

# Permit-wide Changes

MPCA made several broad changes that affected most permit sections, and nearly all of the conditions were altered in some way. The changes range from high-level structural changes to small-scale permit condition formatting updates. The following list summarizes the pertinent changes, but it is not an exhaustive list.

- Permit structure changes

  - Added appendices with the verbatim federal performance standard language (e.g., all of NSPS Subpart A, MACT CC, etc.) and accompanying permit conditions for each subject item referencing the applicable citations (e.g., “The permittee must comply with all applicable requirements of 40 CFR pt. 63, subp. A. as follows: 40 CFR 63.6(6), …”).

  - Added and removed different component groups (COMGs). For example, MPCA removed the emergency engine COMGs (COMG 35-37) and listed the associated conditions under the associated EQUI IDs (EQUI 25 (Fire Hall), EQUI 327 (Boiler House), EQUI 335 (Fire Water Pump 1), etc.). Additionally, MPCA added COMG 41 and COMG 42 to consolidate all of the meter requirements from NSPS and MACT CC, respectively,

  - Added many of the meters to the permit (refer to Section 4.0 below for additional details),

  - Removed subject items that are no longer appropriate (e.g., VEPRs, portable engines (COMG 9)) and associated fuel oil conditions that were pending EPA’s SIP approval (e.g., “EACH OF THE FOLLOWING COMG 20 (GP 006) CONDITIONS AND LIMITS WILL BE TERMINATED ON THE DATE THE USEPA APPROVES THE REVISION TO THE SIP”), as requested,

  - Changed some emission unit categories (e.g., “Process Heater”, “Hopper”, “Other Emission Unit”) in MPCA’s TEMPO system; therefore, MPCA had to create new EQUI IDs because of TEMPO’s inflexibility. For example, 28-B-1’s category was changed from the “Stripping Equipment” category to the “Process Heater” category; therefore, MPCA had to create a new EQUI ID (EQUI 1 -> EQUI 789), and

  - Reorganized many emission unit categories to follow a consistent language template for the source category (e.g., meters, engines, tanks, heaters, etc.).

- Formatting and Citation Changes

  - Completed minor formatting changes; including:

    - Removed “Section X of this subpart” and replaced it with the full citation reference (i.e., “40 CFR 6X.XXX(X)),

    - Removed the first letter citation at the beginning of the condition (e.g., “(a) The Permittee …” -> “The Permittee …”),

    - Added “The permittee shall limit” in front of all of the emission limits (e.g., “NOx < 39 tpy” to “The permittee shall limit NOx < 39 tpy”), and

    - Used the full pollutant name instead of the abbreviation (“e.g., “SO2” -> “Sulfur Dioxide”) in specific locations (e.g., “Sulfur Dioxide Emission Calculations and Recordkeeping:”).

  - Added the Minnesota rules that incorporate federal performance standards by reference (e.g., Minn. R. 7011.1435(A) incorporates NSPS J). However, MPCA consistently referenced the state rule for a performance standard that only applies because of being referenced from another applicable subpart. For example, MACT CC references HON (40 CFR 63 Subp. G) storage tank requirements under §63.646, and MPCA referenced the Minnesota rule that incorporates HON (Minn. R. 7011.7040(B)) instead of MACT CC (Minn. R. 7011.7280(A)), and

  - Added more specific references to the Minnesota rules that provide broad authority for permitting requirements (e.g., Minn. R. 7007.0800 Subp. 4 = Monitoring, Subp. 5 = Recordkeeping, Subp. 6 = Reporting), but the references do not materially impact the compliance obligations.

# Total Facility Section (TFAC)

MPCA made the following unique changes to the TFAC section:

- Added new template SO2 equivalent or better dispersion (EBD) conditions, including an EBD modeling at reissuance trigger to incorporate SO2 emission changes during the permit term,

- The PM10 modeling parameters included in Appendix C were updated to reflect the PM10 modeling submitted as part of the 2017 Title V Reissuance application. Appendices B and D reflect the historic modeled parameters and were not updated to the more recent modeling parameters (e.g., temperature, emission rates, etc.) submitted as part of the 2017 Title V Reissuance application or the Air Emission Risk Assessment (AERA) modeling; this discrepancy presents compliance uncertainty for SPPRC and could impact future permitting project timelines (i.e., MPCA requiring additional modeling),

- Removed AERA modeling requirements (this is appropriate because SPPRC addressed MPCA’s AERA modeling requirement),

- Removed PM10 NAAQS modeling at Title V reissuance condition. This is appropriate because the facility’s PM10 modeled impacts are less than the PM10 Significant Impact Level (SIL),

- Added the new Minnesota rule banning trichloroethylene (TCE), and

- Added a broad condition to comply with the associated recordkeeping conditions for small tanks subject to OLD MACT (40 CFR 63 Subp. EEEE).

# Heaters and Boilers

MPCA made the following unique changes to the heaters and boilers EQUI/COMG sections:

- Removed the CEMS operation requirements from the heater’s EQUIs/COMGs to EQUIs/COMGs for the meters. This is a reasonable change, but it reorganizes the permits,

- Removed fuel oil-related conditions as described in Section 1.0,

- Updated the following EQUI numbers due to the emission unit classification changes:

  - From EQUI 1 to EQUI 789: Alky Isostripper Htr 5-28-B-1,

  - From EQUI 6 to EQUI 790: No. 1 Crude Charge Htr 5-1-B-7, and

  - From EQUI 15 to EQUI 798: SGP Dehexanizer Reboiler 5-10-B-1.

SPPRC made the following PTE calculations changes:

- Updated the maximum capacities if the actual operating rates in the previous 5-year period rates exceeded the previously listed maximum capacity values or if the engineering data reviewed indicated the capability of a higher burner capacity,

- Updated any NOx emission factors to the most recent stack test, including an additional 50% safety factor, manufacturer’s guaranteed or, if the stack test occurred more than 10 years ago, the highest emission rate between the AP-42 Chapter 1.4 emission factors and the most recent stack test, including an additional 50% safety factor,

- Updated hazardous air pollutant (HAP)/air toxic (AT) emission factors to reflect site-specific stack testing conducted to support the 2019 AERA, and the Environmental Protection Agency’s (EPA’s) 2015 Emissions Estimation Protocol for Petroleum Refineries (2015 Protocol),

- Added the Lab HVAC1 unit heater (EQUI 349) because it does not qualify as an insignificant activity pursuant to Minn. R. 7007.1300, and

- Updated the greenhouse gas (GHG) global warming potential (GWP) values to reflect the November 29, 2013 amendments to 40 CFR Part 98.

# Meters

MPCA made the following unique changes to meters EQUI/COMG sections:

- Created multiple new EQUIs and COMGs. EQUIs were created to separate meters from their respective emission units. COMGs were created to group identical requirements from multiple EQUIs (e.g., Boiler MACT testing, certification testing and reporting, and monitor design),

- Added EQUIs 795 (SO2 meter) and 796 (O2 meter) for the portable equipment degassing TO; however, the associated conditions are not applicable because the site can utilize the 2018 portable equipment degassing AMP to comply with the requirements,

- Added the following meters to the permit

  - EQUI 350: 38B1 PSA flow meter,

  - EQUI 351: 38B2 PSA flow meter,

  - EQUI 352: Boiler 7 Fuel Gas/Natural Gas flow meter,

  - EQUI 353: Boiler 8 Fuel Gas/Natural Gas,

  - EQUI 354: Flare Thermocouple 1*multiple,

  - EQUI 355: FCC Coke Burn Parametric Monitoring System,

  - EQUI 356: 29-B-2 Fuel Flow Meter (gas),

  - EQUI 357: SRU/SCOT 2 Fuel Gas flow meter,

  - EQUI 358: SRU/SCOT 2 Absorber Off Gas flow meter,

  - EQUI 359: Sulfur Pit 2 Flow Monitoring System,

  - EQUI 360: SRU 2 Treatment Unit Incinerator Thermocouple*multiple,

  - EQUI 361: 36-B-3 Fuel Flow Meter (gas),

  - EQUI 362: 36-B-4 Fuel Flow Meter (gas),

  - EQUI 363: 37B1 Natural Gas flow meter,

  - EQUI 364: SRU/SCOT 3 Natural Gas flow meter,

  - EQUI 365: SRU/SCOT 3 Absorber Off Gas flow meter,

  - EQUI 366: Sulfur Pit 3 Flow Monitoring System,

  - EQUI 367: SRU 3 Treatment Unit Incinerator Thermocouple*multiple,

  - EQUI 368: 1B6 Fuel Gas flow meter,

  - EQUI 369: 8B1 Fuel Gas flow meter,

  - EQUI 370: SDA Fuel Gas flow meter,

  - EQUI 371: 37B2 Natural Gas flow meter,

  - EQUI 372: Thermal Oxidation Temperature transmitter* multiple,

  - EQUI 373: DCS,

  - EQUI 374: 2B3 Natural Gas flow meter,

  - EQUI 375: TREA 26 VCU Thermocouple,

  - EQUI 797: Thermocouple,

  - EQUI 799: 38-B-1 Fuel Flow Meter,

  - EQUI 800: 38-B-2 Fuel Flow Meter,

  - EQUI 801: Boiler 7 Fuel Flow Meter (gas),

  - EQUI 802: Boiler 8 Fuel Flow Meter (gas),

  - EQUI 803: 1-B-6 Fuel Flow Meter (gas),

  - EQUI 804: 8-B-1 Fuel Flow Meter (gas),

  - EQUI 805: SDA Fuel Flow Meter (gas),

  - EQUI 806: Flare Thermocouple 1*multiple,

  - EQUI 807: Temporary Flare Steam Monitor, and

  - EQUI 813: Flare Vent Gas Flow Meter.

# Wastewater Treatment Plant (WWTP)

MPCA made the following unique changes to the WWTP EQUI/COMG sections:

- Added a variety of new inspection, recordkeeping, and operational requirements, including requirements to:

  - Maintain minimum VOC control efficiencies (99%),

  - Install thermocouples for temperature monitoring,

  - Calibrate the temperature monitor every 12 months,

  - Inspect the control device every 5-year period, annually, and quarterly,

  - Keep records on confirmations the temperature recording device is working,

  - Keep records of combustion temperatures,

  - Operate TO according to the O&M Plan, and

  - Permit control equipment replacements.

- Removed the T146 Water Seal Overflow requirements,

- Updated to incorporate more text from the regulations into the requirements, and

- Added new TEMPO IDs for the following existing WWTP equipment:

  - EQUI 809: API Splitter Box,

  - EQUI 810: API Oil/Water Separator,

  - EQUI 811: WWTP Pump Pit, and

  - EQUI 812: DGF Pump pit; concrete basin w/ pumps and is controlled.

SPPRC made the following PTE calculations changes:

- Updated the HAP/AT emission factors for the WWTP thermal oxidizer (TO) (TREA 5) and back-up WWTP TO (TREA 9) to include additional pollutants that are estimated in the actual emission calculations from the Water9 model and past splitter box sampling data, and

- Added emissions estimates for the API separator mid-bay section (EQUI 810) based on floating roof storage tank methodology (AP-42 Section 7.1).

# Equipment Leaks

MPCA made the following unique changes to the equipment leak FUGI/COMG sections:

- Incorporated many conditions by reference to NSPS A, NSPS VVa, and NSPS GGGa.,

- COMG 42 was added to address maintenance vents; the conditions are verbatim rule language with the RSR maintenance vent requirements, as requested, and

- COMG 43 was added to address open-ended line (OELs), and the conditions were included as SPPRC proposed (consistent with FHR Pine Bend’s OEL permit conditions).

SPPRC made the following PTE calculations changes:

- Updated the component counts for each fugitive source ID based on the actual light liquid and gas/vapor counts in LeakDAS/FELDAR (from 2017) and estimated heavy liquid and connector counts based on the similar process units from the 2015 Protocol (including associated safety factors),

- Updated the emission factors from the EPA’s 1995 Protocol for Equipment Leak Emission Estimates (1995 Equipment Leak Protocol) default emission factors to an approach utilizing the 1995 Equipment Leak Protocol’s screening correlation equations (with assumed leaking percentages) and 1996 American Petroleum Institute (API) Development of Emission Factors for Leaks in Refinery Components in Heavy Liquid Service (1996 API Report) heavy liquid emission factor, and

- Update the HAP/AT speciation profiles based on the 2016 Toxic Chemical Release Inventory (TRI) speciation (based on FELDAR data), and chemical-specific evaluations completed in support of the 2019 AERA.

# Flare

MPCA made the following unique changes to the flare EQUI/COMG sections:

- Removed SU/SD conditions consistent with the RSR requirements,

- Removed redundant flare conditions,

- Removed outdated flare requirements (e.g., 300 Btu/scf),

- Incorporated several conditions by reference to MACT CC and NSPS Ja,

- Removed Consent Order condition since the condition has expired

- Added a temporary flare (TREA 34) and associated meters to the flare component group (COMG 34),

- Added permit conditions from the 2018 Administrative Compliance Order permit application, and

- Added RSR conditions using verbatim rule language.

SPPRC made the following PTE calculations changes:

- Updated the hazardous air pollutant (HAP)/air toxic (AT) emission factors to reflect site-specific stack testing conducted to support the 2019 AERA, and the 2015 Protocol.

# Tanks

MPCA made the following unique changes to the tank EQUI/COMG sections:

- Added the new MACT CC RSR tank requirements (§63.660), as requested. Since many tanks still comply with §63.646 (which references HON – 40 CFR 63 Subpart G) or the NSPS Kb overlap requirements, it is necessary to have both sets of conditions (historical compliance options and new compliance options),

- Updated MACT CC RSR recordkeeping and reporting requirements (63.655) to be consistent with the regulatory language,

- Added clarifying language (“Until the first degassing and cleaning event after June 30, 2014, or January 30, 2026, whichever is first” or “After the …”) to all of the MACT CC-related conditions to make the permit easier to navigate,

- Added the Tank Degassing TO and associated permit conditions as represented in the 2018 Tank Degassing TO permit application, and

- Restructured nearly all of the conditions to follow the new tank language template, so few conditions are directly comparable to the existing permit.

SPPRC made the following PTE calculations changes:

- Updated each storage tank’s service representations (i.e., gasoline, heavy naphtha, reformate, etc.) to reflect potential service options based on service history and discussions with Operations staff,

- Revised the HAP/AT representations for stocks based on chemical-specific reviews conducted to support the 2019 AERA,

- Adjusted each storage tank’s throughput, temperature, and RVP representations based on production limits for associated process units (e.g., sour kero throughput = DU capacity = 10,500 bpd), and maximum monthly temperatures and RVP values (with seasonal assumptions and safety factors) from recent operations, and

- Updated the VOC and HAP/AT emissions as the highest emission rate of each potential service option.

# FCC

MPCA made the following unique changes to the FCC EQUI section:

- Added “filterable” to the NSPS J PM limit to be consistent with the regulation and provide regulatory interpretation certainty for SPPRC,

- Removed the Method 202 test method reference in the 5-year PM performance test requirement (permit condition 6.8.1).

- Added RSR requirements, including startup, shutdown, and hot standby (SUSD hot standby) work practice standards,

- Added the AERA-related SO2 3-hour block average limit and the associated limiting language (i.e., “This is a state-only requirement and is not enforceable by the EPA Administrator or citizens under the Clean Air Act”), and

- Removed CEMS/COMS-related conditions and added a cross-reference to newly created meter EQUIs where the requirements are housed (e.g., EQUI 220 (FCC SO2), 221 (FCC O2), 222 (FCC NOx), etc.).

SPPRC made the following PTE calculations changes:

- Updated the hourly SO2 limit based on the 2019 AERA,

- Updated the maximum hourly coke burn rate (from 17,000 lb/hr to 18,000 lb/hr) and the corresponding PM, PM10, and PM2.5 emission rates based on the updated coke burn rate and recent testing results (PM10 and PM2.5).

- Adjusted the VOC emission factor to account for partial burn operation (200 hr/yr as a conservative estimate), and

- Updated HAP/AT emission factors to reflect site-specific testing (2015 and 2018), catalyst sampling analytical data (2018 and 2019), and 2015 Protocol values.

# Engines

MPCA made the following unique changes to the engine EQUI/COMG sections:

- Removed portable non-road engines from the permit (COMG 19), as requested,

- Removed all of the engine COMGs (COMG 35-37) and listed the permit conditions under the individual engine EQUIs (EQUI 25 (Fire Hall), EQUI 327 (Boiler House), EQUI 335 (Fire Water Pump 1), etc.),

- Added specific emission limits for NSPS JJJJ/IIII certified engines, which have no particular requirements to test against the emission limits unless the engine is not maintained according to the manufacturer’s recommendations,

- Reclassified the engine in TEMPO from pump to engine and created a new TEMPO ID (EQUI 791: Emergency Backup Diesel Pump (Emergency Stationary Engine)),

- Updated the SO2 limit to reflect the State’s ULSD requirement (from 0.5 lb SO2 /MMBtu to 0.0015 lb SO2 /MMBtu),

- Adjusted existing monthly fuel recordkeeping requirement from “fuel usage and type” to only “fuel type”, and added the requirement to the facility’s other engines (EQUI 46, EQUI 49, EQUI 333, EQUI 334, and EQUI 791), and

- Added a variety of new conditions following MPCA’s standard template language for engines (MACT ZZZZ and NSPS JJJJ/IIII).

SPPRC made the following PTE calculations changes:

- Removed portable non-road engine calculations, and

- Updated benzene, acenaphthylene, anthracene, benzo(a)anthracene, and dibenzo(a,h)anthracene emission factors based on the March 2018 stack testing.

# Loading

MPCA made the following unique changes to the loading EQUI/COMG sections:

- Added barge loading (EQUI 346) and heavy liquid loading (EQUI 347 – Rail and EQUI 348 – Truck) as EQUIs (previously listed as insignificant activities), as requested,

- Incorporated MACT CC RSR-related changes for the barge loading,

- Rearranged loading requirements to the loading rack component group (COMG 28), the loading rack EQUI (EQUI 28), the VRU (TREA 18), or to the VCUs (primary – TREA 26 or backup – TREA 25), as appropriate,

- Reorganized MACT CC and other requirements (i.e., some rules are incorporated by reference, and some are verbatim in the permit), and

- Added the new AERA-based barge loading LCO monthly recordkeeping requirement.

SPPRC made the following PTE calculations changes:

- Updated the light liquid loading rack (EQUI 28) HAP/AT emission rates based on the 2018 stack testing,

- Added barge loading (EQUI 346), heavy liquid loading (EQUI 347 – Rail and EQUI 348 – Truck) emission calculations based on worst-case materials, highest actual throughput (including a safety factor), and updated speciation profiles,

- Added sulfur loading (EQUI 792) and Sulfur Pit #2 and #3 (EQUIs 793 and 794) emission calculations based on 2018 sampling data conducted to support the 2019 AERA, and

- Incorporated the barge light cycle oil (LCO) loading limit based on the 2019 AERA.

# Sulfur Recovery Units (SRUs)

MPCA made the following unique changes to the SRUs EQUI/COMG sections:

- Changed permit condition 5.8.2 language to allow for SRU 2 (EQUI 16) to use actual emissions instead of the historical potential emissions (11.9 tpy NOx), consistent with the other units in the permit limit calculation. Note, the current SRU 2 PTE is 2.18 tpy NOx, based on AP-42 Table 8.13-2 emission factors,

- Moved requirements to other units (EQUI 166 (SRU 2 SO2), EQUI 167 (SRU 2 O2), EQUI 210 (SRU 3 SO2), EQUI 211 (SRU 3 O2), and COMG 41 (NSPS General Requirements)),

- Added requirements to reference other units with applicable requirements (EQUI 166 (SRU 2 SO2), EQUI 167 (SRU 2 O2), EQUI 210 (SRU 3 SO2), EQUI 211 (SRU 3 O2), and COMG 41 (NSPS General Requirements)), and

- Added new conditions related to the consent decree.

SPPRC made the following PTE calculations changes:

- Updated the NOx and CO emission factors based on AP-42 Section 8.13 (2015),

- Added carbonyl sulfide and carbon disulfide emissions based on the 2015 Protocol emission factors,

- Updated HAP/AT emission rates based on the 2018 stack test, and

- Revised and/or removed pollutant emission rates from the 2001 stack test based on chemical-specific reviews conducted for the 2019 AERA.

# Catalyst

MPCA made the following unique changes to the catalyst-related equipment EQUI/COMG sections:

- Created a new TREA for FCC Catalyst Unloading (TREA 32) with similar conditions to the other existing catalyst baghouse TREAs,

  - Added the pressure drop range and updated the pressure drop monitoring requirements, and

  - Corrections actions include Method 9 opacity observations if operating beyond the pressure drop range. Method 9 opacity observations require unique operator training (commonly refer to as “smoke school”).

- Opacity limits (≤20%) were maintained at the associated EQUIs (EQUI 29 – Fresh, EQUI 34 – NOx Additive, EQUI 35 – SO2 Additive, and EQUI 45 - Spent).

- Updated the emission unit-to-control device relationships to reflect the actual configuration (i.e., the exhaust stream from the spent catalyst hopper (after the FCC Regenerator) is not routed to the baghouse),

- Added requirements to have a minimum control efficiency for the baghouses (99%).

SPPRC made the following PTE calculations changes:

- Updated the catalyst handling equipment PM/PM10/PM2.5 emissions based on the AP-42 Section 11.12 emission factor (controlled cement unloading to elevated storage silo (Pneumatic)), and

- Adjusted the fresh and spent catalyst handing (EQUI 29 and EQUI 45) HAP/AT emission factors based on the updated FCC PM speciation (refer to Section 9.0).

# Reformer

MPCA made the following unique changes to the reformer EQUI sections:

- Removed SSM-related conditions (e.g., SSMP and recordkeeping) and added RSR-related active depressuring/purging conditions, and

- Added HCl concentration values from the most recent performance test, as requested.

SPPRC made the following PTE calculations changes:

- Updated to the VOC and HAP/AT emission factors based on the 2015 Protocol, and

- Adjusted the maximum reformer regeneration venting hours to 10 days per year (240 hours per year).